3GPP TSG-WG4 Meeting #73	R4-147349

# San Francisco, USA, 17th – 21st November, 2014

Agenda item: 	7.15.1

Source: 	Qualcomm Incorporated

Title: 	UE demodulation performance due to introduction of DCH enhancements

Document for:	Discussion

# 1	Introduction

In RAN4#72bis, [1], [2] and [3] presented performance results due to introduction of DCH enhancements covering single link DCH performance corresponding to section 8.2 and 8.3 in TS25.101, handover performance corresponding to section 8.7 in TS25.101 and downlink power control performance corresponding to section 8.8 in TS25.101 corresponding to reference measurement channels in [4] as agreed in [6]. This contribution presents some additional performance results for DCH enhancements which were not included in [1].

# 2	Performance Results

## 2.1	Combining of TPC commands from radio links of different radio link sets

This test corresponds to section 8.7.2 in TS25.101. There are 2 tests to verify TPC combining over 4 slots from Cell1 and Cell2. Cell 1 uses the pattern {0,0,1,1} and Cell 2 uses the pattern {0,1,0,1}. Test 1 is the case of static propogation condition without AWGN source and Test 2 is Case 3 multi-path fading propagation condition. For R99 test scenario, the requirement for Test 1 is that the required power changes over the 4 consecutive slots shall be Down, Down, Down, Up. The requirement for DCH enhancements can remain the same as well.

For Test 2, we observed the following UP command ratio and DOWN command ratio for DCH enhancements:

| Ratio (Transmitted power UP) / (Total number of slots) | Ratio (Transmitted power DOWN) / (Total number of slots) |
| --- | --- |
| 0.271 | 0.729 |

## 2.2	Combining of reliable TPC commands from radio links of different radio link sets

This test corresponds to section 8.7.3 in TS25.101 where the UE transmit power is initialised to -15 dBm. Test 1 verifies that the UE follows only the reliable TPC commands in soft handover. Test 2 verifies that the UE follows all the reliable TPC commands in soft handover. Based on our simulations, we observed that for Test 1 UE output power falls within -15 ± 5 dB and for Test 2 UE output power falls within -15 ± 3 dB for 90% of the time.

## 2.3	DL power control performance

This section shows of DL power control performance results corresponding to sections 8.8.1, 8.8.2 and 8.8.3 due to updates in simulator design assumptions.

## 2.3.1 Power control in the downlink, constant BLER target

For the test on Power control in the downlink, constant BLER target corresponding to section 8.8.1, the following are the simulation results for the 90% downlink power ratio

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.1 | 8.29 | Case 4 | 9,-1 |  |
|  |  | Basic Config 20ms |  | Basic Config 10ms |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Test1 | FULL-DCCH | -19.11 |  | -16.12 |  |
|  | NULL-DCCH | -21.01 |  | -18.11 |  |
| Test2 | FULL-DCCH | -12.02 |  | -7.24 |  |
|  | NULL-DCCH | -13.14 |  | -10.21 |  |

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.1 | 8.29 | Case 4 | 9,-1 |  |
|  |  | Full Config 10ms |  | Full Config 10ms NULL UL |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Test1 | FULL-DCCH | -19.12 |  | -19.13 |  |
|  | NULL-DCCH | -21.02 |  | -21 |  |
| Test2 | FULL-DCCH | -11.21 |  | -11.2 |  |
|  | NULL-DCCH | -13.07 |  | -13.1 |  |

## 2.3.2 Power control in the downlink, initial convergence

For the test on Power control in the downlink, initial convergence corresponding to section 8.8.2, the following tables show the Ec/Ior lower bound (LB), 10% Ec/Ior and Ec/Ior upper bound (UB).

| Section # | Subsection | Table | Channel | Geometry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.2 | 8.31 | AWGN | 9,-1 |  |  |  |
|  |  | Basic Config 20ms T1 |  |  | Basic Config 20ms T2 |  |  |
|  |  | EcIor LB | EcIor 10% | EcIor UB | EcIor LB | EcIor 10% | EcIor UB |
| Test 1 | FULL-DCCH | -19.1 | -17.1 | -13.7 | -19.7 | -19.4 | -16.8 |
|  | NULL-DCCH | -18.8 | -17.1 | -13.6 | -20.0 | -19.4 | -17.0 |
| Test 2 | FULL-DCCH | -18.2 | -16.5 | -13.0 | -19.4 | -19.2 | -16.8 |
|  | NULL-DCCH | -18.2 | -16.4 | -13.0 | -19.8 | -19.3 | -16.6 |
|  |  | Basic Config 10ms T1 |  |  | Basic Config 10ms T2 |  |  |
|  |  | EcIor LB | EcIor 10% | EcIor UB | EcIor LB | EcIor 10% | EcIor UB |
| Test 1 | FULL-DCCH | -18.1 | -17.4 | -13.4 | -19.7 | -19.1 | -15.7 |
|  | NULL-DCCH | -18.1 | -17.4 | -13.2 | -21.7 | -19.4 | -16.7 |
| Test 2 | FULL-DCCH | -16.8 | -16.0 | -12.6 | -20.9 | -18.7 | -15.9 |
|  | NULL-DCCH | -17.2 | -16.0 | -12.6 | -20.9 | -19.0 | -16.2 |
|  |  | Full Config 10ms T1 |  |  | Full Config 10ms T2 |  |  |
|  |  | EcIor LB | EcIor 10% | EcIor UB | EcIor LB | EcIor 10% | EcIor UB |
| Test 1 | FULL-DCCH | -17.9 | -16.8 | -13.4 | -20.2 | -19.2 | -16.9 |
|  | NULL-DCCH | -18.1 | -17.4 | -13.2 | -21.9 | -19.4 | -16.9 |
| Test 2 | FULL-DCCH | -17.2 | -16.4 | -12.5 | -19.7 | -18.9 | -17.2 |
|  | NULL-DCCH | -17.2 | -16.0 | -12.6 | -20.8 | -19.4 | -16.2 |
|  |  | Full Config 10ms NULL UL T1 |  |  | Full Config 10ms NULL UL T2 |  |  |
|  |  | EcIor LB | EcIor 10% | EcIor UB | EcIor LB | EcIor 10% | EcIor UB |
| Test 1 | FULL-DCCH | -17.2 | -16.5 | -12.2 | -19.9 | -19.4 | -16.8 |
|  | NULL-DCCH | -17.0 | -16.3 | -12.2 | -20.7 | -19.4 | -16.4 |
| Test 2 | FULL-DCCH | -17.6 | -15.9 | -11.6 | -20.6 | -19.1 | -16.6 |
|  | NULL-DCCH | -16.9 | -16.3 | -12.3 | -20.7 | -20.0 | -15.9 |

## 2.3.3 Power control in the downlink, wind-up

For the test on Power control in the downlink, wind-up corresponding to section 8.8.3, the following are the simulation results for the 90% downlink  power ratio

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 | 5 |  |
|  |  | Basic Config 20ms |  | Basic Config 10ms |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Stage 3 | FULL-DCCH | -16.16 |  | -15.07 |  |
| Stage 3 | NULL-DCCH | -19.05 |  | -17.02 |  |

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 | 5 |  |
|  |  | Full Config 10ms |  | Full Config 10ms NULL UL |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Stage 3 | FULL-DCCH | -16.23 |  | -16.17 |  |
| Stage 3 | NULL-DCCH | -19.03 |  | -18.1 |  |

## 2.3.4 Power control in the downlink, different transport formats

For the test on Power control in the downlink, different transport formats corresponding to section 8.8.4, the following are the simulation results for the 90% downlink  power ratio

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 | 5 |  |
|  |  | Basic Config 20ms |  | Basic Config 10ms |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Stage 1 | FULL-DCCH | -19.11 |  | -16.12 |  |
| Stage 2 | NULL-DCCH | -21.01 |  | -18.11 |  |

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 | 5 |  |
|  |  | Full Config 10ms |  | Full Config 10ms NULL UL |  |
|  |  | 90% EcIor |  | 90% EcIor |  |
| Stage 1 | FULL-DCCH | -19.12 |  | -19.13 |  |
| Stage 2 | NULL-DCCH | -21.02 |  | -21 |  |

# 4	Conclusion

This contribution presents performance results with DCH enhancements for

- Combining of TPC commands from radio links of different radio link sets

- Combining of reliable TPC commands from radio links of different radio link sets

- DL power control results

# 5	Reference

- R4-146432, “UE demodulation performance results due to introduction of DCH enhancements”

- R4-145699, “DCH Enhancement Performance in Single Link and SHO Test Cases”, MediaTek

- R4-145700, “DCH Enhancement Performance in Power Control Test Cases”, MediaTek

- R4-145141, “New DL reference measurement channels for DCH enhancements," Qualcomm Incorporated

- R4-145140, “Further discussion on UE RAN4 requirements due to introduction of DCH enhancements”, Qualcomm Incorporated

- R4-145387, “WF on UE Requirements for DCH enhancements”, Qualcomm Incorporated