3GPP TSG-WG4 Meeting #73	R4-147356

# San Francisco, USA, 17th – 21st November, 2014

Agenda item: 	7.16.1

Source: 	Qualcomm Incorporated

Title: 	E-AGCH performance with improved granting

Document for:	Discussion

# 1		Introduction

In RAN4#72, it was agreed in [1] that new demodulation test cases, performance metric and requirements shall be defined for improved E-AGCH granting in the context of EUL enhancements WI.

Three test cases to be defined are (1) No E-AGCH (2) E-AGCH for other UEs (3) E-AGCH for UE under test, and minimum requirements shall be specified for each test case. The performance metric i.e. percentage of incorrect decisions by UE shall be a function of FA_PC/MD_PC which are rate of false alarm/mis-detection respectively based on E-AGCH presence check, and, CS/CF is E-AGCH CRC check success/failure rate respectively. The following table summarizes the three test cases and the performance metric as a function of E-AGCH presence check and E-AGCH CRC check.

| Test | Node B transmits | E-AGCH presence check | E-AGCH CRC check | Performance Metric |
| --- | --- | --- | --- | --- |
| 1 | No E-AGCH | False alarm ‘FA_PC’ (Detects the presence) | - | FA_PC Presence check passes |
| 2 | E-AGCH for other UEs | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Pass ‘CS’ | (MD_PC) |
| 3 | E-AGCH for UE under test | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Fail ‘CF’ | (MD_PC) + (1- MD_PC)*CF |

# 2	E-AGCH Performance with Time Multiplexed Granting

## 2.1	Test 1 - No E-AGCH transmitted

Ior/Ioc (dB) is set to 0 dB. When the propagation channel is AWGN, the false alarm probability i.e. probability that E-AGCH presence check block detects the presence is 0.105%.

## 2.2	Test 2 and Test 3 - Node B Transmits E-AGCH

Test scenario 2 is the case where E-AGCH transmitted is for other UE and not for UE under test. Test scenario 3 is the case where E-AGCH transmitted is for UE under test. Ior/Ioc (dB) is set to 0 dB. The E-RNTI for UE under test is set to 0 (0x0000) and E-RNTI for Other UE is set to 51966 (0xCAFE).

| E-AGCH Ec/Ior (dB) | Test 3 E-AGCH for UE under test | Test 2 E-AGCH for other UE |
| --- | --- | --- |
|  | Performance Metric (MD_PC) + (1- MD_PC)*CF | Performance Metric (MD_PC) + (1-MD_PC)*CS |
| -17 | 0 | 0 |
| -18 | 0 | 0 |
| -19 | 0 | 0 |
| -20 | 0.002 | 0 |
| -21 | 0.022 | 0.001 |
| -22 | 0.217 | 0.037 |
| -23 | 1.919 | 0.732 |
| -24 | 10.668 | 6.109 |
| -25 | 31.738 | 22.817 |
| -26 | 58.963 | 48.13 |
| -27 | 80.892 | 71.289 |
| -28 | 92.642 | 86.132 |
| -29 | 97.48 | 93.482 |

## 2.3	Test 4 - Node B Transmits E-AGCH (Type 1 UE Receiver)

Test scenario 4 is the case where E-AGCH transmitted is for UE under test and UE uses Type 1 receiver. Ior/Ioc (dB) is set to 0 dB. The E-RNTI for UE under test is set to 0 (0x0000).

| E-AGCH EcIor (dB) | Test 4 E-AGCH for Type 1 UE under test |
| --- | --- |
|  | Performance Metric (MD_PC) + (1- MD_PC)*CF |
| -20 | 0 |
| -21 | 0 |
| -22 | 0 |
| -23 | 0 |
| -24 | 0.012 |
| -25 | 0.182 |
| -26 | 1.44 |
| -27 | 7.047 |
| -28 | 20.807 |
| -29 | 41.374 |
| -30 | 62.592 |
| -31 | 79.532 |
| -32 | 89.798 |
| -33 | 95.286 |

# 3		Conclusion

This contribution has presented E-AGCH performance results for the AWGN channel conditions.

Proposal: The following tables show the proposed E-AGCH Ec/Ior requirements for the various scenarios

Table 1: Minimum requirement for E-AGCH detection when no E-AGCH is transmitted

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | E-AGCH (dB) | (dB) | Performance Metric= Prob of False Alarm |
| 1 | - | DTX | 0 | 0.0015 |

Table 2: Minimum requirement for E-AGCH detection when E-RNTI is not set for UE under test – single link

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | E-AGCH (dB) | (dB) | Performance Metric= PMD + (1- PMD)*(1-PCRC) |
| 2 | AWGN | -21 | 0 | 0.001 |

Table 3: Minimum requirement for E-AGCH detection when E-RNTI is set for UE under test – single link

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | E-AGCH (dB) | (dB) | Performance Metric= PMD + (1- PMD)*PCRC |
| 3 | AWGN | -21 | 0 | 0.01 |

Table 4: Enhanced performance requirement Type 1 for E-AGCH detection when E-AGCH is for UE under test – single link

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | E-AGCH (dB) | (dB) | Performance Metric= PMD + (1- PMD)*PCRC |
| 4 | AWGN | -24 | 0 | 0.01 |

# 4		Reference

- R4-146670, “WF on E-AGCH demodulation performance requirement for further EUL”, Qualcomm Incorporated, Ericsson, Nokia Networks