3GPP TSG-RAN WG4 #73	R4-147433

San Francisco, USA, 17th Nov – 21th Nov. 2014

Source: 	Ericsson

Title:	Further discussion on LC_MTC RLM parameters setup

Agenda Item:	7.3.4

Document for:	Discussion

# Introduction

In the RAN4#72 meeting, LC_MTC RLM parameters setup was further discussed. The following WF was acceptable for the group [1]:

- For OOS, the ratio of PDCCH RE energy to average RS RE energy is:

  - 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell (no change to current setup for legacy UE)

  - when two or four antenna ports are used for cell-specific reference signal transmission by the PCell.

    - Option 1: 4 dB (as baseline)

    - Option 2: 1 dB (No change to current setup for legacy UE)

- For INS, the ratio of PDCCH RE energy to average RS RE energy is:

  - when single antenna port is used for cell-specific reference signal transmission by the Pcell

    - Option 1: 0 dB (as baseline, no change to current setup for legacy UE)

    - Option 2: 1 dB

  - When two or four antenna ports are used for cell-specific reference signal transmission by the Pcell

    - Option 1: 1 dB (as baseline)

    - Option 2: -3 dB (no change to current setup for legacy UE)

- Both for OOS and INS, the “Ratio of PCFICH RE energy to average RS RE energy” of legacy UE is reused.

- The PDCCH boosting and RLM evaluation period issues shall be concluded in RAN4 #72bis meeting.

The main concern for the RLM parameters change (i.e, boost power for hypothetical PDCCH transmission) is:

- Whether PDSCH can be reliably detected by the cell-edge LC_MTC UE when the coverage of control channel of LC_MTC is kept similar to legacy UE’s coverage.

- Whether system information can be reliably detected by the cell-edge LC_MTC UE when the coverage of control channel of LC_MTC is kept similar to legacy UE’s coverage

In this paper, we provide simulation results to check these concerns under the condition in which UE claims In-Sync or Out-of-Sync.

# Simulation results

## Link level simulation results for PDSCH with the lowest MCS (MCS0)

For LC_MTC UE, when it is in the cell edge and the cell is planned according to normal UE coverage, at least two simple mechanisms can be employed to provide service, one is downlink power control, and one is to use multiple HARQ retransmissions. For MTC type communication, high data rate is not the main target, the reliability is much more important. To achieve reliability, both power control and multiple HARQ retransmissions are feasible. With reference to the analysis in [2] the range of PDSCH power boost is much wider than that of PDCCH. Obviously, power control can be used for reliable PDSCH transmission. Hence, in this section, we skip the power control mechanism and focus on the investigation of multiple HARQ retransmission mechanism.

In Figure 1, Link level simulation results for PDSCH are provided. The simulation parameters are shown in Table 1. The out-sync bound and in-sync bound are given by link level simulation as shown in Appendix A. In the simulation, no power boost is used and the lowest MCS (MCS0) is selected as the target MCS. Further, residual BLER is used as the metric and the residual BLER after HARQ transmission is defined in [3] as follows:

: Number of correctly decoded MAC PDUs

: Number of transmitted MAC PDUs (Retransmitted MAC PDUs are not counted)

Out-Sync bound

In-Sync bound

Figure 1: Link level performance for PDSCH when MCS0 is used (Note: NO PDSCH power boost is considered)

Table 1: Simulation parameters for MCS0 PDSCH

| Parameters | Unit |  |
| --- | --- | --- |
| Channel model |  | ETU 30 |
| Bandwidth | MHz | 10 |
| Downlink power Allocation | dB | ρA=-3 ρB=-3 (Note 1) |
| Allocated PRBs (NPRB) | PRB | 6 |
| TBS index (ITBS) |  | 0 |
| Antenna configuration |  | 2x1 |
| Maximum number of HARQ transmissions |  | 4 or 8 |
| Receiver |  | Practical receiver |
| Note 1: PB=1, NO power boost for PDSCH |  |  |

From Figure 1, we can see that at the target SNR of In-Sync (-5.1dB), the residual BLER with 4 transmissions is much lower than 0.001. In other words, the data can be reliably received when UE claims it is In-sync with only 4 transmissions. At the target SNR of Out-Sync (-9.7 dB), the residual BLER with 8 transmissions is about 0.002. When UE claims it is Out-Sync, more HARQ transmissions are needed to keep the link. For other channel condition, the similar observations hold. In summary, we have the following observations:

Observation 1: LC_MTC can reliably receive PDSCH with lowest MCS when LC_MTC in the cell-edge when the coverage of the cell is determined by normal UE coverage.

## Discussion on the SIB(s)/paging issue

Besides PDSCH transmission reliability, another concern is about the SIB(s)/paging transmission and reception. Compared with the PDSCH granted by DCI scrambled with C-RNTI, the PDSCH granted by DCI scrambled with SI-RNTI or P-RNTI can be with much lower coding rate. According to 36.213, if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI, and DCI format 1A is used, the column indicator NPRB used to decide the transport block size according to Table 7.1.7.2.1-1 of 36.213 is set as NPRB1A, rather than directly derived from the number of allocated PRBs. According to 36.212,  NPRB1A could be 2 or 3. As one example, assume 6 PRBs are allocated, NPRB1A=2, MCS =0, the coding rate is shown Table 2. From the table, we can see that the coding rate of SIB(s)/paging can be much lower than normal PDSCH.

Table 2: The coding rate difference for PDSCH scrambled with different RNTI

|  | Transport block size | Coding rate |
| --- | --- | --- |
| For PDSCH granted by DCI scrambled with C-RNTI | 152 (MCS0) | 0.096 |
| For PDSCH granted by DCI scrambled with SI-RNTI, P-RNTI or RA-RNTI | 32 | 0.020 |

In Figure 2, link level simulation results are provided for SIB with the parameters given in Table 3. From the figure, we can see that under out-sync condition, when 12 PRBs are used for SIB-1 transmission, LC_MTC can reliably detect it. Considering the low transmission rate of SIB, paging etc, the cost is marginal. For example, for SIB-1, about 3‰ additional overhead is needed if SIB transmission needs modification.

In-Sync bound

Out-Sync bound

Figure 2: SIB/paging performance for LC_MTC with the parameters given in Table 3

Table 3: Parameters for SIB-1 transmission

| Parameters | Unit |  |
| --- | --- | --- |
| Channel model |  | ETU 30 |
| Bandwidth | MHz | 10 |
| Downlink power Allocation | dB | ρA=-3 ρB=-3 (Note 1) |
| SIB-1 MAC PDU size |  | 17 bytes |
| Allocated PRBs (NPRB) | PRB | 6 or 12 |
| NPRB1A |  | 2 |
| Antenna configuration |  | 2x1 |
| Receiver |  | Practical receiver |
| Note 1: PB=1, NO power boost for PDSCH |  |  |

Based on the above analysis, we can have the following observations:

Observation 2: System information can be reliably detected with additional resource allocation.

Based on the above observation, we can confirm the New RLM setup is reasonable. Hence, we propose:

- For OOS, the ratio of PDCCH RE energy to average RS RE energy is:

  - 4 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell.

- For INS, the ratio of PDCCH RE energy to average RS RE energy is:

  - 1 dB; When two or four antenna ports are used for cell-specific reference signal transmission by the Pcell

# Conclusion

Based on link level simulation results, we have the following observations:

Observation 1: LC_MTC can reliably receive PDSCH with lowest MCS when LC_MTC is at the cell-edge when the coverage of the cell is determined by normal UE coverage.

Observation 2: System information can be reliably detected with additional resource allocation.

Based on the above observation, we proposed:

- For OOS, the ratio of PDCCH RE energy to average RS RE energy is:

  - 4 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell.

- For INS, the ratio of PDCCH RE energy to average RS RE energy is:

  - 1 dB; When two or four antenna ports are used for cell-specific reference signal transmission by the Pcell

# Appendix A: Link level simulation results for RLM

According to the LC_MTC discussion, the parameters as shown in Table 4 and Table 5 can be set as baseline for LC_MTC parameters, as shown in [1]. As one example, ETU 30 channel, 10 MHz is used and the antenna configuration is 2x1. Here, 1x1 is omitted since the baseline parameters for 1x1 are the same as 1x2.   Simulation results are shown in Figure 3. Based on the simulation results, the target SNR for RLM is tabulated in Table 6. More specifically, the target SNR for UE to claim In-Sync is -5.1 dB and the target SNR for UE to claim Out-of-Sync is -9.7dB for ETU 30 Channel. In the next section, the detection reliability of PDSCH under -5.1 dB (corresponding to In-Sync bound) and -9.7 dB (corresponding to Out-of-Sync) is investigated.

Table 4: PDCCH/PCFICH transmission parameters for out-of-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1A |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 10 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4; Bandwidth = 1.4 MHz 8; Bandwidth 3 MHz |
| Ratio of PDCCH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 4 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Note 1: DCI format 1A is defined in clause 5.3.3.1.3 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 5: PDCCH/PCFICH transmission parameters for in-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1C |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 10 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4 |
| Ratio of PDCCH RE energy to average RS RE energy | 0 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Note 1: DCI format 1C is defined in clause 5.3.3.1.4 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

-9.7 dB (Out-Sync)

-5.1 dB (In-Sync)

Figure 3: Link level simulation results for LC_MTC RLM (Antenna configuration: 2x1)

Table 6: The target SNR for LC_MTC RLM

|  | Target SNR at target BLER |
| --- | --- |
| In-Sync | -5.1 dB (@2% BLER) |
| Out-Sync | -9.7 dB (@10% BLER) |

# Reference

R4-145556, Way Forward on RLM Requirements for LC-MTC, Huawei, HiSilicon, Ericsson, Aug. 2014

R4-71AH-0050, RLM parameters setup for Low cost MTC with 1 Rx, Ericsson, June, 2014

3GPP TS 36.101 V11.8.0, User Equipment (UE) radio transmission and reception.