3GPP TSG-RAN WG4 (Radio) Meeting #74	R4-150466

Athens, Greece, 9 – 13 February 2015

Agenda Item:	7.22.2.2

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for TR 36.851: Coexistence Studies for LTE Advanced Carrier Aggregation of Band Combination (25 + 41)

Document for:	Approval

# 1.	Introduction

The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12 [1-3]. But the related text proposals [4-5] were approved into the Rel-13 TR [6]. In this contribution, we provide a text proposal to put the corresponding BS materials into the Rel-12 TR [7] to avoid unnecessary confusion.

# 2.	Text Proposal

<Start of change>

##### 7.2.X.1.2	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 25 and Band 41 DL carriers can be calculated as shown in Table 7.2.X.1.2-1 below:

Table 7.2.X.1.2-1: Band 25 and Band 41 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3860 | 3990 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5790 | 5985 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 501 | 760 | 4426 | 4685 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1170 | 1494 | 2997 | 3450 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6356 | 6680 | 6922 | 7375 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 1736 | 2189 | 2431 | 2755 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1910 | 2015 | 2476 | 2710 |

It can be seen from Table 7.2.X.1.2-1 that the 2nd IMD products caused by BS supporting carrier aggregation of Band 25 and Band 41 may fall into the BS receive band of Bands 12, 17, 28 and 44, and the 3rd IMD products may fall into the BS receive band of Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 22, 23, 25, 33, 34, 35, 36, 37, 38, 39, 41 and 42. Note that the calculation in Table 7.2.X.1.2-1 (except the last row) assumes the BS is transmitting with the whole 65 MHz frequency of Band 25 and the whole 194 MHz frequency of Band 41. If the BS is only transmitting an up to 20 MHz DL in Band 25 and Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 2, 3, 4, 9, 10 or 35 as shown in the last row in Table 7.2.X.1.2-1. Moreover, the 3rd IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25, as the transmit-receiver frequency separation is 80 MHz.

Note that Bands 1, 7, 11, 21, 28, 33, 34, 38, 39 and 44 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Furthermore, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the focus here is on the IMD falling into Bands 12, 17, 22, 23, 25 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 17, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 25 and 41 BS transmitters do not share the same antenna with Band 12, 17, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 23 and 25 UL, and hence Bands 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Bands 25 and 41 BS transmitters should not share the same antenna with Band 23 or 25 BS receiver to prevent BS receiver desensitization, or Band 12, 17, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 17, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	R4-148124, “Introduction of additional band combinations for 3DL inter-band CA”, Ericsson, CATT, CMCC.

[2]	R4-148121, “Introduction of 3DL CA combinations”, Nokia Networks.

[3]	R4-148122, “Introduction of 3DL CA combinations”, Nokia Networks.

[4]	R4-146170, “TP for Rel-13 2DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 41)”, Alcatel-Lucent, Sprint.

[5]	R4-146840, “TP for Rel-13 2DL TR 36.8xx: TIB and RIB values of LTE-A Inter-band Carrier Aggregation of Band 25A and Band 41A (1UL)”, Alcatel-Lucent, Sprint.

[6]	R4-15xxxx, “Rel-13 2DL TR 36.8xx: LTE-A Inter-band Carrier Aggregation v0.3.0”, Ericsson.

[7]	R4-15xxxx, “TR 36.851 V0.15.0: Rel-12 Inter-band Carrier Aggregation (2DL/1UL)”, Ericsson.