3GPP TSG-RAN WG4 Meeting #74                           R4-150554

# Athens, Greece, 9 – 13 Feb, 2015

Agenda Item:	9.3.1

Source: 	CMCC

Title: 			   High speed train scenarios

Document for:	Discussion

- Introduction

A study item on LTE performance enhancement under high speed scenario was approved in RAN#66 meeting [1].

In this contribution, we discuss the high speed train scenarios based on practical network deployment and provide corresponding network deployment parameters for further evaluation.

- Existing high speed scenario

The first objective of the study item is to enhance the requirements under existing high speed scenario.

According to the agreed SID, 350km/h is the first priority target speed. So far, EVA600 is considered as the largest Doppler shift for high speed scenario. However, the Doppler shift depends on both speed and frequency. If high frequency is deployed for high speed train, the Doppler shift will be larger than 600Hz. Hence, we need to determine the target Doppler shift, not only the speed. In Table 1, we calculate the Doppler shift under different speed and frequency.

Table 1: Doppler shift under different speed and frequency

| Speed (km/h) | Doppler Shift (Hz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band 39 (1.9GHz) |  | Band 40 (2.3GHz) |  | Band 41 (2.6GHz) |  | Band 42 (3.5GHz) |  |
| Downlink/Uplink | DL | UL | DL | UL | DL | UL | DL | UL |
| 150 | 264 | 528 | 319 | 639 | 361 | 722 | 486 | 972 |
| 200 | 352 | 704 | 426 | 852 | 481 | 963 | 648 | 1296 |
| 250 | 440 | 880 | 532 | 1065 | 602 | 1204 | 810 | 1620 |
| 300 | 528 | 1056 | 639 | 1278 | 722 | 1444 | 972 | 1944 |
| 350 | 616 | 1231 | 745 | 1491 | 843 | 1685 | 1134 | 2269 |
| 400 | 704 | 1407 | 852 | 1704 | 963 | 1926 | 1296 | 2593 |
| 450 | 792 | 1583 | 958 | 1917 | 1083 | 2167 | 1458 | 2917 |

Take CMCC as an example, band 39 and band 41 are deployed for high speed train. According to Table 1, take 350km/h as the target speed, we need to consider at least 843Hz frequency shift for downlink and 1685Hz for uplink.

Hence, we propose to specify associated requirement based on EVA channel model with larger Doppler shift (e.g. 850Hz for downlink, and 1700Hz for uplink).

Proposal 1: Specify associated performance requirements based on EVA channel model with larger Doppler shift (e.g. 850Hz for downlink and 1700Hz for uplink).

- New high speed scenario

The other objective of the study item is to identify new high speed scenario and investigate the potential enhancement. In this section, new high speed scenario and corresponding parameters are provided based on practical network deployment.

In China, most high speed railways are built in suburb between two cities. In order to have better network performance, dedicated network is deployed along the high speed railway. To avoid interference, separate carriers are utilized for dedicated network and public network. To avoid frequent handover, cell combination is applied: multiple RRHs are connected to one BBU with fiber. Then, from physical layer point of view, the cell coverage is extended significantly. Figure 1 shows the dedicated high speed train scenario in our network.

Figure 1: Dedicated high speed train scenario

The above high speed scenario is quite different from the existing scenarios. Compared to existing HST channel model, the new scenario has multiple taps. Hence, we propose to define new high speed channel model based on the identified scenario and specify associated requirements.

Proposal 2: Define new high speed channel model based on identified high speed train scenario (dedicated network with RRH deployments) and specify the associated requirements.

In Table 2, we provide corresponding deployment parameters based on our practical network for further study.

Table 2: Parameters for dedicated high speed train scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| RRH Railway track distance | 100m~300m (>50m) |
| Distance between RRH | 1km |
| Cell ISD | 6km (6 RRHs connect to 1 BBU) |
| RRH height (compared to railway track) | 15m~25m |

- Conclusion

In this contribution, we discuss high speed scenarios based on practical network deployment and provide corresponding network deployment parameters for further study. The proposals are as follows:

Proposal 1: Specify associated performance requirements based on EVA channel model with larger Doppler shift (e.g. 850Hz for downlink and 1700Hz for uplink)

Proposal 2: Define new high speed channel model based on identified high speed train scenario (dedicated network with RRH deployments) and specify the associated requirements.

- Reference

- R4-142307, New SI proposal: LTE performance enhancement under high speed scenario, NTT DOCOMO, INC., Huawei, HiSilicon.