3GPP TSG-RAN4 Meeting #74 	R4-151105

# Athens, 9 – 13 Feb, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2752 | rev | 1 | Current version: | 10.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maximum allowed layers for multiple monitoring for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA |  |  |  |  |  |  | Date: |  | 2015-01-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - TS 36.133 defines in 8.1.2.1.1.1 the minimum number of carriers per RAT group and the maximum allowed layers of 7 for multiple monitoring for the combination of the RAT groups. However, it is not clearly the maximum 7 layers are effective carriers mentioned in 8.1.2.1.1.1. - In 8.1.2.1.1.1, it defines in one place “one GSM layer corresponds to 32 cells”, and in another place that “one GSM layer corresponds to 32 carriers”. It is unclear which one is the right definition, since one GSM cell is not exactly the same as one GSM carrier due to frequency reuse. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the clarification that the maximum 7 layers are effective number of carriers - In 8.1.2.1.1.1, clarifify that “one GSM layer corresponds to 32 carriers”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of the total carrier frequency layers supported by a 2DL CA UE is not defined correctly, and the definition of one GSM layer is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.1.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<First changes>

##### 8.1.2.1.1	Monitoring of multiple layers using gaps

When monitoring of multiple inter-frequency E-UTRAN and inter-RAT (UTRAN, GSM) using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (RSRP, RSRQ, RSTD, UTRAN TDD P-CCPCH RSCP, UTRAN FDD CPICH measurements, GSM carrier RSSI, etc.) of detected cells on all the layers

The effective total number of frequencies excluding the frequencies of the PCell and SCell being monitored is Nfreq, which is defined as:

Nfreq = Nfreq, E-UTRA + Nfreq, UTRA + Mgsm + Nfreq, cdma2000 + Nfreq, HRPD

where

Nfreq, E-UTRA is the number of E-UTRA carriers being monitored (FDD and TDD)

Nfreq, UTRA is the number of UTRA carriers being monitored (FDD and TDD)

MGSM is an integer which is a function of the number of GSM carriers on which measurements are being performed. MGSM is equal to 0 if no GSM carrier is being monitored. For a MGRP of 40 ms, MGSM is equal to 1 if cells on up to 32 GSM carriers are being measured. For a MGRP of 80 ms, MGSM is equal to ceil(Ncarriers,GSM /20) where Ncarriers,GSM is the number of GSM carriers on which cells are being measured.

Nfreq, cdma2000 is the number of cdma2000 1x carriers being monitored

Nfreq, HRPD is the number of HRPD carriers being monitored

8.1.2.1.1.1	Maximum allowed layers for multiple monitoring

The UE shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, 3 FDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 1 FDD E-UTRA inter-frequency carrier for RSTD measurements, and

-	Depending on UE capability, 3 TDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 1 TDD E-UTRA inter-frequency carrier for RSTD measurements, and

-	Depending on UE capability, 3 FDD UTRA carriers, and

-	Depending on UE capability, 3 TDD UTRA carriers, and

-	Depending on UE capability, 32 GSM carriers (one GSM layer corresponds to 32 carriers), and

-	Depending on UE capability, 5 cdma2000 1x carriers, and

-	Depending on UE capability, 5 HRPD carriers

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 7 effective carrier frequency layers comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers.

<End changes>