3GPP TSG RAN WG4 Meeting #74	         R4-151161

Athens, Greece, February 9th – 13th, 2015

- Agenda Item:	5.1.2

- Source:	LG Electronics, Nokia Corporation

- Title:	Collection of MSD simulation result for CA_1A-3A and CA_1A-8A UE

Document for:	Approval

- Introduction

In this contribution, we collect MSD values from interested companies.

- Summary of MSD results

Table 1 shows the MSD levels from interested companies.

Table 1 Provided MSD levels for 2ULs inter-band CA band combinations

| Source Band combination |  |  |  |  |  | Nokia (R4-150636, R4-150637) | Qualcomm (R4-73AH-0104) | Media Tek (R4-151158) | LGE (R4-151159, R4-146947) | AVG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A_3A | 1 | 1950 | 5 | 25 | 2140 | 22.6 | 30 | 23 | 21.8 | 24.4 |
|  | 3 | 1760 | 5 | 25 | 1855 |  |  |  |  |  |
| CA_1A_8A | 1 | 1965 | 5 | 25 | 2155 | 7.2 | 5.4 | 5.9 | 4.3 | 5.7 |
|  | 8 | 887.5 | 5 | 25 | 932.5 |  |  |  |  |  |

From the collected MSD results, we propose MSD level in Table 2

Table2 Proposed MSD levels for B1+B3 and B1+B8

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_1A_3A | 1 | 1950 | 5 | 25 | 2140 | [24] | FDD |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  |
| CA_1A_8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  |

- Conclusions

In this contribution, we provide the MSD levels for B1+B3 and B1+B8 for 2ULs inter-band CA UE in Table 2.

- Reference

- R4-73AH-0104	“2UL inter-band CA MSD for B1+B3 and B1+B8”, Qualcomm

- R4-150636 	“MSD for 2UL CA_1A_3A”, Nokia

- R4-150637	“MSD for 2UL CA_1A_8A”, Nokia

- R4-151158	“MSD for 2UL CA_1A-3A and 1A-8A”, Media Tek

- R4-151159	“MSD for 2UL CA_1A-3A”, LG Electronics