3GPP TSG-RAN4 Meeting #74 	R4-150470

# Athens, Greece, 9 – 13 February 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.853 | CR | 2 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Coexistence Studies for 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B25_B41_B41-Core |  |  |  |  |  |  | Date: |  | 2015-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12. But the related text proposals were approved into the Rel-13 TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Put the corresponding BS materials into the Rel-12 TR to avoid unnecessary confusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and unnecessary confusion may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.X.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 41 DL carriers can be calculated as shown in Table 6.X.1-1 below:

Table 6.X.1-1: Band 41 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |
| 3rd order IMD products (with maximum channel bandwidth) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 2456 | 2730 |

It can be seen from Table 6.X.1-1 that the 3rd order IMD products of BS transmitting contiguous carriers in Band 41 may fall into the BS receive band of Bands 7, 30, 38, 40 and 41. Note that the calculation in Table 6.X.1-1 (except the last row) assumes the BS is transmitting with the whole 194 MHz DL frequency of Band 41. If the BS is only transmitting an up to 40 MHz DL in Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of the Band 30 or 40 as shown in the last row in Table 6.X.1-1.

Note that Bands 7 and 38 are not intended for use in the same geographical area as Band 41. Moreover, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the 3rd IMD products of BS transmitting two contiguous carriers in Band 41will not fall into the BS receive band of any operating band currently defined in the 3GPP standards for use in the same geographical area.

##### 6.X.2	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 25, Band 41 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.X.2-1 below:

Table 6.X.2-1: Band 25, Band 41 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 | 2496 | 2690 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2997 |  |  | 3450 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6922 |  |  | 7375 |  |  |

It can be seen from Table 6.X.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (25 + 41 + 41) may fall into the BS receive band of Bands 1, 2, 3, 4, 9, 10, 22, 23, 25, 33, 34, 35, 36, 37, 39 and 42. However, Bands 1, 3, 9, 10, 33, 34 and 39 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 35, 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 2, 4, 22, 23, 25 and 42. Note that the 3rd order IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 4, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (25 + 41 + 41) BS transmitter does not share the same antenna with Band 4, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 2, 23 and 25 UL, and hence Bands 2, 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (25 + 41 + 41) BS transmitter should not share the same antenna with Band 2, 23 or 25 BS receiver to prevent BS receiver desensitization, or with Band 4, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 4, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>