3GPP TSG-RAN WG4 #74	R4-151107

Athens, Greece, 9th Feb – 13th Feb 2015

Source: 	Ericsson, Samsung, LGE

Title:	Link level simulation assumption for TM10

Agenda Item:	7.5.2

Document for:	Information

# Introduction

In this contribution, link level simulation assumption for TM10 is provided. Interested companies are welcomed to provide link level analysis based on the provided link level simulation setup. The link level simulation assumptions are subjected to change in case any problems are identified.

# TM10 test setup for CRS-IC receiver

| Parameter |  | Unit | TP 1 | TP 2 | TP3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 | 0 | 0 |
|  | ρB | dB | 0 (Note 1) | 0 | 0 |
|  | σ | dB | -3 | -3 | -3 |
| Beamforming model |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal 0 configuration |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 1 configuration |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010001000000000 | N/A | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  |  | N/A | 2/ 0000010000000000 |  |
|  |  | dB | TBD | TBD | TBD |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 127 | 128 |
| Number of control OFDM symbols |  |  | 2 | 2 | 1 |
| Timing offset between TPs |  |  | N/A | Reference Value in Table 8.3.1.3.2-3 | [3] us |
| Frequency offset between TPs |  | Hz | N/A | [200] | [300]Hz |
| Number of allocated resource blocks |  | PRB | 50 | 50 | The load is TBD |
| PDSCH transmission mode |  |  | 10 | 10 | TM3 |
| Probability of occurrence of PDSCH transmission(Note 3) |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Note 1: Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. The probability of occurrence of PQI set in each TP is equal. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |

Table 1: Test Parameters for timing offset compensation with DPS transmission

Table 8.3.1.3.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 2 | CSI-RS 1 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.1.3.2-3: Performance Requirements with DPS transmission

| Test Number | Timing offset(us) | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | Option 1: 2us Option 2: -0.5 us | TBD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | 2x2 Low | 70 | TBD | ≥2 |
| Note 1: The propagation conditions for TP 1and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of both TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

# Conclusion

Companies are welcomed to provide link level simulation results based on the above test setup.