3GPP TSG-RAN WG4 #74bis	R4-151338

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Discussion on the time offset model for D2D demodulation requirements

Agenda Item:	6.5.2

Document for:	Discussion

# Introduction

In RAN4#74, time offset model issue was raised in [1] and [2]. However, there was no conclusion on how to model the time offset. Time offset model is important for D2D UE demodulation behavior. In this paper, we share our analysis on the time offset model for D2D demodulation requirements.

# Time offset model for D2D demodulation requirements

## System setup

In Figure 1, one system model for WAN and sidelink transmission and reception is given. In the system, there are two eNBs (eNB1 and eNB2) and two UEs (UE1 and UE2). Each eNB has one UE. UE1 is the D2D transmitter and UE2 is the D2D receiver.

Figure 1: System model for WAN and sidelink transmission and reception

In the figure, TUE1eNB1, TUE2UE1 and  TUE2eNB2 represents:

TUE1eNB1		The propagation delay between eNB1 and UE1

TUE2UE1		The propagation delay between UE1 and UE2

TUE2eNB2		The propagation delay between eNB2 and UE2

The system model described in Figure 1 is a general model. In the time offset model analysis, we can tune some parameters to enable the general model to capture the case where the UE1 and UE2 belong to the same eNB or the case where one UE is out of coverage. The details are shown in the following sections.

According to the timing specification given in 3GPP 36.211, the timing for the side link is given in Figure 2.  The quantity  differs between channels and signals according to

and for frame structure type 1  and  for frame structure type 2 unless stated otherwise.

Figure 2: Sidelink timing relation

If the UE has a serving cell fulfilling the S criterion according to [3, clause 5.2.3.2], the timing of reference radio frame  equals that of downlink radio frame  in the cell with the same uplink carrier frequency as the sidelink, otherwise the timing of reference radio frame  is implicitly obtained from [4].

## Time offset model communication

### Time offset model for communication mode 1

For communication mode 1, the synchronize signal and PSCCH is transmitted based on DL timing, and PSSCH is transmitted based on uplink timing, as shown in section 2.1. The timing relationship can be described as Figure 3.

Figure 3: The timing relationship between different channels and different link (including WAN link and D2D link)

In Figure 3, the Δ represents the time offset for the two different synchronization sources. For different scenarios, the Δ value may different, it can be given as:

- When UE1 and UE2 are in the same eNB, Δ=0

- When UE1 and UE2 are in different eNB and the eNBs are synchronized, Δ=+/-3us (The value given in 36.133 for TDD system can be set as reference)

- When UE1 and UE2 are in different eNB and the eNBs are asynchronous, Δ can be any value within +/-0.5 ms.

- When one of the UE is out of the coverage, the Δ can be any value within +/-0.5 ms.

In Figure 3, δ represents the synchronization error. According to 36.133, δ reflect D2D transmitter timing accuracy. When DL timing is used for D2D transmissions the transmit timing accuracy can be assumed to be equal to the ±12 Ts for BW ≥ 3MHz.

From Figure 3, we can see that the offset between the received synchronization signal timing and the received PSSCH timing is given by:

Offset1=TUE1eNB1+TUE2UE1+δ--TUE1eNB1+TUE2UE1+δ=2×TUE1eNB1

The offset between the received signal for WAN link and the received PSSCH is given by:

Offset2=Δ+TUE2eNB2--TUE1eNB1+TUE2UE1+δ=Δ+TUE2eNB2+TUE1eNB1-TUE2UE1-δ

For Offset1, if the site size is 1732m, the Offset1 is about 11.5 us for the worst case. For Offset2, as discussed in the above, the value can be any number within +/-0.5 ms. In the test, 0.5 ms could be considered for some test cases.

Proposal 1: The time advance for PSSCH compared with SA or synchronization signal (Offset1) shall be modelled for communication mode 1.

Proposal 2: The time advance for PSSCH compared with SA or synchronization signal is not necessary within CP, it could be more than CP. As one example, it can be set to 11.5 us for communication mode 1.

Proposal 3: The time offset between the side link and WAN link (Offset2) can be set as 0.5 ms in some test case for communication mode 1.

In Offset2, we can divide it into two parts:  Offset21 and  Offset22 . Offset21 is fixed for the same reception node, and Offset22 is varied for different transmitter, Offset21 and  Offset22 are given by:

Δ+TUE2eNB2Offset21+TUE1eNB1-TUE2UE1-δOffset22

After simple calculation, we can get:

maxOffset22=TUE1eNB1-δ

and

minOffset22=-TUE2UE1-δ

The maximum TUE1eNB1 and TUE2UE1 can be assumed as the maximum propagation delay within one cell. If the cell size is 1732 m, the maximum propagation delay could be 5.8 us, the range of Offset22 could be:

[-6.16 us, 6.16 us]

In this case, extended CP may be the proper configuration.

If the site size is 500 m, the maximum propagation delay could be 1.67 us and the range of Offset22 could be:

[-2.05 us, 2.05 us]

In this case, normal CP may be the proper configuration. This range is corresponding to the range of offset between different side links. In both cases, the above range is in the order of CP. Hence, the time offset between the side links can be set in the order of CP.

Observation 1: The time offset between the side links for communication mode 1 can be set in the order of CP if the CP is properly configured.

### Time offset model for communication mode 2

For communication mode 2, all the signals and channels are transmitted based on DL timing as shown in section 2.1. The timing relationship can be described as Figure 3.

Figure 4: The timing relationship between different channels and different link (including WAN link and D2D link)

Since all the signals and channels are transmitted based on the same timing, the offset between the received synchronization signal timing and the received PSSCH timing is given by:

Offset1=0

The offset between the received signal for WAN link and the received PSSCH is given by:

Offset2=Δ+TUE2eNB2-TUE1eNB1+TUE2UE1+δ=Δ+TUE2eNB2-TUE1eNB1-TUE2UE1-δ

For Offset2, it could be:

- When UE1 and UE2 are in the same cell, Offset2 is within [-δ,-2TUE2UE1-δ]

- When UE1 is in out-of-coverage, Offset2 could be within [-0.5, +0.5] ms.

- When the eNB are asynchronous, Offset2 could be within [-0.5, +0.5] ms

- When the two eNB are synchronized with some accuracy, Δ could be set as [-3us +3us], it will lead Offset2 exceed CP length with high probability.

To have more general coverage, Offset2 could be set as 0.5 ms in some test cases.

Based on discussion in section 2.2.1, if Offset1 and Offset2 are given as section 2.2.1, communication mode 2 is just a special case from reception point of view. In order to reduce the test case, we can just use communication mode 1 to verify the UE behavior.

Observation 2: If the time offsets are properly defined for communication mode 1, the reception behavior for communication mode 2 can be verified as well.

Similar like section 2.2.1, In Offset2, we can divide it into two parts:  Offset21 and  Offset22 . Offset21 is fixed for the same reception node, and Offset22 is varied for different transmitter. Offset21 and  Offset22 are given by:

Δ+TUE2eNB2Offset21-TUE1eNB1-TUE2UE1-δOffset22

After simple calculation, we can get:

maxOffset22=-δ

and

minOffset22=-2TUE1eNB1-δ

If the cell size is 1732 m, the maximum propagation delay could be 5.8 us, the range of Offset22 could be:

[-11.9 us 0.39 us]

If the cell size is 500 m, the maximum propagation delay would be 1.67 us, the range of Offset22 could be:

[-3.72 us 0.39 us]

This range is corresponding to the range of offset between different side links. Based on the above analysis, the time offset between the side links can be set in the order of CP.

Observation 3: The time offset between the side links for communication mode 2 can be set in the order of CP if the CP is properly configured.

### Remark on D2D communication

As described in [5], in practical system, there are multiple scenarios for communication as shown in Table 1.

Table 1: Multiple scenarios for communication

| Scenarios | Reception D2D UE | Transmission D2D UE |
| --- | --- | --- |
| 1 | in-coverage | in-coverage , intra-cell |
| 2 | in-coverage | in-coverage , inter-cell, synchronization |
| 3 | in-coverage | in-coverage , inter-cell, un-synchronization |
| 4 | in-coverage | out-of-coverage |
| 5 | out-of-coverage | in-coverage |
| 6 | out-of-coverage | out-of-coverage |

If one test case is designed for each scenario, it is really costly. From transmission point of view, there are some differences for Mode 1 and mode 2 communications with respect to resource allocation. However, from reception point of view, the same physical layer procedure for reception is applied for Mode 1 and mode 2 communications if the time offset is set in a general way.  Hence, for UE performance, it may be not necessary to elaborate the tests for all scenarios. Only one scenario is selected for communication. With scenarios 3, the parameters are generalized.

Proposal 4: The reception procedure is agnostic to the communication mode; scenario 3 can be used for the test setup.

### Time offset model for Discovery

For discovery, all the signals and channels are transmitted based on DL timing as discussed in Section 2.1. The timing relationship is similar like communication mode 2, which can be described as Figure 5.

Figure 5: The timing relationship between different channels and different link (including WAN link and D2D link)

Since synchronization signals and the PSDCH follow the same timing (DL timing), Hence, the offset between the received synchronization signal and the received PSDCH is given by:

Offset1=0

The offset between the received signal for WAN link and the received PSDCH is given by:

Offset2=Δ+TUE2eNB2-TUE1eNB1+TUE2UE1+δ=Δ+TUE2eNB2-TUE1eNB1-TUE2UE1-δ

Similar like Offset2 described in communication mode 2, for asynchronous network, the value could be within [-0.5, +0.5] ms, and for synchronized network or the case where both UEs are in the same cells, the value may be the order of CP length.

Proposal 5：In the test cases where the sidelink synchronization signals are transmitted, Offset2 could be in the order of 0.5 ms; In the test cases where the sidelink synchronization signals are not transmitted, Offset2 could be in the order of CP length.

When Offset2 is in the order of 0.5 ms, w1 could be selected in discSyncWindow. When Offset2 is in the order of CP length, w2 could be selected in discSyncWindow.

For the offset between the sidelinks, similar as observation 3, the time offset between the side links for discovery can be set in the order of CP if the CP is properly configured.

Hence, we suggest:

Proposal 6: The time offset between the side links for communication and discovery can be set in the order of CP if the CP is properly configured.

# Conclusion

In this paper, we provide our analysis on the time offset model for D2D demodulation, we have the following proposals:

Proposal 1: The time advance for PSSCH compared with SA or synchronization signal shall be modelled for communication mode 1.

Proposal 2: The time advance for PSSCH compared with SA or synchronization signal is not necessary within CP, it could be more than CP. As one example, it can be set to 11.5 us for communication mode 1.

Proposal 3: The time offset between the side link and WAN link can be set as 0.5 ms in some test case for communication mode 1.

Proposal 4: The reception procedure is agnostic to the communication mode; scenario 3 can be used for the test setup.

Proposal 5：In the test cases where the sidelink synchronization signals are transmitted, Offset2 could be in the order of 0.5 ms; In the test cases where the sidelink synchronization signals are not transmitted, Offset2 could be in the order of CP length.

Proposal 6: The time offset between the side links for communication and discovery can be set in the order of CP if the CP is properly configured.

In addition to some observations:

Observation 1: If the time offsets are properly defined for communication mode 1, the reception behavior for communication mode 2 can be verified as well.

Observation 2: If the time offsets are properly defined for communication mode 1, the reception behavior for communication mode 2 can be verified as well.

Observation 3: The time offset between the side links for communication mode 2 can be set in the order of CP if the CP is properly configured.

# Reference

- R4-150156, Discussion on D2D demodulation performance requirements, Intel, Feb. 2015.

- R4-150097, Discussion on time-offset for D2D demodulation requirements, Huawei, HiSilicon, Feb. 2015.

- 3GPP TS 36.304, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

- R4-150097, Discussion on time-offset for D2D demodulation requirements, Huawei, HiSilicon, Feb. 2015.