3GPP TSG-RAN WG4 Meeting #74bis 	R4-151494

Rio de Janeiro, Brazil, 20– 24  April, 2015

Source:	DISH Network

Title:	TP for section 7.1 (UE architecture protecting Band 1 requirements)

Agenda item:	8.1.2 [LTE_1980_2170_REG1]

Document for:	Approval

## Abstract

This document provides a TP for section 7.1 (UE architecture protecting Band 1 requirements) based on the revised WI agreed at RAN#67 and the agreed R4 way forward [1]

## 1	UE architecture protecting Band 1 requirements

In R4-151276 the following way forward was agreed in terms of the UE architecture to protect existing Band 1 requirements.

  - A 2x90 band plan is agreed [BXXX] 1920-2010 // 2110-2200MHz

  - The duplexer assumption for [BXXX] requirements specification will be:

    - The lower duplexer is LTE Band 1 (2x60MHz)

    - The upper duplexer is 2x90MHz

  - For a UE that supports both LTE Band 1 and the new band [BXXX], no relaxation is allowed  for Band 1 when all the carrier(s) are located within Band 1 operating frequency range

  - For BXXX, we study the specification impact of the following approaches:

    - Specifying requirements based on channel bandwidth assignment  (e.g. when all the channel bandwidth(s) is confined within LTE Band 1 frequency range, at least the Band 1 requirements apply)

    - UEs that support the new band [BXXX] shall also support LTE band 1 and all its capabilities

    - Other approaches are not precluded

In order to meet address these objective the working assumption will be a dual duplexer as shown below in Figure 1-1

Figure 1-1: Dual duplexer assumption for [BXXX]

The lower duplexer (B1) or Band 1 device, would meet all the legacy and performance requirements as per existing Band 1 terminals and ensure no relaxation is allowed when all the carrier(s) are located within the Band 1 operating frequency range.  Existing Band 1 requirements for regulatory and co-existence requirements will be maintained as well as NS_05 requirements to meet PHS co-existence.

The upper duplexer, would support the extended spectrum and would allow for new network signaled (NS_X) values to be defined to account for new deployment scenarios as per ECC report 52 for both the upper and lower band edges.

## 2		TP for TR 36.862

It is proposed that the attached text proposal in section 4 is approved and included in TR 36.862 sub-clause 7.1

## 3	References

R4-151276; Way forward on Region 1 MSS band; NTT DOCOMO, INC., KDDI, DISH, Solaris Mobile, KT, Nokia Corporation, LGE, Nokia Networks, Sony Mobile, Ericsson

## 4	TR36.862

------------- Start of text proposal ---------------

# 7	General issues

## 7.1	UE architecture protecting existing Band 1 requirements

In R4-151276 the following way forward was agreed in terms of the UE architecture to protect existing Band 1 requirements.

- A 2x90 band plan is agreed [BXXX] 1920-2010 // 2110-2200MHz

- The duplexer assumption for [BXXX] requirements specification will be:

    - The lower duplexer is LTE Band 1 (2x60MHz)

    - The upper duplexer is 2x90MHz

- For a UE that supports both LTE Band 1 and the new band [BXXX], no relaxation is allowed  for Band 1 when all the carrier(s) are located within Band 1 operating frequency range

- For BXXX, we study the specification impact of the following approaches:

    - Specifying requirements based on channel bandwidth assignment  (e.g. when all the channel bandwidth(s) is confined within LTE Band 1 frequency range, at least the Band 1 requirements apply)

    - UEs that support the new band [BXXX] shall also support LTE band 1 and all its capabilities

    - Other approaches are not precluded

In order to meet address these objective the working assumption will be a dual duplexer as shown below in Figure 7.1-1

Figure 7.1-1: Dual duplexer assumption for [BXXX]

The lower duplexer (B1) or Band 1 device, would meet all the legacy and performance requirements as per existing Band 1 terminals and ensure no relaxation is allowed when all the carrier(s) are located within the Band 1 operating frequency range.  Existing Band 1 requirements for regulatory and co-existence requirements will be maintained as well as NS_05 requirements to meet PHS co-existence.

The upper duplexer, would support the extended spectrum and would allow for new network signaled (NS_X) values to be defined to account for new deployment scenarios as per ECC report 52 for both the upper and lower band edges.

## 7.2	UE-to-UE co-existence issues with other 3GPP bands

------------- End of text proposal ---------------