3GPP TSG-RAN WG4 Meeting #74bis	 R4-151560

RIO DE JANEIRO, BRAZIL, APRIL 20 - 24, 2015

Agenda Item:	9.2.1

Source: 	Alcatel Lucent

Title: 	Doppler Shift for HST Scenario with Leaky Cable

Document for:	Discussion

# Introduction

In RAN4#74, two WFs were agreed: one on high speed train scenarios [1] and the other on evaluation for high speed train scenarios [2]. Although more detail for the parameters for the scenarios are still under discussion [3], the basic scenarios may be summarised in the following table

Table 1: Basic high speed train (HST) scenarios

| Scenarios | Environment | Method for extending eNB signals | RRH IDs | Repeater in Carriage |
| --- | --- | --- | --- | --- |
| 1 | Open space | RRHs | Same | No |
| 3 |  | No | N/A | Yes with leaky cable |
| 4 |  | No | N/A | No |
| 2a | Tunnel | RRHs | Same | Yes with leaky cable |
| 2b |  | RRHs | Different | Yes with leaky cable |
| 2c |  | Leaky cables | N/A | Yes with leaky cable |
| 2d |  | RRHs | Same | No |
| 2e |  | RRHs | Different | No |

As shown in the above table, from RF environment point of view, both open area and tunnel cases are considered. For tunnel environment, either RRHs or leaky table may be used to extend the eNB coverage. In the following, we discuss the downlink Doppler frequency shift when leaky cable is used to extend the eNB coverage, assuming the repeaters installed in the carriage only provide the function of amplifying the downlink RF power without changing the RF signals, such as compensating the Doppler frequency shift.

# Discussion

For high speed train (HST) scenarios, one main issue to be investigated is the Doppler frequency shift (DFS).

From downlink point of view, a UE may receive signals directly from a BS (e.g., scenario 3 and 4 in Table 1) or from a nearby RRH (e.g., scenario 1) or from the leaky cable installed along with the railway (scenario 2c). When a UE receives signals directly from a BS or an RRH, the downlink power to the UE is normally treated as from one direction, and a single Doppler shift model may be used for evaluating the UE’s performance. For example, in TS 36.101, under high speed train condition, the Doppler shift is given by

where  is the Doppler shift and  is the maximum Doppler frequency (Figure 1), with the Doppler shift trajectory shown in Figure 2.

Figure 1. Illustration of downlink Doppler shift

Figure 2. Doppler shift trajectory (TS 36.101 Figure B.3-1)

One of the common ways to provide radio coverage inside the tunnels for high speed trains is achieved using leaky (radiating) cable. This technique does not require special planning, and provides a good coverage in all types of tunnel. As shown in the figure 3, leaky leakage coaxial cable is cut periodically with the openings. Each of the openings is equivalent to a mini-antenna capable of sending and receiving RF signals.

Figure 3. Illustration of a radiating cable

For the scenario with leaky cables deployed in tunnel environment, the HST DFS may be different with the model presented in TS 36.101, since the same downlink RF signals do not come from one direction, but from both front and back directions (Figure 4). As shown in the figure, a repeater in the train may receive downlink signals from leaky cable in both front and back directions. A repeater is a device that receives, amplifies and transmits the radiated or conducted RF carrier both in the downlink direction and in the uplink direction. The repeater is basically a two-way amplifier. It receives signals on one port, amplifies the signals and directs them out to another port. The same procedure happens the other way around. Due to the speed of the train, the repeater received signal frequency for the signals transmitted from the front of the train will be higher than the BS transmitting signal frequency, and the received signal frequency for the signals transmitted from the back of the train will be lower than the BS transmitting signal frequency. Thus, the Doppler shift would be centred at the transmitting signal frequency with the Doppler spread in the range from [-,].

Figure 4: Doppler shift distribution when leaky cable is used under the same cell coverage

Here, we like to point out that while it is clear that the Doppler shift spread in the range from [-, +], the distribution of the received RF power within [-, +]  would depend on the channel path loss. The RF power distribution could be concentrated at the centre of carrier frequency (“ᴖ” shape), or could be at both sides of the Doppler shift spread(“ᴗ” shape). Further investigation may be needed if necessary.

When the train is moving at the overlapping coverage area of two cells, the Doppler shift spread will be different for the signals received from the two cells. Although the Doppler shift spread for signals from both cells will still in the range from [-, +] , but the Doppler shift spread for serving cell will have smaller range within [-, +]  (Figure 5).

Figure 5: Doppler shift distribution for with leaky cable under the overlapping coverage of two cells

# Summary

In this paper, we discussed the downlink Doppler frequency shift when leaky cable is used to extend the eNB coverage, with the assumption that the repeaters in the carriage without changing the DL RF frequency. In this case, the Doppler shift will not be a single Doppler shift trajectory is modelled.  Instead, the received downlink carrier frequency will be centred at the transmitting downlink carrier frequency with Doppler shift of [-, +] , where the  is the maximum Doppler shift.

# References

- RP-150515, “New SI proposal: Measurement gap enhancement”, Intel

- 3GPP TS 36.133, “E-UTRA: Requirements for support of radio resource management”.

- 3GPP TS 36.211, “E-UTRA: Physical channels and modulation”

- 3GPP TS 36.213, “Physical layer procedures”

- 3GPP TS 36.101, “E- User Equipment (UE) radio transmission and reception”

- 3GPP TS 36.104, “E-UTRA: Base Station (BS) radio transmission and reception”