3GPP TSG-RAN WG4 #74bis	R4-152055

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Discussion on the test cases and side condition for CRS-IM (non-TM10)

Agenda Item:	7.5.1

Document for:	Discussion

# Introduction

In RAN4#74 meeting, the group agreed one WF on how to move forward for the test cases and side condition setup for CRS-IM (non-TM10) [1]. In this paper, we provide link level simulation results for different combination of interference, modulation, and neighbor cell load and transmission mode. Based on the link level simulation results, some proposals are provided for the test cases and their corresponding side conditions.

# Link level simulation assumption

According to the agreements in [1], we have the following agreements:

- Reusing the FeICIC interference modelling parameters regarding number of interference cells, time offset and frequency offset.

- Non-colliding CRS shall be included in the tests setup

- For modulation, the following options can be considered:

  - Around MCS#5

  - Around MCS#14

  - Around MCS#20

- For resource utilization, the following options can be considered:

  - Option 1: 10%

  - Option 2: 20%

  - Option 3: 30%

  - Option 4: 50%

- The interference set

  - Option 1: the 1st set (corresponding to 5%-tile)

  - Option 2: The 10th set (corresponding to 50%-tile)

  - Option 3: The 16th set (corresponding to 80%-tile)

- Evaluate tests for following transmission mode

  - TM2, TM3, TM4, TM9

For resource utilization, in order to reduce the simulation efforts, only 10% and 30% are selected to investigate. For the interference set, the interference level for RU=30% is selected and applied for all other resource utilization for simplification. According to the interference modelling agreed in 36.863, as shown in Table 1, the interference level for different option is given in Table 2.

In summary, in Table 3, the key parameters are given according to the above agreements. The parameters with varied value are summarized in Table 4. In the next sections, simulation results are shown for selected cases.

Table 1: Average of the evaluation results on 5%-tile Geometry when RU=30% (a.k.a. Table 6.3.2-4 of 36.863)

| Set | Ior2Noc | Ior3Noc | Ior1Noc |
| --- | --- | --- | --- |
| 1 | 1.340471 | -0.73621 | 3.066401 |
| 2 | 3.922121 | 0.993835 | 4.484912 |
| 3 | 5.187652 | 1.922775 | 5.238305 |
| 4 | 6.05476 | 2.421963 | 5.779836 |
| 5 | 6.898605 | 3.003966 | 6.321041 |
| 6 | 7.553384 | 3.273023 | 6.799797 |
| 7 | 8.160243 | 2.876637 | 7.058987 |
| 8 | 8.630169 | 3.315201 | 7.446207 |
| 9 | 9.124317 | 3.271485 | 7.711411 |
| 10 | 9.693093 | 3.685917 | 8.182885 |
| 11 | 10.19666 | 3.780769 | 8.552738 |
| 12 | 10.78737 | 5.862466 | 9.369883 |
| 13 | 11.30776 | 5.396398 | 9.583789 |
| 14 | 11.96284 | 6.454303 | 10.30369 |
| 15 | 12.76359 | 7.823391 | 11.21865 |
| 16 | 13.99194 | 9.088526 | 12.36336 |
| 17 | 15.3589 | 10.74759 | 13.76737 |
| 18 | 17.44036 | 13.573 | 16.01893 |
| 19 | 21.18595 | 19.28102 | 20.15115 |
| 20 | 29.049 | 27.70898 | 28.10518 |

Table 2: The interference levels for different interference set

| Interference options | INR1 and INR2 value for each option |
| --- | --- |
| Option 1: the 1st set (corresponding to 5%-tile) | [INR1, INR2]= [1.34,-0.73] dB |
| Option 2: The 10th set (corresponding to 50%-tile) | [INR1, INR2]= [9.7,3.7] dB |
| Option 3: The 16th set (corresponding to 80%-tile) | [INR1, INR2]= [14,9.1] dB |

Table 3: Key parameters for the link level simulation

| Parameter |  |  |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |
| System bandwidth |  |  |  | 10 MHz |  |
| Duplex mode |  |  |  | FDD |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |
|  |  |  |  | Use different channel seed for between cells |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | INR1 | INR2 |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |
|  |  |  |  | Time-delay wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | [3 us] | [-1 us] |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | 300 Hz | -100 Hz |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modelled serving and the first two interfering cells |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |
|  |  |  |  | 0 dB in all modeled cells |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |
| Feedback mode |  |  |  | None |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |
|  |  | Modulation |  | Defined in |  |
|  |  | Code rate |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |
|  |  | CRC |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |
|  |  | PMI |  | TM3: N/A |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |
|  |  | Code rate |  | - |  |
|  |  | Channel coding, rate matching |  | - |  |
|  |  | CRC |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |
| Cyclic prefix |  |  |  | Normal |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |
|  |  |  | CRS-IM | Without CRS-IM |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |

Table 4: Varied parameters for the link level simulation

| Parameters | Varied value for each parameters |
| --- | --- |
| MCS | MCS=5(QPSK), or MCS=14(16QAM) or MCS=64QAM |
| Neighbour cell load | 10% or 30% |
| Interference level | Set 1: [INR1, INR2]= [1.34,-0.73] dB Set 2: [INR1, INR2]= [9.7,3.7] dB Set 3: [INR1, INR2]= [14,9.1] dB |
| Transmission mode | TM2, or TM3 or TM4 (rank-1) or TM9 |

# Link level simulation results

## Lower load vs higher load

In Figure 1 and Figure 2, link level results are given for different interference load with the same condition for other parameters. In this simulation, TM2, 64QAM and interference set 3 are used. When neighbour cell load is 10% percent, the gain is about 5.4 dB.  While when the load is 30% percent, the gain is about 1.74 dB.  With the load increase, the gain is shrunk. Similar observations have been observed for other transmission mode and modulation.

Figure 1: Link level results for TM2+64QAM+10%load+Interference set 3

Figure 2: Link level results for TM2+64QAM+30%load+Interference set 3

Observation 1:  The neighbour cell load has significant impact on the CRS-IC gain.

As agreed in [1], we have at least two purposes to define the CRS-IM test:

- Purpose 1: To achieve MMSE-IRC+CRS-IC gain with low partial load under homogeneous scenarios.

- Purpose 2: When the interference condition is not favorable for CRS-IC, there is no performance loss compared with MMSE-IRC when CRS assistance information is provided.

Based on observation 1, to serve the purpose 1, low neighbour cell load is preferable. Hence,

Proposal 1: In the test cases which are designed to achieve MMSE-IRC+CRS-IC gain with low partial load under homogeneous scenarios, 10% load is used.

## Lower MCS vs higher MCS

In Figure 3, Figure 4 and Figure 5, link level results are given for different MCS with the same condition for other parameters. In the simulation, TM2, 10% load and interference set 2 are used. When the MCS=20, the gain with CRS-IM is about 3.06 dB; When the MCS=14, the gain with CRS-IM is about 2.61 dB; When MCS=5, the gain with CRS-IM is about 2.35 dB.  With the decrease of the MCS, the gain is slightly decreased, but the gain is not so sensitive to the MCS. To have more coverage test, we can select 16QAM or 64QAM for different test cases.

Proposal 2: MCS=14 or MCS=20 can be selected for different test cases to increase the coverage.

Figure 3: Link level results for TM2+64QAM+10%load+Interference set 2

Figure 4: Link level results for TM2+16QAM+10%load+Interference set 2

Figure 5: Link level results for TM2+QPSK+10%load+Interference set 2

## Lower interference vs higher interference

In Figure 6, Figure 7 and Figure 8, link level results are given for different interference set with the same condition for other parameters. In the simulation, TM2, 10% load and 16QAM are used. When interference set 1 is used, the gain with CRS-IM is about 0.69 dB; when interference set 2 is used, the gain is about 2.61 dB; when interference set 3 is used, and the gain is about 5.14 dB. To serve the first purpose, interference set 3 is preferable. Moreover, when interference 3 is used, the second interference level is about 9 dB, it is strong enough. With this setup, two-cell CRS cancelation can be forced to be implemented. For the second test purpose, the interference set 1 can be considered.

Proposal 3: In the test cases which are designed to achieve MMSE-IRC+CRS-IC gain with low partial load under homogenous scenarios, interference set 3 is preferable. In the test cases which are designed for robustness test, interference set 1 can be considered.

Figure 6: Link level results for TM2+16QAM+10%load+Interference set 1

Figure 7: Link level results for TM2+16QAM +10%load+Interference set 2

Figure 8: Link level results for TM2+16QAM +10%load+Interference set 3

## The tabulation of CRS-IC gain

In Table 5, CRS-IC gain for different test cases and different side condition is tabulated. In the “CRS-IC” and no-CRS-IC column, the target SNR@70% throughput is given. The gain is equal to the target SNR@70% without CRS-IC minus the target SNR@70 with CRS-IC.

Table 5: CRS-gain for different test cases and different side conditions

| load | TM | INR1 (dB) | INR2 (dB) | Mod | CRS-IC (dB) | no-CRS-IC (dB) | Gain (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10% | TM2 | 9.7 | 3.7 | 64QAM | 9.66 | 12.72 | 3.06 |
|  |  | 14 | 9.1 | 64QAM | 10.49 | 15.89 | 5.40 |
|  |  | 1.34 | -0.73 | 64QAM | 9.27 | 9.80 | 0.53 |
|  |  | 9.7 | 3.7 | 16QAM | 4.90 | 7.51 | 2.61 |
|  |  | 14 | 9.1 | 16QAM | 5.60 | 10.75 | 5.14 |
|  |  | 1.34 | -0.73 | 16QAM | 4.27 | 4.96 | 0.69 |
|  |  | 9.7 | 3.7 | QPSK | -2.79 | -0.44 | 2.35 |
|  |  | 14 | 9.1 | QPSK |  | 2.63 |  |
|  |  | 1.34 | -0.73 | QPSK | -3.51 | -2.83 | 0.68 |
|  | TM3 | 9.7 | 3.7 | 64QAM | 18.43 |  |  |
|  |  | 14 | 9.1 | 64QAM | 18.73 |  |  |
|  |  | 1.34 | -0.73 | 64QAM | 17.54 | 17.50 | -0.05 |
|  |  | 9.7 | 3.7 | 16QAM | 11.54 | 14.55 | 3.01 |
|  |  | 14 | 9.1 | 16QAM | 12.27 | 17.88 | 5.61 |
|  |  | 1.34 | -0.73 | 16QAM | 11.19 | 11.63 | 0.43 |
|  |  | 9.7 | 3.7 | QPSK | 1.48 | 4.20 | 2.71 |
|  |  | 14 | 9.1 | QPSK | 2.20 | 7.67 | 5.47 |
|  |  | 1.34 | -0.73 | QPSK | 0.88 | 1.50 | 0.63 |
|  | TM4 | 9.7 | 3.7 | 64QAM | 10.41 | 13.45 | 3.04 |
|  |  | 14 | 9.1 | 64QAM | 11.18 | 16.84 | 5.66 |
|  |  | 1.34 | -0.73 | 64QAM | 9.96 | 10.56 | 0.59 |
|  |  | 9.7 | 3.7 | 16QAM | 5.48 | 8.22 | 2.74 |
|  |  | 14 | 9.1 | 16QAM | 6.64 | 11.41 | 4.78 |
|  |  | 1.34 | -0.73 | 16QAM | 4.95 | 5.54 | 0.59 |
|  |  | 9.7 | 3.7 | QPSK |  | -0.07 |  |
|  |  | 14 | 9.1 | QPSK |  | 3.10 |  |
|  |  | 1.34 | -0.73 | QPSK | -3.13 | -2.52 | 0.61 |
|  | TM9 | 9.7 | 3.7 | 64QAM | 12.47 | 15.72 | 3.25 |
|  |  | 14 | 9.1 | 64QAM | 13.37 | 19.26 | 5.89 |
|  |  | 1.34 | -0.73 | 64QAM | 12.03 | 12.43 | 0.41 |
|  |  | 9.7 | 3.7 | 16QAM | 7.23 | 9.91 | 2.68 |
|  |  | 14 | 9.1 | 16QAM | 8.00 | 13.26 | 5.25 |
|  |  | 1.34 | -0.73 | 16QAM | 6.51 | 7.11 | 0.60 |
|  |  | 9.7 | 3.7 | QPSK | -0.71 | 1.70 | 2.42 |
|  |  | 14 | 9.1 | QPSK |  |  |  |
|  |  | 1.34 | -0.73 | QPSK | -1.49 | -0.88 | 0.62 |
| 30% | TM2 | 9.7 | 3.7 | 64QAM | 12.41 | 14.15 | 1.74 |
|  |  | 14 | 9.1 | 64QAM | 16.15 | 17.89 | 1.74 |
|  |  | 1.34 | -0.73 | 64QAM | 10.39 | 10.80 | 0.42 |
|  |  | 9.7 | 3.7 | 16QAM | 7.94 | 9.43 | 1.49 |
|  |  | 14 | 9.1 | 16QAM | 11.97 | 13.24 | 1.27 |
|  |  | 1.34 | -0.73 | 16QAM | 5.39 | 5.91 | 0.52 |
|  |  | 9.7 | 3.7 | QPSK | 0.92 | 1.97 | 1.04 |
|  |  | 14 | 9.1 | QPSK | 5.04 | 6.00 | 0.96 |
|  |  | 1.34 | -0.73 | QPSK | -2.31 | -1.82 | 0.49 |
|  | TM3 | 9.7 | 3.7 | 64QAM |  |  |  |
|  |  | 14 | 9.1 | 64QAM |  |  |  |
|  |  | 1.34 | -0.73 | 64QAM | 18.49 | 18.42 | -0.07 |
|  |  | 9.7 | 3.7 | 16QAM | 14.40 | 16.09 | 1.69 |
|  |  | 14 | 9.1 | 16QAM | 18.06 | 19.98 | 1.92 |
|  |  | 1.34 | -0.73 | 16QAM | 12.23 | 12.59 | 0.36 |
|  |  | 9.7 | 3.7 | QPSK | 4.95 | 6.14 | 1.19 |
|  |  | 14 | 9.1 | QPSK | 8.99 | 10.19 | 1.20 |
|  |  | 1.34 | -0.73 | QPSK | 1.91 | 2.52 | 0.61 |
|  | TM4 | 9.7 | 3.7 | 64QAM | 12.65 | 14.87 | 2.22 |
|  |  | 14 | 9.1 | 64QAM | 16.12 | 18.50 | 2.38 |
|  |  | 1.34 | -0.73 | 64QAM | 11.00 | 11.36 | 0.36 |
|  |  | 9.7 | 3.7 | 16QAM | 8.16 | 9.76 | 1.60 |
|  |  | 14 | 9.1 | 16QAM | 12.05 | 13.58 | 1.53 |
|  |  | 1.34 | -0.73 | 16QAM | 5.86 | 6.46 | 0.61 |
|  |  | 9.7 | 3.7 | QPSK | 0.99 | 1.96 | 0.97 |
|  |  | 14 | 9.1 | QPSK | 4.99 | 5.93 | 0.94 |
|  |  | 1.34 | -0.73 | QPSK | -2.13 | -1.56 | 0.57 |
|  | TM9 | 9.7 | 3.7 | 64QAM | 15.18 | 17.25 | 2.08 |
|  |  | 14 | 9.1 | 64QAM | 18.48 |  |  |
|  |  | 1.34 | -0.73 | 64QAM | 13.09 | 13.32 | 0.22 |
|  |  | 9.7 | 3.7 | 16QAM | 10.20 | 11.77 | 1.57 |
|  |  | 14 | 9.1 | 16QAM | 14.01 | 15.60 | 1.59 |
|  |  | 1.34 | -0.73 | 16QAM | 7.64 | 8.15 | 0.51 |
|  |  | 9.7 | 3.7 | QPSK | 3.11 | 4.09 | 0.98 |
|  |  | 14 | 9.1 | QPSK | 7.01 | 8.28 | 1.27 |
|  |  | 1.34 | -0.73 | QPSK | -0.29 | 0.21 | 0.50 |

# Discussion on the side condition setup and test case list

In [1], for PDSCH, TM2, TM3, TM4, TM9 and TM10 are agreed to be considered for test cases. We can have the following test cases and its corresponding additional test purposes as shown in

Table 6: Test case list and its corresponding side condition

| Test case | TM | Test purposes | Side condition |
| --- | --- | --- | --- |
| 1 | TM2 | - To achieve MMSE-IRC+CRS-IC gain - To cover CRS-based open loop transmission scheme | 10%load + 64QAM + Int. set 3 |
| 2A | TM3 | - To achieve MMSE-IRC+CRS-IC gain - To cover two-layer transmission | 10%load+16QAM+Int. set 3 (note 1) |
| 2B | TM3 | - To achieve robustness test | 30%load+16QAM+Int. set 1(note 1) |
| 3 | TM4 | - To achieve MMSE-IRC+CRS-IC gain - To cover close loop MIMO | 10%load + 16QAM + Int. set 3 |
| 4 | TM9 | - To achieve MMSE-IRC+CRS-IC gain - To cover DMRS based scheme | 10%load + 64QAM + Int. set 3 |
| others | TM10 | TBD | TBD |
| Note 1: For two-layer transmission, it is better not to use 64QAM. For 64QAM, 70% throughput cannot be achieved within 20 dB. To avoid the high SNR problem, we can select 16QAM for two-layer test. |  |  |  |

# Conclusion

In this paper, we provide link level simulation based on the agreed link level simulation assumption. Based on the results, we have the following proposals:

Proposal 1: In the test cases which are designed to achieve MMSE-IRC+CRS-IC gain with low partial load under homogeneous scenarios, 10% load is used.

Proposal 2: MCS=14 or MCS=20 can be selected for different test cases to increase the coverage.

Proposal 3: In the test cases which are designed to achieve MMSE-IRC+CRS-IC gain with low partial load under homogenous scenarios, interference set 3 is preferable. In the test cases which are designed for robustness test, interference set 1 can be considered.

Based on the proposals, we give detail side condition for different test cases.

# Reference

- R4-151095, WF on CRS-IM demodulation and CSI for non-TM10 TMs, Ericsson, Samsung, NSN, Qualcomm, Huawei, MediaTek, Feb, 2015.