3GPP TSG-RAN WG4 #74bis	R4-152058

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Link level simulation assumption for MTC RLM

Agenda Item:	6.1.1

Document for:	Discussion

# Introduction

In RAN4#74 meeting, link level simulation assumptions were agreed in [1]. In this contribution, we provide our link level simulation results for MTC RLM based on the agreed link level simulation. Based on the link level results, SNR1~SNR5 are proposed.

# Link level simulation results for RLM (FDD)

Simulation assumptions for FDD RLM are shown in Table 1 and Table 2. In Figure 1, link level results for out-of-sync are given. In Figure 2, link level results for in-sync are given. The target SNR@10% BLER for out-of-sync and the target SNR@2% BLER for in-sync are tabulated in Table 3.

Table 1: PDCCH/PCFICH transmission parameters for out-of-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1A |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4; Bandwidth = 1.4 MHz 8; Bandwidth 3 MHz |
| Ratio of PDCCH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 4 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Correlation Matrix and Antenna Configuration | 2x1 Low |
| E-UTRA Channel Bandwidth | 10 MHz |
| Channel model | ETU 70, AWGN |
| Note 1: DCI format 1A is defined in clause 5.3.3.1.3 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 2: PDCCH/PCFICH transmission parameters for in-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1C |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4 |
| Ratio of PDCCH RE energy to average RS RE energy | 1 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Antenna configuration | 2x1 |
| Channel model | ETU 70, AWGN |
| Note 1: DCI format 1C is defined in clause 5.3.3.1.4 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Figure 1: PDCCH performance based on Out Sync assumption (FDD)

Figure 2: PDCCH performance based on In Sync assumption (FDD)

Table 3: Target SNR for RLM (FDD)

| Duplex | RLM | Channel | Target SNR |
| --- | --- | --- | --- |
| FDD | Out_sync | AWGN | -12.2 |
|  |  | ETU70 | -8.5 |
|  | In_Sync | AWGN | -8.5 |
|  |  | ETU70 | -4.5 |

Reusing Rel-8 margin and methodology for SNR1~SNR5, SNR1~SNR5 is given in Table 4.

Table 4: proposed SNR1~SNR4 for FDD RLM

|  | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
| AWGN | -6 | -9.2 | -15.2 | -11 | -6 |
| ETU70 | -2 | -5.5 | -11.5 | -7 | -2 |

# Link level simulation results for RLM (TDD)

Simulation assumptions for FDD RLM are shown in Table 5 and Table 6. In Figure 3, link level results for out-of-sync are given. In Figure 4, link level results for in-sync are given. The target SNR@10% BLER for out-of-sync and the target SNR@2% BLER for in-sync are tabulated in Table 7.

Table 5: PDCCH/PCFICH transmission parameters for out-of-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1A |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4; Bandwidth = 1.4 MHz 8; Bandwidth 3 MHz |
| Ratio of PDCCH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 4 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Correlation Matrix and Antenna Configuration | 2x1 Low |
| E-UTRA Channel Bandwidth | 10 MHz |
| Channel model | ETU 70, AWGN |
| Special subframe configuration | 6 |
| Uplink-downlink configuration | 1 |
| Note 1: DCI format 1A is defined in clause 5.3.3.1.3 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 6: PDCCH/PCFICH transmission parameters for in-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1C |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4 |
| Ratio of PDCCH RE energy to average RS RE energy | 1 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Antenna configuration | 2x1 |
| Channel model | ETU 70, AWGN |
| Special subframe configuration | 6 |
| Uplink-downlink configuration | 1 |
| Note 1: DCI format 1C is defined in clause 5.3.3.1.4 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Figure 3: PDCCH performance based on Out Sync assumption (TDD)

Figure 4: PDCCH performance based on In Sync assumption (TDD)

Table 7: Target SNR for RLM (TDD)

| Duplex |  | Channel | Target SNR |
| --- | --- | --- | --- |
| TDD | Out_sync | AWGN | -11 |
|  |  | ETU70 | -6.3 |
|  | In_Sync | AWGN | -7.4 |
|  |  | ETU70 | -3.1 |

Reusing Rel-8 margin and methodology for SNR1~SNR5, SNR1~SNR5 are given in Table 8.

Table 8: proposed SNR1~SNR4 for TDD RLM

|  | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
| AWGN | -4.9 | -8 | -14 | -9.9 | -4.9 |
| ETU70 | -0.6 | -3.3 | -9.3 | -5.6 | -0.6 |

# Link level simulation results for RLM (HD-FDD)

For HD-FDD, since there is one DL subframe per frame, the performance is expected to be close to TDD. Hence, for HD-FDD, TDD results are reused here, as shown in Table 11.

Table 9: PDCCH/PCFICH transmission parameters for out-of-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1A |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4; Bandwidth = 1.4 MHz 8; Bandwidth 3 MHz |
| Ratio of PDCCH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 4 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Correlation Matrix and Antenna Configuration | 2x1 Low |
| E-UTRA Channel Bandwidth | 10 MHz |
| Channel model | ETU 70, AWGN |
| Downlink subframes | One DL subframe (any subframe out of #0 to #9) per frame, which is randomly decided by the test system when executing the tests |
| Note 1: DCI format 1A is defined in clause 5.3.3.1.3 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 10: PDCCH/PCFICH transmission parameters for in-sync (LC_MTC)

| Attribute | Value |
| --- | --- |
| DCI format | 1C |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 5 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4 |
| Ratio of PDCCH RE energy to average RS RE energy | 1 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell. |
| Antenna configuration | 2x1 |
| Channel model | ETU 70, AWGN |
| Downlink subframes | One DL subframe (any subframe out of #0 to #9) per frame, which is randomly decided by the test system when executing the tests |
| Note 1: DCI format 1C is defined in clause 5.3.3.1.4 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 11: Target SNR for RLM (HD-FDD)

| Duplex |  | Channel | Target SNR |
| --- | --- | --- | --- |
| HD-FDD | Out_sync | AWGN | -11 |
|  |  | ETU70 | -6.3 |
|  | In_Sync | AWGN | -7.4 |
|  |  | ETU70 | -3.1 |

Reusing Rel-8 margin and methodology for SNR1~SNR5, SNR1~SNR5 are given in Table 12.

Table 12: proposed SNR1~SNR4 for HD-FDD RLM

|  | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
| AWGN | -4.9 | -8 | -14 | -9.9 | -4.9 |
| ETU70 | -0.6 | -3.3 | -9.3 | -5.6 | -0.6 |

# Conclusion

In this paper, Link level results for MTC RLM are provided. Based on the link level simulation results and the Rel-8 methodology and margin for SNR1~SNR5, SNR1~SNR5 are proposed. We hope the group can consider the proposal for SNR1~SNR5.

# Reference

- R4-151050, Link level simulation assumption for MTC RLM, Ericsson, Feb, 2015.