3GPP TSG-RAN WG4 #74bis	R4-152352

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Discussion on the control channel test for CRS-IM

Agenda Item:	7.5.1

Document for:	Discussion

# Introduction

In RAN4#74 meeting, one WF paper about CRS-IM was approved [1]. One open issue is about the control channel.  In this paper, we provide some analysis on the PDCCH performance with CRS-IM.

# Consideration on simulation setup for control channel

## PDCCH/PCFICH

In Table 1, the link level simulation parameters are provided. Most of the parameters reuse FeICIC setup except the interference profile and cell ID configuration. Here, non-colliding CRS is configured. Link level results are provided in Figure 1 and Figure 2. From Figure 1 and Figure 2, we can see that with INR1=1.34 dB and INR2=-0.73 dB, the gain is about 1.5 dB. When INR1=9.7 dB and INR2 = 3.7 dB, the gain is about 6 dB.

Table 1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port | Noc | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.4-2 | TBD | TBD |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 128 | 1 |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| Number of PHICH groups (Ng) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 8: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. |  |  |  |  |  |

From implementation point of view, CRS-IC has been implemented in FeICIC. There is not too much efforts to apply the corresponding implementation to homogenous case.

Figure 1: PDCCH performance with INR1=1.34 dB and INR2=-0.73 dB

Figure 2: PDCCH performance with INR1=9.7 dB and INR2=3.7 dB

Based on the results, we have the following observations:

- Observation 1: With 2-cell CRS-IC, more than 2 dB gain is observed with 10% load and medium interference level (corresponding to the 10th bin).

- Observation 2:  With 2-cell CRS-IC, at least 0.7 dB can be achieved for PDCCH for 30% load and lower interference level (corresponding to the 1st bin).

## PHICH

In Table 2, the link level simulation parameters are provided. Most of the parameters reuse FeICIC setup except the interference profile and cell ID configuration. Here, non-colliding CRS is configured. Link level results are provided in Figure 3 and Figure 4. From Figure 3 and Figure 4, we can see that with INR1=1.34 dB and INR2=-0.73 dB, the gain is about 2 dB. When INR1=9.7 dB and INR2 = 3.7 dB, the gain is about 7dB.

Table 2: Test Parameters for PHICH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port | Noc | dBm/15kHz | -98 (Note 1) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.5.1.2.4-2 | TBD | TBD |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 128 | 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| Number of PHICH groups (Ng) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 8: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell 2 and Cell 3 in the test. |  |  |  |  |  |

Figure 3: PHICH performance with INR1=1.34 dB and INR2=-0.73 dB

Figure 4: PDCCH performance with INR1=9.7 dB and INR2=3.7 dB

Based on the results, we have the following observations:

Observation 3:  With 2-Cell CRS-IC, more than 4 dB performance gain is observed with 10% load and medium interference level (corresponding to the 10th bin).

Observation 4：With 2-Cell CRS-IC, more than 2 dB performance gain is observed with 30% load and low interference condition (corresponding to the 1st bin).

Based on observation 1 ~ 4, we have the following proposals:

Proposal 1: PDCCH/PCFICH and PHICH performance requirements are defined for CRS-IM.

# Conclusion

In this paper, link level simulation results are provided for control channel with/without CRS-IC. Based on the link level simulation, we have the following observations:

Observation 1: With 2-cell CRS-IC, more than 2 dB gain is observed with 10% load and medium interference level (corresponding to the 10th bin).

Observation 2:  With 2-cell CRS-IC, at least 0.7 dB can be achieved for PDCCH for 30% load and lower interference level (corresponding to the 1st bin).

Observation 3:  With 2-Cell CRS-IC, more than 4 dB performance gain is observed with 10% load and medium interference level (corresponding to the 10th bin).

Observation 4：With 2-Cell CRS-IC, more than 2 dB performance gain is observed with 30% load and low interference condition (corresponding to the 1st bin).

Based on these observations, we have the following proposal:

Proposal 1: PDCCH/PCFICH and PHICH performance requirements are defined for CRS-IM.

# Reference

- R4-151095, WF on CRS-IM demodulation and CSI for non-TM10 TMs, Ericsson, Samsung, NSN, Qualcomm, Huawei, MediaTek, Feb, 2015.