3GPP TSG-RAN WG4 Meeting #74bis                        R4-151901

# Rio de Janeiro, Brazil, 20 – 24 Apr, 2015

Agenda Item:	7.3.1

Source: 	CMCC, CATR, Bluetest

Title: 			   TDD eNodeB emulator settings for MIMO OTA test

Document for:	Approval

- Introduction

In RAN4#66 meeting, contribution R4-130881 was approved to define TDD eNodeB emulator settings for MIMO OTA test. However, some test houses found this configuration is difficult to keep connection between eNodeB and DUT.

Recently, RAN4 MIMO OTA subgroup has decided to carry out harmonization measurement campaign during the following meetings. It becomes necessary to modify TDD eNodeB emulator settings to make this measurement campaign carried out successfully.

In this contribution, we propose the eNodeB emulator setting for TR37.977 and harmonization measurement campaign.

- TDD eNodeB emulator setting consideration

RMC defined in TS36.521-1 were chosen for FDD and TDD MIMO OTA test in TR37.977. However, problems were found during TDD MIMO OTA real test especially when the anechoic chamber has a large size. The throughput of other TDD RMC configurations with 20MHz bandwidth and 64QAM are either too high or too low, thus existing TDD RMC configurations are not suitable for MIMO OTA test. A new RMC for TDD MIMO OTA test needs to be defined.

CCSA as the China standard organization has published its first edition of MIMO OTA test specification at the end of 2014 [3]. In this specification TDD eNodeB emulator settings were defined and tests can carry out easily with proper throughput in AC and RC.

It would be helpful for the UE venders and certification groups if the eNodeB emulator settings in 3GPP and CCSA could be aligned. Based on the above considerations, we recommend the eNodeB emulator settings below to be adopted in TR37.977 and used in harmonization measurement campaign.

Table 7.1-2: Settings for TDD eNodeB emulator

| eNodeB settings | Unit | Value |
| --- | --- | --- |
| Physical channel |  |  |
| Connection mode of UE |  | Connection established |
| DL MIMO mode |  | 2 x 2 open loop spatial multiplexing |
| Duplex mode |  | TDD |
| Operating band (UL / DL channel) |  | Band 38 (38000) Band 39 (38450) Band 40 (39150) Band 41 (40620) |
| Schedule type |  | Reference Measurement Channel (RMC) |
| Reference Channel |  | Table 7.1-3 |
| Up/Downlink Frame Configuration |  | 1 |
| Special Frame configuration |  | 7 |
| Bandwidth DL | MHz | 20 |
| Number of RBs DL |  | 100 |
| Start RB DL |  | 0 |
| Modulation DL |  | 64QAM |
| TBS Idx DL |  | 16 (RMC defined, Note 1) |
| Bandwidth UL | MHz | 20 |
| Number of RBs UL |  | 100 |
| Start RB UL |  | 0 |
| Modulation UL |  | QPSK |
| TBS Idx UL |  | 6 |
| Transmit power control | dBm | 10/20 MHz, open loop (Note 2) |
| PDSCH power offset relative to RS EPRE | dB | A = -3 B = -3 |
| Number of HARQ transmissions |  | 1 (no HARQ re-transmissions) |
| AWGN |  | OFF |
| DL power level (RS EPRE) | dBm / 15 kHz | Set at eNodeB simulator with correction from calibration |
| Number of subframes for FOM measurement |  | 2000 minimum for static channel 20000 minimum for faded channel (Note 3) |
| NOTE 1: This RMC is defined in Table 7.1-3. Subframes 0, 1, 4, 6 and 9 utilize DL TBS 16, subframe 5 is unused for DL data transimission and other subframes are for UL. NOTE 2: No uplink power control. NOTE 3: These values might need to be increased for frequency and mobile speed reasons. |  |  |

Table 7.1-3: Fixed Reference Channel two antenna ports for 20MHz TD-LTE

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks (Note 3) |  | 100 |
| Uplink-Downlink Configuration (Note 1) |  | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 4+2 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 0.4 |
| Information Bit Payload (Note 3) |  |  |
| For Sub-Frames 4,9 | Bits | 32856 |
| For Sub-Frames 1,6 |  | 24496 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 32856 |
| Number of Code Blocks (Notes 2 and 3) |  |  |
| For Sub-Frames 4,9 |  | 5 |
| For Sub-Frames 1,6 |  | 4 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  | 5 |
| Binary Channel Bits (Note 3) |  |  |
| For Sub-Frames 4,9 | Bits | 82800 |
| For Sub-Frames 1,6 |  | 67968 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 80712 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 14.756 |
| UE Category |  | ≥ 1 |
| Note 1: As per Table 4.2-2 in TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: Given per component carrier per codeword |  |  |

- Conclusion

In this contribution, we discuss the TDD eNodeB emulator settings for MIMO OTA. It is proposed that:

Proposal: The new TDD eNodeB settings are adopted in TR37.977 and used in harmonization measurement campaign.

- Reference

- 3GPP TR 37.977

- 3GPP TS36.521-1 UE conformance specification Radio transmission and reception Part 1: Conformance Testing

- CCSA, Performance requirement and measurement method for MIMO antenna terminal Part 1：LTE wireless terminal