3GPP TSG-RAN WG4 #74bis	R4-152063

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Further discussion on eIMTA CSI test

Agenda Item:	6.2.3

Document for:	Discussion

# Introduction

In RAN4 #72bis meeting, some agreements for CSI/demodulation test setup were achieved and captured in [1] as:

- CSI requirements

  - No new PMI, and RI tests are defined for LTE TDD eIMTA as agreed in R4-145371

  - At least following test cases are agreed to be introduced for UEs which support feature group#7-3 only:

      - Test case 1A: TM1-9 aperiodic CQI requirement with below test purposes:

        - UE’s capability to handle Rel-12 CSI subframe set configuration

        - UE’s capability to support new aperiodic CSI reporting procedure based on 2 bit CSI request field.

        - UE’s capability to buffer aperiodic CSI measurements in case when the CSI reference resource is before the A-CSI trigger

        - UE’s capability to handle PDSCH rate matching around two ZP-CSI-RS configurations.

      - Test case 1B:TM10 aperiodic CQI requirement with below test purposes:

        - UE’s capability to handle Rel-12 CSI subframe set configuration

        - UE’s capability to support new aperiodic CSI reporting procedure based on 2 bit CSI request field.

        - UE’s capability for TM10 CSI measurement with 2 CSI-IM per CSI process

        - UE’s capability to buffer aperiodic CSI measurements in case when the CSI reference resource is before the A-CSI trigger

  - Additionally, it is agreed to introduce test case(s) for UEs which support feature group#7-1 &7-3 for TM1-9. The test details are FFS.

In [2] and [3], the CSI setup are further refined.  In this paper, based on the latest agreements, we refine our view on the test setup for CSI test.

# Test setup for CSI

## Test case 1A: Non-TM10 aperiodic CQI reporting requirement for feature group #7-3 only

For aperiodic CSI test, test case for PUSCH 3-1 (CSI reference symbol)  (Section 9.3.1.2.2 of 36.101) could be re-used as shown in Table 1. In the setup, two Rel-12 subframe sets are configured and two ZP-CSI-RS are configured. The Rel-12 subframe set is

0 0 0 1 1 1 1 0 1 0.

The example is shown in Figure 1. In this example, there are two CSI subsets, subframe 3, 4, 5, and 8 belong to CSI subset 1, and subframe 0, 1, 6, and 9 belong to CSI subset 2. Subframe 3 and subframe 8 are the downlink subframes with the capability to trigger CSI report.

Further, in order to verify the UE’s capability to buffer aperiodic CSI measurements in case when the CSI reference resource is before the A-CSI trigger, Subframe 3 is used for scheduling a report whose associated CSI reference resource is in the different CSI subset from the triggering downlink subframe. For example, subframe 3 triggers a report for CSI subset 2. In order to avoid some UE to made short cut and just use the CSI measurements in the latest downlink subframes rather than buffer the CSI measurements according to the specification in case the CSI reference resource is before the A-CSI trigger, we can set all downlink subframes after A-CSI trigger but before the reported uplink subframe with the same interference level as the downlink subframe with the trigger. In this example, subframe 4, 5 are configured with the same interference level as subframe 3.

Figure 1: Aperiodic CSI test setup for TM2

For transmission mode, we prefer to use CRS-based transmission scheme for test case 1A. Since CSI reference resource is before trigger, CRS-based transmission scheme may need buffer more channel state information. Hence, it is more challenge for the UE implementation. Hence, CRS-based transmission scheme is more valuable for this test.

Table 1: Sub-band test for TDD

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 2 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| SNR (Note 3) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 (set0) -108 (set 1) | -98 (set0) -108 (set 1) |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| CRS reference signals |  |  | Antenna port 0, 1 |  |
| CSI reference signals |  |  | Antenna port 15,16 |  |
| tdd-Config in SIB-1 |  |  | 2 |  |
| csi-MeasSubframeSet-r12 |  |  | 0001111010 |  |
| Zero power CSI reference signal (ZPId=1) |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 1000010000000000 |  |
|  |  | CSI-RS subframe configuration ICSI-RS | 0 |  |
| Zero power CSI reference signal (ZPId=2) |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 1000010000000000 |  |
|  |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 3 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |
| CSI-RS reference signal configuration |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |
| Reporting interval (Note 4) |  | ms | 5 |  |
| CQI delay |  | ms | TBD |  |
| Reporting mode |  |  | PUSCH 3-0 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 and #8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2 and #7. |  |  |  |  |

## Test case 1B: TM10 aperiodic CQI reporting requirement for feature group #7-3 only

As agreed in [2], the test purpose of Test 1B is:

  - UE’s capability to handle Rel-12 CSI subframe set configuration

  - UE’s capability to support new aperiodic CSI reporting procedure based on 2 bit CSI request field.

  - UE’s capability for TM10 CSI measurement with 2 CSI-IM per CSI process

  - UE’s capability to buffer aperiodic CSI measurements in case when the CSI reference resource is before the A-CSI trigger

For this test, the test case designed for Minimum requirement PUCCH 1-1 (With Single CSI Process) (Section 9.2.4.2 of 36.101) can be reused with minor modification. The modification is as shown in Table 2 marked as yellow background. The main modification is to add the Rel-12 subframe set, and configure two CSI-IMs. The BLER metric can be reused for each subframe set. Other metric can be further discussed.

Table 2: PUCCH 3-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 |  |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation (Note 1) |  | dB | 0 | 0 |  |
|  |  | dB | 0 | 0 |  |
|  | Pc | dB | -6 | -6 |  |
|  |  | dB | -3 | N/A |  |
| Cell ID |  |  | 0 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 | (Note 2) |  |
| csi-MeasSubframeSet-r12 |  |  | 0001000010 | N/A |  |
| Interference model |  |  | Use different interference level for two CSI subframe set |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 | N/A |  |
| CSI-RS configuration |  |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  |
| The second Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0010000000000000 | 3 / 1000010000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | N/A |  |
| The second CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  |
| SNR (Note 3) |  | dB | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 (set 0) -108 (set 1) |  |  |
| Modulation / Information bit payload |  |  | (Note4) | QPSK / 4392 (TBD) |  |
| Max number of HARQ transmissions |  |  | 1 | N/A |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b | N/A |  |
| Physical channel for RI reporting |  |  | PUSCH | N/A |  |
| PUCCH Report Type for RI/ first PMI |  |  | TBD | N/A |  |
| Reporting periodicity |  | ms | Npd = TBD | N/A |  |
| CQI Delay |  | ms | TBD | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 | N/A |  |
| ri-ConfigIndex |  |  | 805 (Note 6) | N/A |  |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |  |
| PDSCH scheduled sub-frames |  |  | Aligned with csi-MeasSubframeSet-r12 |  |  |
| Timing offset between TPs |  | us | 0 |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  |
| Note1: Reference measurement channel RC.10 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: REs for antenna ports 0 and 1 CRS have zero transmission power. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: N/A. Note 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 6: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |

# Conclusion

In this contribution, we provide details on how to setup the CSI test for eIMTA. The parameters are listed in Table 1~Table 2. We hope the group can consider these setups in the eIMTA CSI discussion.

# Reference

- R4-146641, WF for demodulation and CSI test coverage of TDD eIMTA, CATT, Qualcomm, Huawei, Intel, Ericsson, Oct 2014

- R4-147897, Way forward for eIMTA CSI tests, Ericsson, CATT, Huawei, Intel, Qualcomm, Nov. 2014

- R4-151057, Way forward for eIMTA CSI simulation assumption, CATT, Qualcomm, Intel, Huawei, Ericsson, Feb. 2015.