3GPP TSG RAN WG4 Meeting #74bis                                      	      R4-152218

Rio de Janeiro, Brazil

April 20 - 24, 2015

Agenda Item:	5.1.2

Source:	Motorola Mobility

Title:	A-MPR Reduction for Contiguous CA with Almost-Contiguous Resource Allocations

Document for:   Approval

# Introduction

For intra-band contiguous carrier aggregation, the A-MPR allowed for non-contiguous resource allocations can be very large when CA_NS signalling is used.  The situation is much better for contiguous resource allocations, since the A-MPR allowed for contiguous allocations is generally much less.  For example, when CA_NS_01 is signalled, the A-MPR allowed for an allocation ratio of 0.5 is 9.5 dB, regardless of the placement of the allocation within the aggregated carriers.  Conversely, for a contiguous allocation with an allocation ratio of 0.5, the allowed A-MPR can be as small as 5 dB for the aggregation of two 15 MHz carriers.

While contiguous resource allocations are preferred from an A-MPR perspective, there is an issue in that contiguous PUSCH allocations that span the boundary of the aggregated carriers will overlap the PUCCH resource blocks that are semi-statically configured at the edge of the component carriers, and will thus degrade the performance of the PUCCH.  It can also be observed that if the aggregated component carriers are equal in size, any contiguous allocation ratio having an allocation ratio of 0.5 or greater will either overlap both PUCCH regions on one component carrier, or will overlap at least one PUCCH region on both carriers.  As a result, though contiguous resource allocations are preferred from an A-MPR perspective, it will generally not be possible to assign contiguous allocations which span the boundary between carriers or which have an allocation ratio greater than or equal to 0.5 dB while avoiding the PUCCH region.

This issue has been discussed previously in [1-3].  In [1], it was proposed that the A-MPR for contiguous resource allocations could be modified to apply to resource allocations that are contiguous except for puncturing by the PUCCH region. In [2], this proposal was considered for the case of general emissions requirements without CA_NS signalling.  In [3], the A-MPR reduction for the proposal in [1] was considered in more detail for CA_NS_01, CA_NS_02, and CA_NS_03, and a draft CR for CA_NS_01 was provided in the Appendix.

In this contribution, the proposal, in [1] is considered further.  In particular, the reduction of the allowed A-MPR is evaluated for resource allocations which are contiguous except for puncturing by the PUCCH region for CA_NS_01 through CA_NS_06.  Additionally, the A-MPR margin is evaluated for these same allocations for for CA_NS_01, CA_NS_02, CA_NS_03, and CA_NS_05 for aggregated channel bandwidths of 30 MHz and 39.8 MHz.   Finally, a draft CR is provided in the Appendix modifying the A-MPR allowed for almost contiguous resource allocations for CA_NS_01.

# A-MPR Difference between Contiguous and Non-Contiguous Allocations

As discussed in [1], there are two methods used in TS 36.101 for specifying the maximum allowed A-MPR for intra-band carrier aggregation when CA_NS signalling is used.  For each CA_NS_0x, a table is defined for contiguous resource allocations and a formula is defined for non-contiguous resource allocations.  In general, contiguous resource allocations are preferred because the allowed A-MPR is typically significantly less for a contiguous allocation than for a non-contiguous allocation with the same number of resource blocks.

For a non-contiguous resource allocation, the allowed A-MPR is a function of the allocation ratio only.  The A-MPR allowed for an allocation ratio of 0.5 for CA_NS_01 through CA_NS_06 is shown in Table 1.  For contiguous resource allocations, the allowed A-MPR is a function of both the size of the allocation and the placement of the allocation.  In Table 1, the minimum A-MPR is shown for a contiguous allocation with an allocation ratio of 0.5, where the minimum is take over all possible placements of the allocation.   Since the allowed A-MPR depends on the size of the component carriers, the minimum A-MPR is provided for the aggregation of two 15 MHz carriers as well as for the aggregation of two 20 MHz carriers.

- Table 1: A-MPR reduction from using contiguous A-MPR tables rather than non-contiguous A-MPR formula for contiguous allocations

| CA_NS signalled | A-MPR for A = 0.5 with non-contiguous formula | Minimum A-MPR for contiguous allocation with A = 0.5 (75+75 RBs) | Minimum A-MPR for contiguous allocation with A = 0.5 (100+100 RBs) | A-MPR reduction for contiguous allocation (75+75 RBs) | A-MPR reduction for contiguous allocation (100+100 RBs) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 | 9.5 | 5 | 6 | 4.5 | 3.5 |
| CA_NS_02 | 9.5 | 2 | 3 | 7.5 | 6.5 |
| CA_NS_03 | 11.5 | 2 | 4 | 9.5 | 7.5 |
| CA_NS_04 | 4.5 | 4 | 3 | 0.5 | 1.5 |
| CA_NS_05 | 9.5 | 5 | 6 | 4.5 | 3.5 |
| CA_NS_06 | 11.5 | 2 | 0 | 9.5 | 11.5 |

The difference in A-MPR for contiguous and non-contiguous allocations for the contiguous aggregation of two component carriers with CA_NS_0x signalling is illustrated in Figure 1.  This example shows the maximum difference in the A-MPR between the non-contiguous and contiguous formulas for an allocation ratio of 0.5. If

four RB’s are semi-statically configured for the PUCCH at the edge of each of two contiguously aggregated 15 MHz component carriers, then all resource block allocations with more than 67 RB’s must be non-contiguous due to blocking by the PUCCH region. However, because 67 RB’s is not an allowed allocation, the largest allowed contiguous allocation is 64 RB’s.  Similarly, if four RB’s are semi-statically configured for the PUCCH at the edge of each of two contiguously aggregated 20 MHz component carriers, then all resource block allocations with more than 92 RB’s must be non-contiguous due to blocking by the PUCCH region.  However, since 92 RB’s is not an allowed allocation, the largest allowed contiguous allocation is 90 RB’s.

# Reducing A-MPR for Almost-Contiguous Resource Allocations

In [1], it was proposed that the A-MPR for almost contiguous resource allocations could be specified as the minimum of the existing A-MPR for non-contiguous resource allocations and a second A-MPR defined for almost-contiguous resource allocations which is the sum of the A-MPR for the smallest contiguous RB allocation containing the non-contiguous resource allocation and an adjustment factor which depends on the fraction of RB’s which are punctured from the contiguous allocation.  In particular, the specification [3] already defines RBstart and RBend for any resource block allocation as

RBstart 	Indicates the lowest RB index of transmitted resource blocks

RBend	Indicates the highest RB index of transmitted resource blocks

Furthermore, for a contiguous resource allocation, the specification [3] defines

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

For any resource allocation, the total number of transmitted resource blocks is denoted in [3] as

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

It can be noted that the A-MPR for any contiguous resource allocation can be determined from tables 6.2.4A.1-1 through 6.2.4A.8-1 for CA_NS_01 through CA_NS_08, respectively, using only  RBstart, RBend, and LCRB.  If we denote the A-MPR for a contiguous allocation as

MC(RBstart, RBend, LCRB)

then the A-MPR for an almost contiguous resource allocation punctured only by the PUCCH can denoted as

MAC(RBstart, RBend, NRB_alloc),

and can be computed as

MAC(RBstart, RBend, NRB_alloc) = min{ MC(RBstart, RBend, RBend – RBstart + 1)

+  10*log10((RBend – RBstart + 1) /NRB_alloc),

MA  }

where MA denotes the existing A-MPR for the non-contiguous resource allocation with the given allocation ratio equal to A = NRB_alloc / NRB_agg.

A draft CR showing how this change can be incorporated into the specification is provided in the Appendix for CA_NS_01, and the same approach follows directly for CA_NS_02, CA_NS_03, and CA_NS_05.

# Examples of A-MPR reduction with CA_NS_0X signalling

In this section, several examples are provided indicating the A-MPR reduction that would result with the proposed change.  In particular, we consider the contiguous aggregation of two 15 MHz component carriers.  It is assumed that four PUCCH resources are semi-statically allocated at the edge of the two carriers.  Figures 1-6 indicate the A-MPR reduction that would result with the proposed change with the signalling of CA_NS_01 through CA_NS_06.  In each Figure, the vertical axis gives the number of allocated RB’s.  If the allocation does not cross the boundary between the two component carriers, then the allocation is contiguous and the number of allocated RB’s is given by

NRB_alloc = RBend – RBstart + 1 .

Alternatively, if the allocation crosses the boundary between the two carriers, the allocation is not contiguous, but is instead almost-contiguous, and the number of allocated RB’s is given by

NRB_alloc = (RBend – RBstart + 1) – 8,

since there are eight RB’s allocated for PUCCH at the edge of the two carriers.

In Figures 1-6, only the almost-contiguous allocations are shown, since only these allocations benefit from the proposed change.  Thus, for all of the allocations shown in the table, it is the case that

NRB_alloc = (RBend – RBstart + 1) – 8.

Also, for all of the resource allocations shown, the number of RB’s on each component carrier falls within the set of the allowed PUSCH allocation sizes.

Figure 1:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_01 with the proposed change (aggregation of two 15 MHz carriers with 4 PUCCH resources on each).

Figure 2:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_02 with the proposed change (aggregation of two 15 MHz carriers with 4 PUCCH resources on each).

Figure 3:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_03 with the proposed change (aggregation of two 15 MHz carriers with 4 PUCCH resources on each).

Figure 4:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_04 with the proposed change for 16QAM modulation (aggregation of two 15 MHz carriers with 4 PUCCH     resources on each).

Figure 5:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_05 with the proposed change (aggregation of two 15 MHz carriers with 4 PUCCH resources on each).

Figure 6:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_06 with the proposed change (aggregation of two 15 MHz carriers with 4 PUCCH resources on each).

As noted above, any allocation larger than 64 RB’s cannot be contiguous due to blocking by the PUCCH region, and thus without the proposed change, the non-contiguous A-MPR tables must be used for these allocations.  It can be observed that for an almost-contiguous allocation of 65 RB’s, the reduction in A-MPR can exceed 4.5 dB for CA_NS_01, CA_NS_02, CA_NS_03, CA_NS_05, CA_NS_06. Conversely, the A-MPR reduction for CA_NS_04 and 16QAM modulation can be seen to be quite small, except in the case of small allocations which can be sent contiguously on one of the two aggregated component carriers.

# Simulation Results for CA_NS_01, CA_NS_02, CA_NS_03, and CA_NS_05

In this section, simulation results are presented for CA_NS_01, CA_NS_02, CA_NS_03, and CA_NS_05 for aggregated bandwidths of 30 MHz (75 + 75 RB’s) and 39.8 MHz (100 + 100 RB’s). It can be noted that for these CA_NS_0x cases, these are the only two aggregated bandwidths for which A-MPR is defined for contiguous resource allocations.  Simulation results are not presented for CA_NS_04 because the A-MPR reduction associated with the proposed change is quite small except in the case of small allocations which can be sent contiguously on one of the two aggregated component carriers.

## CA_NS_01

The additional emissions requirements that apply with the signalling of CA_NS_01 are given in Table 2 below.

Table 2: Additional requirements for CA_NS_01 (TS 36.101 Table 6.6.3.3A.1-1)

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 1 |
| NOTE 1: Applicable when the aggregated channel bandwidth is confined within frequency range 1940 – 1980 MHz |  |  |  |  |  |  |

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth (300 kHz).

Simulations were performed for aggregated bandwidths of 30 MHz (75+75 RB’s) and 39.8 MHz (100+100 RB’s) with nominal carrier spacing.  The modulator impairments used in the simulations were the nominal impairments which apply above 1 GHz:

- Modulator IQ – image = -25 dB

- Modulator carrier leakage = -25 dBc

- Modulator C_IM3 = -60 dBc

The operating point of the PA was set such that the UTRAACLR1 requirement of 33 dB is just met with a 100 RB QPSK allocation.  The output power at which the requirement is just met is defined to be 22 dBm.

In the simulations, it was assumed that the outermost 4 resource blocks of each of the two component carriers were allocated for the PUCCH.  16QAM modulation was used for all simulations.  All possible “almost contiguous” resource allocations were simulated, where almost contiguous resource allocations are resource allocations which cross the boundary between the carriers and which are contiguous with the PUCCH region.

The simulation results in Figures 7 and 8 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_01 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively.  In general, the A-MPR allowed with the proposed change is sufficient to meet the CA_NS_01 requirements as indicated by the positive margins.  However, there are a few cases for which the A-MPR margin is negative, and in these cases the A-MPR allowed with the proposed change is not sufficient.  The maximum negative margins seen in Figures 7 and 8 are -0.8 dB and -0.6 dB, respectively.

As a result of these negative margins, all possible contiguous allocations were simulated for CA_NS_01 for both 30 and 39.8 MHz bandwidth (with no RB’s allocated for PUCCH).  From these simulations, it was found that the with the PA biased to just meet the UTRAACLR1 requirement of 33 dB for a 100 RB QPSK allocation, the CA_NS_01 emissions requirements could not be met with the A-MPR defined for contiguous allocations in Table 6.2.4A.1-1.  In order to meet the CA_NS_01 requirements with the allowed A-MPR, it was necessary to back of the PA bias point by 1.5 dB.

The simulation results in Figures 9 and 10 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_01 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively when the PA bias point is backed off by 1.5 dB.  Since the margin is always positive with this bias point, the A-MPR allowed with the proposed change is seen to be sufficient to meet the CA_NS_01 emissions requirements.

Figure 7:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_01 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each.

Figure 8:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_01 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each.

Figure 9:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_01 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

Figure 10:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_01 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

## CA_NS_02

The additional emissions requirements that apply with the signalling of CA_NS_02 are given in Table 3 below.

Table 3:  Additional requirements (TS 36.101 Table 6.6.3.3A.2-1)

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1900 | - | 1915 | -15.5 | 5 | 1, 2 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1, 2 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The simulation results in Figures 11 and 12 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_02 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively.  In general, the A-MPR allowed with the proposed change is sufficient to meet the CA_NS_02 requirements as indicated by the positive margins.  However, there are a few cases for which the A-MPR margin is negative, and in these cases the A-MPR allowed with the proposed change is not sufficient.  The maximum negative margins seen in Figures 11 and 12 are -0.8 dB and -0.9 dB, respectively.

The simulation results in Figures 13 and 14 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_02 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively when the PA bias point is backed off by 1.5 dB.  Since the margin is always positive with this bias point, the A-MPR allowed with the proposed change is seen to be sufficient to meet the CA_NS_02 emissions requirements.

Figure 11:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_02 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each.

Figure 12:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_02 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each.

Figure 13:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_02 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

Figure 14:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_02 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

## CA_NS_03

The additional emissions requirements that apply with the signalling of CA_NS_03 are given in Table 4 below.

Table 4:  Additional requirements (TS 36.101)

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1880 | - | 1895 | -40 | 1 |  |
| Frequency range | 1895 | - | 1915 | -15.5 | 5 | 1, 2 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1, 2 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The simulation results in Figures 15 and 16 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_03 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively.  In general, the A-MPR allowed with the proposed change is sufficient to meet the CA_NS_03 requirements as indicated by the positive margins.  However, there are a few cases for which the A-MPR margin is negative, and in these cases the A-MPR allowed with the proposed change is not sufficient.  The maximum negative margins seen in Figures 15 and 16 are -0.7 dB and -0.9 dB, respectively.

The simulation results in Figures 17 and 18 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_03 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively when the PA bias point is backed off by 1.5 dB.  Since the margin is always positive with this bias point, the A-MPR allowed with the proposed change is seen to be sufficient to meet the CA_NS_03 emissions requirements.

Figure 15:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_03 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each.

Figure 16:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_03 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each.

Figure 17:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_03 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

Figure 18:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_03 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

## CA_NS_05

The additional emissions requirements that apply with the signalling of CA_NS_05 are given in Table 5 below.

Table 5:  Additional requirements (TS 36.101 Table 6.6.3.3A.4-1)

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2620 | - | 2645 | -15.5 | 5 | 1, 2, 3 |
| Frequency range | 2645 | - | 2690 | -40 | 1 | 1, 3 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: This requirement is applicable for carriers with aggregated channel bandwidths confined in 2570-2615 MHz. |  |  |  |  |  |  |

The simulation results in Figures 19 and 20 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_05 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively.  In general, the A-MPR allowed with the proposed change is sufficient to meet the CA_NS_05 requirements as indicated by the positive margins.  However, there are a few cases for which the A-MPR margin is negative, and in these cases the A-MPR allowed with the proposed change is not sufficient.  The maximum negative margins seen in Figures 19 and 20 are -0.8 dB and -0.8 dB, respectively.

The simulation results in Figures 21 and 22 indicate the A-MPR margin with the proposed change relative to the A-MPR needed to meet the CA_NS_05 requirements for aggregated bandwidths of 30 and 39.8 MHz respectively when the PA bias point is backed off by 1.5 dB.  Since the margin is always positive with this bias point, the A-MPR allowed with the proposed change is seen to be sufficient to meet the CA_NS_05 emissions requirements.

Figure 19:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_05 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each.

Figure 20:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_05 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each.

Figure 21:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_05 emissions requirements for the aggregation of two 15 MHz carriers (75 + 75 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

Figure 22:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_05 emissions requirements for the aggregation of two 20 MHz carriers (100 + 100 RB’s) with 4 PUCCH resources on each (PA bias backed off by 1.5 dB).

# Summary

In this contribution as well as references [1-3], it has been noted that the A-MPR allowed for non-contiguous resource allocation is often much larger than that allowed for a contiguous resource allocation having the same allocation ratio.  As a result, contiguous resource allocations are generally preferred since less A-MPR is allowed for these allocations.  However, if the aggregated carriers have equal bandwidth, there is a problem in that it is not possible to have a contiguous resource allocation with an allocation ratio of 0.5 or greater in a subframe in which any user transmits on the PUCCH.

In order to allow reduced A-MPR for resource allocations having an allocation ratio of 0.5 or greater, it has been proposed to define an almost-contiguous resource allocation as an allocation which is contiguous except for puncturing by the PUCCH region at the edge of the component carriers and to define the A-MPR for such almost-contiguous allocations as described in Section 3.  The significant reduction in A-MPR that can be achieved with this proposal is shown in Section 4, and this reduction is on the order of 4.5 dB for allocation ratios of 0.5.  The simulation results in Section 5 show the A-MPR margin with the proposed change.  While the margin can be slightly negative in a few cases when the PA operating point is set using a 20 MHz carrier with full allocation, it seems that the A-MPR will be adequate when the  PA operating point is set so as to meet the CA_NS_0X emissions requirement with the A-MPR allowed for contiguous allocations.

A draft CR indicating the specification changes needed to incorporate the proposed change into the TS 36.101 specification for CA_NS_01 is provided in the Appendix.  It is proposed that these same changes also be adopted for CA_NS_02, CA_NS_03, and CA_NS_05.

Proposal:  Modify the A-MPR allowed for almost contiguous resource allocations as indicated draft CR in the Appendix for CA_NS_01, CA_NS_02, CA_NS_03, and CA_NS_05.

# References

- R4-146503, A-MPR for Contiguous CA with Almost-Contiguous Resource Allocations and CA_NS Signaling, Motorola Mobility, RAN4 #72bis.

- R4-147328, Almost-contiguous resource allocation, Nokia, RAN4 #73.

- R4-150925, A-MPR for CA with Almost-Contiguous Resource Allocations and CA_NS signaling, Motorola Mobility, RAN4 #74.

- 3GPP TS 36.101

Appendix:  Draft CR for TS 36.101 Sections 3.1 and 6.2.4A.1

## << Start of changes >>

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply in the case of a single component carrier. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Aggregated Transmission Bandwidth Configuration: The number of resource block allocated within the aggregated channel bandwidth.

Almost-contiguous resource allocation: A non-contiguous resource allocation spanning the boundary between two aggregated carriers which is equivalent to a first contiguous resource allocation punctured by a second contiguous resource allocation where the second contiguous resource allocation spans the boundary between the aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

## << Unchanged sections omitted >>

#### 6.2.4A.1	A-MPR for CA_NS_01 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCC and the SCC for contiguously aggregated signals is specified in table 6.2.4A.1-1.

Table 6.2.4A.1-1: Contiguous allocation A-MPR for CA_NS_01

| CA_1C: CA_NS_01 | RBstart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK and 16-QAM [dB] |
| --- | --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 23 and 176 – 199 | > 0 | N/A | ≤ 12.0 |
|  | 24 – 105 | > 64 | N/A | ≤ 6.0 |
|  | 106 – 175 | N/A | > 175 | ≤ 5.0 |
| 75 RB / 75 RB | 0 – 6 and 143 – 149 | 0 < LCRB ≤ 10 | N/A | ≤ 11.0 |
|  |  | > 10 | N/A | ≤ 6.0 |
|  | 7 – 90 | > 44 | N/A | ≤ 5.0 |
|  | 91 – 142 | N/A | > 142 | ≤ 2.0 |
| NOTE 1: RB_start indicates the lowest RB index of transmitted resource blocks NOTE 2: L_CRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL { min(MA, MAC), 0.5}

Where MA is defined as follows

MA =	-22.5 A + 17		; 0 ≤ A < 0.20

-11.0 A + 14.7			; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1

Where MAC is defined as follows

MAC = 	MC + 10 * log10((RBend – RBstart + 1) / NRB_alloc)

;almost-contiguous resource allocation

MA		; otherwise

Where A = NRB_alloc / NRB_agg., and MC is the contiguous resource allocation A-MPR from Table 6.2.4A.1-1 that applies for a contiguous allocation with

LCRB = RBend – RBstart + 1.

## << End of changes >>