3GPP TSG-RAN WG4 RAN4 #75                                         R4-153293

Fukuoka, Japan, 25 – 29 May, 2015

Agenda item:			6.6.1

Source:			Samsung

Title:				NAICS simulation alignment results

Document for:			Discussion

1	Introduction5

In 3GPP RAN4 #74bis meeting, RAN4 has reached an agreement for NAICS UE demodulation test cases [1][2]. In this contribution, we provide our 2nd round alignment results for the performance gain verification cases. Particularly, we provided CRS based TM related results in this paper.

- Simulation Assumption

We focus on the performance gain verification test cases with fixed interference type for performance alignment in this contribution. The simulated test case is aligned with the suggestion in [1] as shown in Table 1.

Table 2 listed the assumed HL signaling on blind detection.

Table 3 listed the general simulation assumptions.

Table 1: Simulated Case

| Test | TMs | Inter. ON/OFF | MCS | Rank | Antenna Config. | Interf. Type | CRS Colliding |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TM2/2/2 | ON/ON | MCS 8/rand/rand MCS 9/rand/rand | 1/1/1 | 2x2 | Random | Colliding |
| 3 | TM4/4/4 | ON/ON | MCS 8/rand/rand MCS 9/rand/rand | 1/rand/rand | 2x2 | Random | Colliding |

Table 2: Simulation Assumptions on Blind Detection

| Parameter | Value |
| --- | --- |
| PA subset | {-6dB, -3dB, 0dB} |
| TM subset | {TM2, TM3, TM4, TM6, TM9} |
| Blind detection granularity | 1 PRB pair |
| PCFICH/PDCCH detection | Genie-aided |
| Serving cell PMI | Random wideband PMI for serving cell per TTI |
| Interference cell PMI | Random wideband PMI for interference cell per TTI |

Table 3: General Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 50RB |
| Cell ID | [0, 6, 1] |
| Receiver | R-ML |
| Interference profiles | INF Level: INR1 = 13.91dB, INR2 = 3.34dB |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | Not configured |
| Channel Estimation | Realistic Channel Estimation |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 3 |
| Tx EVM | 6% |
| Scheduled SF | 1, 2, 3, 4, 6, 7, 8, 9 |
| Receiver Type | MMSE-IRC without CRS-IC R-ML with CRS-IC |

- Simulation Results

Figure 1~3 shows the simulation results of case 1 and 3 correspondingly.

Figure 1: Simulation Results of Case 1

Figure 2: Simulation Results of Case 3

Table 4 and Table 5 summarized the required SINR @70% and 85% throughput.

Table 4: Required SINR @70% throughput

|  | Serving Cell MCS | MMSE-IRC | R-ML | Performance Gain |
| --- | --- | --- | --- | --- |
| Test Case 1 | MCS 8 | -4.6dB | -6.9dB | 2.3dB |
|  | MCS 9 | -3.4dB | -5.4dB | 2.0dB |
| Test Case 3 | MCS 8 | -3.7dB | -5.5dB | 1.8dB |
|  | MCS 9 | -2.3dB | -3.9dB | 1.6dB |

Table 5: Required SINR @85% throughput

|  | Serving Cell MCS | MMSE-IRC | R-ML | Performance Gain |
| --- | --- | --- | --- | --- |
| Test Case 1 | MCS 8 | -2.6dB | -4.5dB | 1.9dB |
|  | MCS 9 | -1.3dB | -2.9dB | 1.6dB |
| Test Case 3 | MCS 8 | -1.0dB | -2.0dB | 1.0dB |
|  | MCS 9 | 0.5dB | -0.5dB | 1.0dB |

- Conclusion

In this contribution, we provide our 2nd round alignment results for the performance gain verification cases. Particularly, we provided CRS based TM related results in this paper.

- Reference

[1] R4-152355, “Draft Updates to TS36.101 for Rel-12 NAICS”, MediaTek, RAN4#74bis, Rio de Janeiro, 20th – 24th April, 2015.

[2]	R4-152356, “NAICS Discussions: Meeting Minutes”, MediaTek, RAN4#74bis, Rio de Janeiro, 20th – 24th April, 2015.