3GPP TSG-RAN WG4 #75	R4-153351

Fukuoka, Japan, May 25~May 29, 2015

Source: 	Ericsson

Title:	Simulation assumption for TM9 in CRS-IM WI

Agenda Item:	7.5.1

Document for:	Approval

# Introduction

In [1], it was agreed to have TM9 test for CRS-IM and most of key parameters were agreed for PDSCH test. In this contribution, simulation assumptions for TM9 are proposed in CRS-IM WI. Interested companies are encouraged to provide alignment results in the next meeting based on the proposed simulation parameters.

# Simulation assumption for TM9

## TM9 (FDD)

Table 1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |  |
| System bandwidth |  |  |  | 10 MHz |  |  |
| Duplex mode |  |  |  | FDD |  |  |
| Transmission mode in serving cell |  |  |  | TM9-1layer |  |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for interference cell |  |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |  |
|  |  |  |  | Use different channel seed for between cells |  |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | INR1= 10.45 dB | INR2 = 4.6 dB |  |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |  |
|  |  |  |  | Time-delay wrt. serving cell |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | [3 us] | [-1 us] |  |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | 300 Hz | -100 Hz |  |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modelled cells |  |  |
|  | B |  |  | -3 dB in all modelled cells (PB=1) |  |  |
|  |  |  |  | 0 dB in all modelled cells |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 for serving cell |  | N/A for aggressor cells |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | 5/2 for serving cell |  | N/A for aggressor cells |
| CSI reference signal configuration |  |  |  | 8 for serving cell |  | N/A for aggressor cells |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | [3 / 0010000000000000] for serving cell |  | N/A for aggressor cells |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |  |
| Feedback mode |  |  |  | None |  |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |  |
|  |  | Precoding granularity |  | Frequency domain: 1 PRG Time domain: 1 ms |  |  |
|  |  | Beamforming model |  | Annex B.4.1 |  |  |
|  |  | Modulation |  | MCS=14, the information bits are 12960 bits for the subframes other than subframe#5, the reference channel is defined in Table 2. |  |  |
|  |  | Code rate |  |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |  |
|  |  | CRC |  |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |
|  |  | PMI |  | Random precoding |  |  |
|  |  | Modulation |  | 64QAM |  |  |
|  |  | Code rate |  | - |  |  |
|  |  | Channel coding, rate matching |  | - |  |  |
|  |  | CRC |  |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |
|  |  | Average resource utilization |  | {20%} |  |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |  |
| Cyclic prefix |  |  |  | Normal |  |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |  |
|  |  |  | CRS-IM | Without CRS-IM |  |  |
|  |  |  |  | With 2-cell CRS-IM |  |  |

Table 2: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.51 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 12960 |
| For Sub-Frames 2,3,7,8 | Bits | 12960 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 10680 |
| Number of Code Blocks (Note 4) |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 3 |
| For Sub-Frames 2,3,7,8 | Code blocks | 3 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 2 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 24000 |
| For Sub-Frames 2,7 |  | 23600 |
| For Sub-Frames 3,8 |  | 23200 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 11.4360 |
| UE Category |  | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |

## TM9 (TDD)

Table 1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |  |
| Uplink downlink configuration |  |  |  | 1 |  |  |
| Special subframe configuration |  |  |  | 4 |  |  |
| Cyclic prefix |  |  |  | Normal |  |  |
| System bandwidth |  |  |  | 10 MHz |  |  |
| Duplex mode |  |  |  | TDD |  |  |
| Transmission mode in serving cell |  |  |  | TM9-1layer |  |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for interference cell |  |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |  |
|  |  |  |  | Use different channel seed for between cells |  |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | INR1= 10.45 dB | INR2 = 4.6 dB |  |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |  |
|  |  |  |  | Time-delay wrt. serving cell |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | [3 us] | [-1 us] |  |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |
|  |  |  |  | 300 Hz | -100 Hz |  |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modelled cells |  |  |
|  | B |  |  | -3 dB in all modelled cells (PB=1) |  |  |
|  |  |  |  | 0 dB in all modelled cells |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 for serving cell |  | N/A for aggressor cells |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | 5/4 for serving cell |  | N/A for aggressor cells |
| CSI reference signal configuration |  |  |  | 8 for serving cell |  | N/A for aggressor cells |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | [4 / 0010000000000000] for serving cell |  | N/A for aggressor cells |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |  |
| Feedback mode |  |  |  | None |  |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |  |
|  |  | Precoding granularity |  | Frequency domain: 1 PRG Time domain: 1 ms |  |  |
|  |  | Beamforming model |  | Annex B.4.1 |  |  |
|  |  | Modulation |  | MCS=14, the information bits are 12960 bits for the subframes other than subframe#5, The reference channel is defined in Table 2. |  |  |
|  |  | Code rate |  |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |  |
|  |  | CRC |  |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |
|  |  | PMI |  | Random precoding |  |  |
|  |  | Modulation |  | 64QAM |  |  |
|  |  | Code rate |  | - |  |  |
|  |  | Channel coding, rate matching |  | - |  |  |
|  |  | CRC |  |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |
|  |  | Average resource utilization |  | {20%} |  |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |  |
| Cyclic prefix |  |  |  | Normal |  |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |  |
|  |  |  | CRS-IM | Without CRS-IM |  |  |
|  |  |  |  | With 2-cell CRS-IM |  |  |

Table A.3.4.3.3-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.xx TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 5) |
| Uplink-Downlink Configuration (Note 3) |  | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 12960 |
| For Sub-Frame 4,9 | Bits | 12960 |
| For Sub-Frames 1,6 | Bits | 9144 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 10680 |
| Number of Code Blocks (Note 4) |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Code blocks | 3 |
| For Sub-Frames 4,9 | Code blocks | 3 |
| For Sub-Frames 1,6 | Code blocks | 2 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 | Code blocks | 2 |
| Binary Channel Bits |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Bits | 24000 |
| For Sub-Frames 4,9 |  | 22800 |
| For Sub-Frames 1,6 |  | 15744 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 4.4208 |
| UE Category |  | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. |  |  |

# Conclusion

In this contribution, simulation parameters for TM9 are proposed. Interested companies can provide link level results for alignment in the next meeting.

# Reference

- R4-152399, WF on CRS-IM PDSCH demodulation non-TM10, Ericsson, Apr. 2015.