3GPP TSG-RAN WG4 Meeting #76	R4-154441

Beijing, China, 24 – 28 Aug, 2015

Source: 	Samsung

Title: 	Performance Evaluation of MMSE-IRC Algorithm for LTE Base Station Receiving

Agenda Item:	7.6.3

Document for:	Discussion

# Introduction

At last RAN4#75bis conference,  technical proposals on interference modelling methodology for BS MMSE-IRC[1], and the corresponding simulation results[2], link level performance evaluation [3]were agreed. Companies are encouraged to provide the link level simulation results at this conference. So in this contribution, we provide the performance comparison between MMSE and MMSE-IRC algorithms based on the agreed simulation assumptions.

# Simulation Assumptions

As given in  [3], the common parameters for link level evaluation are given in Table 7.1-1. The evaluation cases with specific parameters are given in Table 7.1-2. And the performance under both MMSE and MMSE-IRC receivers should be provided for comparison of the performance.

For link level simulation in Phase I, the simulation results of throughput versus SINR for both MMSE-IRC and MMSE receivers, and SINR gain of MMSE-IRC over MMSE at the 70% relative throughput will be provided. The ideal simulation results will be provided without the impairments.

Table 7.1-1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | 2 |
|  | Noc | dBm/15K | [-98] |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for synchronous case |  | Well aligned: no timing delay and frequency offset between the serving UE and interfering UEs |
| Frequency hopping, TTI bundling |  |  | Disable |

Table 7.1-2: Cases for link level evaluations

| Num | PRB allocation/ Band width | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- | --- |
| 1 | 50 PRB/10MHz | [6] | (EPA5, EPA5) | 1x2 Low | (-1.11, -10.91) |
| 2 | 50 PRB/10MHz | [6] | (EPA5, EPA5) | 1x2 Low | (-0.43, -13.78) |
| 3 | 50 PRB/10MHz | [15] | (EPA5, EPA5) | 1x4 Low | (-1.11, -10.91) |
| 4 | 50 PRB/10MHz | [15] | (EPA5, EPA5) | 1x4 Low | (-0.43, -13.78) |
| 5 | 50 PRB/10MHz | [20] | (EPA5, EPA5) | 1x8 Low | (-1.11, -10.91) |
| 6 | 50 PRB/10MHz | [20] | (EPA5, EPA5) | 1x8 Low | (-0.43, -13.78) |
| 7 | 50 PRB/10MHz | [6] | (EVA70, EVA70) | 1x2 Low | (-1.11, -10.91) |
| 8 | 50 PRB/10MHz | [6] | (EVA70, EVA70) | 1x2 Low | (-0.43, -13.78) |
| 9 | 50 PRB/10MHz | [15] | (EVA70, EVA70) | 1x4 Low | (-1.11, -10.91) |
| 10 | 50 PRB/10MHz | [15] | (EVA70, EVA70) | 1x4 Low | (-0.43, -13.78) |
| 11 | 50 PRB/10MHz | [20] | (EVA70, EVA70) | 1x8 Low | (-1.11, -10.91) |
| 12 | 50 PRB/10MHz | [20] | (EVA70, EVA70) | 1x8 Low | (-0.43, -13.78) |

# Simulation Results

The simulation results of throughput versus SINR for both MMSE-IRC and MMSE receivers, and SINR gain of MMSE-IRC over MMSE at the 70% relative throughput are provided in Table 1.

The detailed results are provided in the attached Excel file.

Table 1 Comparison of MMSE-IRC and MMSE by Link Level Evaluation

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance | MMSE performance | SINR gain at 70% |
| 1 | 50 PRB/10MHz | [6] | (EPA5, EPA5) | 1x2 Low | (-1.11, -10.91) | -6.01 | -2.88 | 3.13 |
| 2 | 50 PRB/10MHz | [6] | (EPA5, EPA5) | 1x2 Low | (-0.43, -13.78) | -9.05 | -3.46 | 5.59 |
| 3 | 50 PRB/10MHz | [15] | (EPA5, EPA5) | 1x4 Low | (-1.11, -10.91) | -4.28 | 0.48 | 4.76 |
| 4 | 50 PRB/10MHz | [15] | (EPA5, EPA5) | 1x4 Low | (-0.43, -13.78) | -7.76 | 0.20 | 7.96 |
| 5 | 50 PRB/10MHz | [20] | (EPA5, EPA5) | 1x8 Low | (-1.11, -10.91) | -3.27 | 1.18 | 4.45 |
| 6 | 50 PRB/10MHz | [20] | (EPA5, EPA5) | 1x8 Low | (-0.43, -13.78) | -7.33 | 1.02 | 8.35 |
| 7 | 50 PRB/10MHz | [6] | (EVA70, EVA70) | 1x2 Low | (-1.11, -10.91) | -4.91 | -2.19 | 2.72 |
| 8 | 50 PRB/10MHz | [6] | (EVA70, EVA70) | 1x2 Low | (-0.43, -13.78) | -6.98 | -2.12 | 4.86 |
| 9 | 50 PRB/10MHz | [15] | (EVA70, EVA70) | 1x4 Low | (-1.11, -10.91) | -2.39 | 1.78 | 4.18 |
| 10 | 50 PRB/10MHz | [15] | (EVA70, EVA70) | 1x4 Low | (-0.43, -13.78) | -5.04 | 1.82 | 6.86 |
| 11 | 50 PRB/10MHz | [20] | (EVA70, EVA70) | 1x8 Low | (-1.11, -10.91) | -1.80 | 2.85 | 4.64 |
| 12 | 50 PRB/10MHz | [20] | (EVA70, EVA70) | 1x8 Low | (-0.43, -13.78) | -4.40 | 2.90 | 7.31 |

# Conclusion

We present link level simulation results as per case defined in [3].

# Reference

[1]	R4-153862, TP on interference modelling methodology for BS MMSE-IRC. China Telecom, 3GPP TSG-RAN WG4 meeting #75, Fukuoka, Japan, 25 - 29 May, 2015.

[2]	R4-153863, TP on simulation results for BS MMSE-IRC interference modelling. China Telecom, 3GPP TSG-RAN WG4 meeting #75, Fukuoka, Japan, 25 - 29 May, 2015.

[3]	R4-153669, TP on link level performance evaluation. China Telecom, 3GPP TSG-RAN WG4 meeting #75, Fukuoka, Japan, 25 - 29 May, 2015.