3GPP TR 36.833-1-08 V0.2.0 (2015-08)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

LTE Advanced intra-band contiguous Carrier Aggregation (CA) in Band 8

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<LTE, Radio>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	5

4.1	Justification	5

4.2	Task Descriptions	6

5	Band and Channel Arrangements	6

5.1	CA operating bands	6

5.2	CA channel bandwidth	6

5.3	CA Bandwidth Class	6

6	E-UTRA RF requirements for UE	6

6.1	Transmitter Characteristics	6

6.1.1 UE Maximum Output Power for intra-band contiguous CA	6

6.1.2 Spectrum emission mask for intra-band contiguous CA	7

6.1.3	Adjacent Channel Leakage Ratio	7

6.1.3.1	Minimum requirement UTRA for CA	7

6.1.3.2  Minimum requirements for CA E-UTRA	8

6.1.4 Transmit intermodulation	8

6.2	Receiver Characteristics	9

6.2.1	Reference sensitivity	9

6.2.2	Blocking characteristics	9

6.2.2.1	In-band blocking	9

6.2.2.2	out-of-band blocking	10

6.2.3	Other Receiver characteristics	10

7	E-UTRA RF requirements for BS	11

8	E-UTRA demodulation performance for BS and UE	11

9	Summary of required changes to E-UTRA specifications	11

9.1	Required changes to TS 36.101	11

9.2	Required changes to TS 36.104	11

9.3	Required changes to TS 36.141	12

9.4	Required changes to TS 36.307	12

Annex A: Change history	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report of the work item for LTE Advanced Contiguous Carrier Aggregation in Band 8 which was initially approved at TSG RAN #67 [2]. The report provides background, analysis of the requirements, and a list of recommended changes to the specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-150194, “New Work Item Description: LTE Advanced intra-band contiguous Carrier Aggregation in Band 8 (2UL/2DL)”, CMCC

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

4	Background

4.1	Justification

Currently, the channel bandwidth for E-UTRA Band 8 can be configured as 1.4MHz, 3MHz, 5MHz and 10MHz, while 15MHz and 20MHz are not supported by the UE RF specification (TS36.101). However, due to gradual spectrum refarming and new spectrum auction, operators are having more than 10MHz contiguous spectrum at Band 8 to deploy E-UTRA system.

4.2	Task Descriptions

The objectives of this work item are the following:

-	Specify the specific RF requirements for band 8 intra-band CA scenario

-	Define the supported transmission bandwidths combination as follows

-	Add the performance requirements for this band combination in the relevant specifications.

-	As this CA combination can be applied for Japan, methods to conforming to Japanese regulations are also addressed in this WI.

5	Band and Channel Arrangements

5.1	CA operating bands

E-UTRA CA for Band 8 is defined to operate in the entire Band 8 range, as defined in Table 5.1-1.

Table 5.1-1 Intra band CA operating band definition for Band 8

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.2	CA channel bandwidth

Table 5.2-1 defines the supported E-UTRA bandwidths for intra-band contiguous CA in Band 8.  5 and10MHz component carrier (CC) channel bandwidths are proposed to be supported in Band 8 intra-band contiguous carrier aggregation.

Table 5.2-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_8B | 5, 10 | 10 | 20 | 0 |
|  | 10 | 5 |  |  |

5.3	CA Bandwidth Class

6	E-UTRA RF requirements for UE

6.1	Transmitter Characteristics

6.1.1 UE Maximum Output Power for intra-band contiguous CA

The following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth.

The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms).

For CA Bandwidth Class A, the requirements in 6.2.2 TS 36.101 apply. For CA Bandwidth Class B, the maximum output power is specified in Table 6.1.1-1.

Table 6.1.1-1: CA UE Power Class for intraband contiguous CA

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_8B |  |  |  |  | 23 | +2/-22 |  |  |
| NOTE 1: Void NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For intra-band contiguous carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

6.1.2 Spectrum emission mask for intra-band contiguous CA

For CA Bandwidth Class B, the power of any UE emission shall not exceed the levels specified in Table 6.1.2-1 for the specified channel bandwidth.

Table 6.1.2-1 General E-UTRA CA spectrum emission mask for Bandwidth Class B

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.45 | -25 | -13 | 1 MHz |
| 19.45-19.9 |  | -13 | 1 MHz |
| 19.9-24.9 |  | -25 | 1 MHz |

    - Adjacent Channel Leakage Ratio

      - Minimum requirement UTRA for CA

UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned carrier aggregated UTRA channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2). The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor =0.22. The assigned carrier aggregated UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.1.3.1-1. If the measured UTRA channel power is greater than –50dBm then the UTRAACLR shall be higher than the value specified in Table 6.1.3.1-1.

Table 6.1.3.1-1: Requirements for UTRAACLR1/2

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.1.3.2  Minimum requirements for CA E-UTRA

Carrier aggregated E-UTRA Adjacent Channel Leakage power Ratio (CA E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned aggregated E-UTRA channel frequency to the filtered mean power centred on an adjacent aggregated E-UTRA channel frequency at nominal channel spacing. The assigned aggregated E-UTRA channel power and adjacent aggregated E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.1.3.2-1. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table Table 6.1.3.2-1.

Table 6.1.3.2-1: General requirements for CA E-UTRAACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 30 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

6.1.4 Transmit intermodulation

User Equipment(s) transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or eNode B receive band as an unwanted interfering signal. The UE intermodulation attenuation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product on both component carriers when an interfering CW signal is added at a level below the wanted signal at each of the transmitter antenna port with the other antenna port(s) if any is terminated. Both the wanted signal power and the intermodulation product power are measured through rectangular filter with measurement bandwidth shown in Table 6.1.4-1.

The requirement of transmitting intermodulation is prescribed in Table 6.1.4-1.

Table 6.1.4-1: Transmit Intermodulation

| CA bandwidth class(UL) | B and C |  |
| --- | --- | --- |
| Interference Signal Frequency Offset | BWChannel_CA | 2*BWChannel_CA |
| Interference CW Signal Level | -40dBc |  |
| Intermodulation Product | -29dBc | -35dBc |
| Measurement bandwidth | BWChannel_CA- 2* BWGB |  |

6.2	Receiver Characteristics

6.2.1	Reference sensitivity

6.2.2	Blocking characteristics

6.2.2.1	In-band blocking

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 6.6.2.1-1 with the uplink configuration set according to Table 7.3.1A-1 in TS 36.101 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2 in TS36.101. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 6.6.2.1-1 and Tables 6.6.2.1-2 being on either side of the aggregated signal.

Table 6.6.2.1-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 |  |  |  |  |
| BWInterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 6.6.2.1-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_8B | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

6.2.2.2	out-of-band blocking

For intra-band contiguous carrier aggreagations the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 6.6.2.2-1 with the uplink configuration set according to Table 7.3.1A-1 in TS 36.101 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2 in TS 36.101. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 6.6.2.2-1 and Tables 6.6.2.2-2 being on either side of the aggregated signal.

Table 6.6.2.2-1: Out-of-band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 6.6.2.2-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_8B | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |

6.2.3	Other Receiver characteristics

CA_8B involves aggregating two downlink component carriers (CCs) with the maximum aggregation bandwidth not exceeding 20 MHz; a CA configuration of CA Bandwidth Class B.  The generic UE receiver requirements for CA Bandwidth Class B are addressed as apart of the work item for Release-12 intra-band contiguous CA in Band 27 (LTE_CA_C_B27) and are documented in the technical report for the work item, TR 36.833-1-27.

7	E-UTRA RF requirements for BS

8	E-UTRA demodulation performance for BS and UE

9	Summary of required changes to E-UTRA specifications

9.1	Required changes to TS 36.101

Required changes in TS 36.101 are shown in Table 9.1-1.  Unaffected sections for changes are omitted.

Table 9.1-1 Required changes in TS 36.101

| Section | Requirement | Expected Changes in TS 36.101 |
| --- | --- | --- |
| 5.5A | Operating bands for CA | A new row will be added in Table 5.5A-1 for CA_8B to the supported Intra-band CA operating bands |
| 5.6A.1 | Channel bandwidths per operating band for CA | A new row will be added in Table 5.6A.1-1 for CA_8B to the supported Intra-band CA operating bands |
| 7.6.1.1A | In-band blocking for intra-band contiguous CA | Requirement for CA_8B to be added to Table 7.6.1.1A-2. |
| 7.6.2.1A | Out-of-band blocking for intra-band contiguous CA | Requirement for CA_8B to be added to Table 7.6.2.1A-2. |
| 6.2.2A | CA UE Power Class for intraband contiguous CA | Add Band 8 to Table 6.2.2A-1 “CA UE Power Class for intraband contiguous CA” |
| 6.6.2.3.2A 6.6.2.3.3A | ACLR requirements for LTE_CA_C_B8 | Add ACLR requirements for CA Bandwidth Class B in table 6.6.2.3.2A -1and table 6.6.2.3.3A-1. |
| 6.7.1A | Transmit intermodulation requirements for LTE_CA_C_B8 | Add Transmit intermodulation requirements for CA Bandwidth Class B in Table 6.7.1A-1. |
| 6.6.2.1A | General E-UTRA CA spectrum emission mask for Bandwidth Class B | Add General E-UTRA CA spectrum emission mask for Bandwidth Class B in 6.6.2.1A. |

9.2	Required changes to TS 36.104

Required changes in TS 36.104 are shown in Table 9.2-1.  Unaffected sections for changes are omitted.

Table 9.2-1 Required changes in TS 36.104

| Section | Requirement | Expected Changes in TS 36.104 |
| --- | --- | --- |
| 5.5 | Operating bands for CA | Add Band 8 to Table 5.5-2 “Intra-band contiguous carrier aggregation bands” |

9.3	Required changes to TS 36.141

Required changes in TS 36.141 are shown in Table 9.3-1.  Unaffected sections for changes are omitted.

Table 9.3-1 Required changes in TS 36.141

| Section | Requirement | Expected Changes in TS 36.141 |
| --- | --- | --- |
| 5.5 | Operating bands for CA | Add Band 8 to Table 5.5-2 “Intra-band contiguous carrier aggregation bands” |

9.4	Required changes to TS 36.307

Required changes in TS 36.307 are shown in Table 9.4-1. Unaffected sections for changes are omitted.

Table 9.4-1 Required changes in TS 36.307

| Section | Requirement | Expected Changes in TS 36.307 |
| --- | --- | --- |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2015-04 | RAN4-74bis | R4- 151895 |  |  | Skeleton, initial version | N/A | 0.0.1 |
| 2015-05 | RAN4-75 | R4-153228 |  |  | R4-152454 TP on operating band for intra-band contiguous Carrier Aggregation in Band 8 R4-151897 TP on Receiver requirements for intra-band contiguous Carrier Aggregation in Band 8 | 0.0.1 | 0.1.0 |
| 2015-08 | RAN4-76 | R4-154499 |  |  | R4-153229 TP on UE maximum output power for intra-band contiguous Carrier Aggregation in Band 8 R4-153747 TP on CA spectrum emission mask for intra-band contiguous Carrier Aggregation in Band 8 R4-153231 TP on ACLR requirements for intra-band contiguous Carrier Aggregation in Band 8 R4-153232 TP on Transmit intermodulation requirements for intra-band contiguous Carrier Aggregation in Band 8 | 0.1.0 | 0.2.0 |