3GPP TSG-RAN WG4 #76	R4-154635

Beijing, China, 24 - 28 Aug 2015

Source: 	Ericsson

Title:	Preliminary link level simulation results for power imbalance test

Agenda Item:	6.2.2

Document for:	Discussion

# Introduction

In RAN4#75 meeting, the group had some extensive discussion on how to setup power imbalance test. In [1], the parameters for the imbalance test were proposed. In this contribution, we provide our preliminary link level simulation for the power imbalance test and share our view on the open issues.

# Simulation assumption

In Table 1, parameters for the power imbalance test are given according to [1].

Table 1: Parameters for the power imbalance test

| Test / Simulation Parameters |  | Test 1 (Discovery) | Test 2 (Communication) |
| --- | --- | --- | --- |
| # E-UTRA cells |  | 1 (serving cell) |  |
| # D2D UEs |  | 2 | 2 |
| D2D UE 1 | Transmission | PSDCH (0 HARQ) | PSCCH/PSSCH |
|  | PSSCH RMC | N/A | QPSK, TCR 2/3 |
|  | Timing offset w.r.t. serving cell | 0 us | 0 us |
|  | Freq offset w.r.t. serving cell | 0 Hz | 0 Hz |
|  | Rx power level | D2D UE2 + [19dB] | D2D UE2 + [19dB] |
|  | RB allocation (PSDCH/PSSCH) | 2RB[4,5] | 2RB[4,5] |
| D2D UE 2 | Transmission | PSDCH (0 HARQ) | PSCCH/PSSCH |
|  | PSSCH RMC | N/A | QPSK, TCR 2/3, 2RBs |
|  | Timing offset w.r.t. serving cell | 0 us | 0 us |
|  | Freq offset w.r.t. serving cell | 0 Hz | 0 Hz |
|  | Rx power level | - | - |
|  | RB allocation (PSDCH/PSSCH) | adjacent 2 RB of UE1[6,7] | adjacent 2 RB of UE1[6,7] |
| Leakage model from UE1 Link to UE 2 Link at Rx UE due to ICS for simulations only |  | Option 1 : -19dBc leakage into adjacent 2 RBs, operating SIR is 0dB for UE2 Option 2 : -22dBc leakage into adjacent 2 RBs, operating SIR is 3dB for UE2 Other options are not precluded (Leakage model over entire BW is not needed. This is for simulation purpose to derive the Link 2 operating SIR) |  |
| Propagation Channel | Serving cell | AWGN |  |
|  | D2D UE 1 |  |  |
|  | D2D UE 2 |  |  |
| Noc |  | -98dBm/15kHz |  |
| Channel BWs |  | 5MHz |  |
| Simulation results needed |  | PSDCH BLER for UE 2 link | PSSCH BLER for UE 2 link |

# Remark on the open parameters

## Leakage model

In 36.104 [2], BS in-channel selectivity test is introduced for power imbalance test. The leakage model and the level of the power imbalance for the BS in-channel selectivity test can be set as reference for D2D power imbalance test.

In Table 2, references sensitivity and the power level for in-channel selectivity for wide area BS are shown.

Table 2: Wide Area BS reference sensitivity levels and in-channel selectivity

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] | Wanted signal mean power | Interfering signal mean power [dBm] | Rx power difference b/w signal and interference | dBc |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | FRC A1-3 in Annex A.1 | -101.5 | -98.5 | -77 | 21.5 | -24.5 |
| 10 | FRC A1-3 in Annex A.1* | -101.5 | -98.5 | -77 | 21.5 | -24.5 |

From the value given in Table 2, we can see that the RX power difference b/w signal and interference is 21.5 dB (= (-77dBm) -(-98.5dBm)). The dBc can be derived as:

(Wanted signal mean power - Reference sensitivity power level) - Interfering signal mean power

The detail value of dBc is shown in the 7th column of Table 2.

From the above analysis, we prefer to modify the parameters for the RX power imbalance and the leakage model.

Proposal 1:  Rx power imbalance level between two links is 21.5 dB and -24.5dBc leakage into adjacent 2 RBs is assumed in the simulation.

## Test metric

In BS in-channel selectivity test, the required SNR is defined for the required SNR when the throughout is larger than 95% of the maximum throughput of the reference measurement channel [2]. As noted in D2D reference test, the throughput is relatively difficult to get the statistics since the transmissions are not continuous. Hence, we can use the BLER=95% as the test metric as used in D2D reference sensitivity test.

# Link level simulation results

In Figure 1, link level simulation results for discovery are given in Figure 1. Link level simulations for communication with bandwidth 5MHz and 10MHz are given in Figure 2 and Figure 3.

Figure 1: Link level simulation results for power imbalance test (Discovery)

Figure 2: Link level simulation results for power imbalance test (5MHz) (Communication)

Figure 3: Link level simulation results for power imbalance test (10MHz) (Communication)

From Figure 1, If BLER=5% as the test metric, we can see that when SIR=0 dB, the BLER is much higher than 5% for discovery even when high wanted power signal is given. When SIR=-3 dB, the required SNR to achieve 5% BLER is about 2.9 dB.  If the simulation results based on SIR=0 dB, the required SNR is much higher. The requirements would be very loose and thus it leads to the test meaningless. So, SIR=3 dB is more reasonable assumption.

The required SNRs for discovery and communication are shown in Table 3.

Table 3: Required SNR for discovery and communication

|  |  | Required SNR (dB) @ BLER=5% |
| --- | --- | --- |
| Discovery |  | 2.9 |
| Communication | 5MHz | -4.24 |
|  | 10MHz | -4.94 |

# Conclusion

In this contribution, link level simulation results are provided for power imbalance test. We also provide our analysis on the power imbalance setup and leakage model. We have the following proposal:

Proposal 1:  Rx power imbalance level between two links is 21.5 dB and -24.5dBc leakage into adjacent 2 RBs is assumed.

# Reference

- R4-153676, WF on D2D Demodulation with Multiple sidelinks, LG Electronics, Huawei, HiSilicon, Qualcomm, Intel, Samsung, May 2015

- 36.104, Base Station (BS) radio transmission and reception, 3GPP