3GPP TSG-RAN WG4 Meeting #76 	R4-154817

Beijing, China 24 – 28 August, 2015

Source:	DISH

Title:	TP for 36.862: Filter data and simulations

Work item	LTE_1980_2170_REG1

Agenda item:	8.1.2

Document for:	Approval

## Abstract

This contribution provides filter data / simulations for Band 1 and Band 65

1	 Discussion

Data is presented for B1 based on duplexer filter specification data provided by filter vendors. For B65, data is based on [1] and simulation data from three filter vendors.

2	Filter data and simulation results

Additional data is presented for;

- B1 (worst case) based on standalone duplexer filter specification data provided by different filter vendors.

- B65 (worst case) is based on simulation data from three filter vendors.

Table 2-1: Band 1 and Band 65 filter data

| E-UTRA BAND | Reference | Insertion loss |  | Isolation |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tx | Rx | Tx | Rx |
|  |  | [dB] | [dB] | [dB] | [dB] |
| B1 | Filter 1 | 2.0 | 2.2 | 53 | 45 |
|  | Filter 2 | 2.3 | 2.4 | 55 | 55 |
|  | Filter 3 | 2.4 | 2.5 | 50 | 55 |
|  | Filter 4 | 1.8 | 2.2 | 55 | 44 |
|  | Filter 5 | 2.2 | 2.2 | 54 | 55 |
|  | Filter 6 | 1.9 | 2.2 | 52 | 43 |
|  | Filter 7 | 1.6 | 2.0 | 51 | 48 |
|  | Filter 8 | 1.5 | 2.0 | 53 | 43 |
|  | Filter 9 | 1.6 | 2.0 | 53 | 43 |
|  | Filter 10 | 2.0 | 2.3 | 53 | 50 |
|  | Filter 11 | 1.8 | 2.2 | 44 | 44 |
| B65 | Filter 12 | 2.5 | 3.9 | 52 | 43 |
|  | Filter 13 | 2.8 | 2.8 | 55 | 52 |
|  | Filter 14 | 2.0 | 2.0 | 55 | 55 |

We note that RAN4 does not provide a common template for specifying the filter requirements as part of the band discussions and the filter data that is normally presented in RAN4 is based on the duplex distance to duplex gap ratio and taking into account the adjacent band co-existence requirement. The key figure of merit is the Tx and Rx insertion loss and Tx/Rx isolation as shown in Table 2.1.

### ---------------- Start of TP -----------------------

### 8.1.6	Filter simulations

Worst case filter simulation results for a MSS 90 MHz filter are provided.  Band 1 filter performance is also provided for the purpose of comparison.

Table 8.1.6-1.  Worst case filter simulation results for MSS band 90 MHz filter and Band 1 for the purpose of comparison.

|  | Vendor A MSS 90 MHz | Vendor B MSS 90 MHz | Vendor C MSS 90 MHz | Vendor B Band 1 | Vendor C Band 1 |
| --- | --- | --- | --- | --- | --- |
| Tx IL | 3.5 | 2.5 | TBD | 1.9 | 2.3 |
| Rx IL | 3.5 | 3.9 | TBD | 2.2 | 2.4 |
| Tx isolation | 47 | 52 | TBD | 55 | 55 |
| Rx isolation | 50 | 43 | TBD | 52 | 55 |

Additional data is presented for;

- B1 (worst case) based on standalone duplexer filter specification data provided by different filter vendors.

- B65 (worst case) is based on simulation data from three filter vendors.

Table 8.1.6-2.  Additional B1 and B65 filter data and simulation results

| E-UTRA BAND | Reference | Insertion loss |  | Isolation |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tx | Rx | Tx | Rx |
|  |  | [dB] | [dB] | [dB] | [dB] |
| B1 | Filter 1 | 2.0 | 2.2 | 53 | 45 |
|  | Filter 2 | 2.3 | 2.4 | 55 | 55 |
|  | Filter 3 | 2.4 | 2.5 | 50 | 55 |
|  | Filter 4 | 1.8 | 2.2 | 55 | 44 |
|  | Filter 5 | 2.2 | 2.2 | 54 | 55 |
|  | Filter 6 | 1.9 | 2.2 | 52 | 43 |
|  | Filter 7 | 1.6 | 2.0 | 51 | 48 |
|  | Filter 8 | 1.5 | 2.0 | 53 | 43 |
|  | Filter 9 | 1.6 | 2.0 | 53 | 43 |
|  | Filter 10 | 2.0 | 2.3 | 53 | 50 |
|  | Filter 11 | 1.8 | 2.2 | 44 | 44 |
| B65 | Filter 12 | 2.5 | 3.9 | 52 | 43 |
|  | Filter 13 | 2.8 | 2.8 | 55 | 52 |
|  | Filter 14 | 2.0 | 2.0 | 55 | 55 |

We note that RAN4 does not provide a common template for specifying the filter requirements as part of the band discussions and the filter data that is normally presented in RAN4 is based on the duplex distance to duplex gap ratio and taking into account the adjacent band co-existence requirement. The key figure of merit is the Tx and Rx insertion loss and Tx/Rx isolation.

## 8.2	E-UTRA BS specific issues

8.2.1	 BS operating band unwanted emission and general