3GPP TSG RAN WG4 Meeting #76                                      	      R4-155081

Beijing, China

August 24-28, 2015

Agenda Item:	4.2.1.2

Source:	Motorola Mobility

Title:	Almost-Contiguous A-MPR for Band 42 with CA_NS_08

Document for:   Discussion

# Introduction

For intra-band contiguous carrier aggregation, the A-MPR allowed for non-contiguous resource allocations can be very large when CA_NS signalling is used.  The situation is much better for contiguous resource allocations, since the A-MPR allowed for contiguous allocations is generally much less.  While contiguous resource allocations are preferred from an A-MPR perspective, there is an issue in that contiguous PUSCH allocations that span the boundary of the aggregated carriers will overlap the PUCCH resource blocks that are semi-statically configured at the edge of the component carriers, and will thus degrade the performance of the PUCCH.  It can also be observed that if the aggregated component carriers are equal in size, any contiguous allocation ratio having an allocation ratio of 0.5 or greater will either overlap both PUCCH regions on one component carrier, or will overlap at least one PUCCH region on both carriers.  As a result, though contiguous resource allocations are preferred from an A-MPR perspective, it will generally not be possible to assign contiguous allocations which span the boundary between carriers or which have an allocation ratio greater than or equal to 0.5 dB while avoiding the PUCCH region.

This issue has been discussed previously in [1-7].  In [1], it was proposed that the A-MPR for contiguous resource allocations could be modified to apply to resource allocations that are contiguous except for puncturing by the PUCCH region.

In this contribution, the proposal in [1] is considered for Band 42 with CA_NS_08 signalling.  Since the A-MPR tables for contiguous resource allocations and the A-MPR formula for non-contiguous resource allocations have not yet been specified, the proposed A-MPR tables and proposed A-MPR equation for CA_NS_08 from [7] are used in the analysis. Using the A-MPR from [7], the reduction of the allowed A-MPR is evaluated for resource allocations which are contiguous except for puncturing by the PUCCH region.  The A-MPR margin is then evaluated for these same almost-contiguous allocations using simulation.

# A-MPR Difference between Contiguous and Non-Contiguous Allocations

As discussed in [1], there are two methods used in TS 36.101 for specifying the maximum allowed A-MPR for intra-band carrier aggregation when CA_NS signalling is used.  For each CA_NS_0x signal from CA_NS_01 to CA_NS_07, a table is defined for contiguous resource allocations and a formula is defined for non-contiguous resource allocations.  In general, contiguous resource allocations are preferred because the allowed A-MPR is typically significantly less for a contiguous allocation than for a non-contiguous allocation with the same number of resource blocks.

The non-contiguous A-MPR formula in [7] is given by

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows:

MA =	20				0 ≤ A < 0.025

23 – 120A			0.025 ≤ A < 0.05

17.53 – 10.59A			0.05 ≤ A ≤ 0.9

8				0.9 ≤ A ≤ 1

With this proposal, the A-MPR allowed for an allocation ratio of 0.5 is 12.5 dB.  In Table 1 below, the

For contiguous resource allocations, the allowed A-MPR is a function of both the size of the allocation and the placement of the allocation.  In Table 1, the minimum A-MPR is shown for a contiguous allocation with an allocation ratio of 0.5 for CA_NS_08 using A-MPR table in [7], where the minimum is taken over all possible placements of the allocation.  The minimum A-MPR for an allocation ratio of 0.5 is shown for each allowed bandwidth combination.  Also shown in Table is the reduction in A-MPR that can be achieved by using a contiguous allocation rather than a non-contiguous allocation for an allocation ratio of 0.5.  In all cases, the A-MPR reduction achievable by using a contiguous allocation is equal to 4.5 dB.

- Table 1: A-MPR reduction from using contiguous A-MPR tables rather than non-contiguous A-MPR formula for contiguous allocations for CA_NS_08  [7]

| Aggregated Bandwidth | A-MPR for A = 0.5 with non-contiguous formula [7] | Minimum A-MPR for contiguous allocation with A = 0.5 [7] | A-MPR reduction for contiguous allocation |
| --- | --- | --- | --- |
| 39.8 MHz | 12.5 | 8 | 4.5 dB |
| 34.85 MHz | 12.5 | 8 | 4.5 dB |
| 29.9 MHz | 12.5 | 8 | 4.5 dB |
| 24.95 MHz | 12.5 | 8 | 4.5 dB |

# Reducing A-MPR for Almost-Contiguous Resource Allocations

In [1], it was proposed that the A-MPR for almost-contiguous resource allocations could be specified as the minimum of the existing A-MPR for non-contiguous resource allocations and a second A-MPR defined for almost-contiguous resource allocations which is the sum of the A-MPR for the smallest contiguous RB allocation containing the non-contiguous resource allocation and an adjustment factor which depends on the fraction of RB’s which are punctured from the contiguous allocation.  In particular, the specification [5] already defines RBstart and RBend for any resource block allocation as

RBstart 	Indicates the lowest RB index of transmitted resource blocks

RBend	Indicates the highest RB index of transmitted resource blocks

Furthermore, for a contiguous resource allocation, the specification [5] defines

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

For any resource allocation, the total number of transmitted resource blocks is denoted in [5] as

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

It can be noted that the A-MPR for any contiguous resource allocation can be determined from the A-MPR table proposed in [7] using only  RBstart, RBend, and LCRB.  If we denote the A-MPR for a contiguous allocation as

MC(RBstart, RBend, LCRB)

then the A-MPR for an almost-contiguous resource allocation punctured only by the PUCCH can denoted as

MAC(RBstart, RBend, NRB_alloc),

and can be computed as

MAC(RBstart, RBend, NRB_alloc) = min{ MC(RBstart, RBend, RBend – RBstart + 1)

+  10*log10((RBend – RBstart + 1) /NRB_alloc),

MA  }

where MA denotes the existing A-MPR for the non-contiguous resource allocation with the given allocation ratio equal to A = NRB_alloc / NRB_agg.  The minimum of MAC(RBstart, RBend, NRB_alloc) and MA is taken to handle the possibility that cases exist for which the almost-contiguous A-MPR MAC(RBstart, RBend, NRB_alloc) is greater than the existing non-contiguous A-MPR  MA.

# A-MPR reduction for CA_NS_08 signalling

In this section, the A-MPR reduction that would result with the proposed change is evaluated for CA_NS_08 using the proposed tables and formula in [7].  It is assumed that 4 PUCCH resources are semi-statically configured at the edge of the two carriers.  Figures 1-4 indicate the A-MPR reduction that would result with the proposed change with the signalling of CA_NS_08 for aggregated bandwidths of 39.8 MHz (100  + 100 RB’s), 34.85 MHz (75 + 100 RB’s), 29.9 MHz (50 + 100 RB’s), and 24.95 MHz (25 RB’s + 100 RB’s). In each Figure, the vertical axis indicates the number of allocated RB’s.  If the allocation does not cross the boundary between the two component carriers, then the allocation is contiguous and the number of allocated RB’s is given by

NRB_alloc = RBend – RBstart + 1 .

Alternatively, if the allocation crosses the boundary between the two carriers, the allocation is not contiguous, but is instead almost contiguous, and the number of allocated RB’s is given by

NRB_alloc = (RBend – RBstart + 1) – 8,

since there are 8 RB’s allocated for PUCCH at the edge of the two carriers.

In Figures 1-4, only the almost-contiguous allocations are shown, since only these allocations benefit from the proposed change.  Thus, for all of the allocations shown in the table, it is the case that

NRB_alloc = (RBend – RBstart + 1) – 8.

Also, for all of the resource allocations shown in Figures 1-4, the number of RB’s on each component carrier falls within the set of the allowed PUSCH allocation sizes.

Figure 1:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_08 for an aggregated bandwidth of 39.8 MHz (100 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

Figure 2:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_08 for an aggregated bandwidth of 34.85 MHz (75 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

Figure 3:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_08 for an aggregated bandwidth of 29.9 MHz (50 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

Figure 4:  Reduction of A-MPR for almost-contiguous resource allocations for CA_NS_08 for an aggregated bandwidth of 24.95 MHz (25 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

If 4 RB’s are semi-statically configured for the PUCCH at the edge of each of two contiguously aggregated 20 MHz component carriers, then all resource block allocations with more than 92 RB’s must be non-contiguous due to blocking by the PUCCH region.  However, since 92 RB’s is not an allowed allocation, the largest allowed contiguous allocation is 90 RB’s, and thus, the largest allocation for which the contiguous A-MPR table can be applied.  It can be noted that for an allocation of 90 RB’s, the A-MPR allowed by the  non-contiguous formula in [7] is 13 dB.

# Simulation of A-MPR with Almost-Contiguous Resource Allocations for CA_NS_08 Signalling

In this section, simulation results are presented for CA_NS_08 for the allowed carrier aggregation bandwidth combinations.   The simulations were performed using a 2 GHz PA model.  It should be noted that concerns were expressed in [9] over the use of these models at 3.5 GHz, as it was observed that the A-MPR could be underestimated for some RB allocations.

The modulator impairments used in the simulations were the nominal impairments which apply above 1 GHz:

- Modulator IQ – image = -25 dB

- Modulator carrier leakage = -25 dBc

- Modulator C_IM3 = -60 dBc

The operating point of the PA was set such that the UTRAACLR1 requirement of 33 dB is just met with a 100 RB QPSK allocation, and the output power at which the requirement was just met was defined to be 22 dBm.

The emissions requirements used were

- E-UTRAACRL = 30dB

- UTRAACLR1 = 33dB

- UTRAACLR2 = 36dB

Also, the general E-UTRA CA spectrum emissions mask for Bandwidth Class C was applied as specified in TS 36.101 Table 6.6.2.1A-1.

When CA_NS_08 is signalled, the following additional requirements apply.

Table 1: CA_NS_08 Additional requirement.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
| 3400 ≤ f ≤ 3800 | -23 (Note 1, Note 3) | 5 MHz |
|  | -40 (Note 2) | 1 MHz |
| Note 1: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth. Note 2: This requirement applies from 3400 MHz up to 25 MHz below the lower E-UTRA channel edge and from 25MHz above the upper E-UTRA channel edge up to 3800 MHz. Note 3: This emission limit can imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

In the simulations, it was assumed that the outermost 4 resource blocks of each of the two component carriers were allocated for the PUCCH.  Since the allowed A-MPR is the same for both QPSK and 16QAM modulation,  16 QAM modulation was used for all simulations as the required A-MPR is typically larger for 16QAM than for QPSK. All possible “almost-contiguous” resource allocations were simulated, where almost-contiguous resource allocations are resource allocations which cross the boundary between the carriers and which are contiguous with the PUCCH region (the same as was done for Figures 1-4).

Figures 5-8 indicate the A-MPR margin that would result with the proposed change for CA_NS_08 for aggregated bandwidths of 39.8 MHz (100  + 100 RB’s), 34.85 MHz (75 + 100 RB’s), 29.9 MHz (50 + 100 RB’s), and 24.95 MHz (25 RB’s + 100 RB’s).   In all cases, the A-MPR margin is positive.  This is similar to the result observed for previous simulation results for CA_NS_01 to CA_NS_07 in [5] and in [6].  Conversely, in [7], measurement results were provided showing that the almost-contiguous A-MPR proposed previously was not sufficient to meet emissions requirements for CA_NS_05 and CA_NS_06.  However, it seems that the PA’s used for the  measurements in [7] did not meet the emissions requirement for contiguous allocations with the A-MPR currently allowed in the specification [9] , and thus it is not surprising that the almost-contiguous A-MPR was not sufficient either as it assumes that the PA is biased so that it can meet the existing requirements for contiguous allocations.

Figure 5:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_08 for an aggregated bandwidth of 39.8 MHz (100 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

Figure 6:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_08 for an aggregated bandwidth of 34.85 MHz (75 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

Figure 7:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_08 for an aggregated bandwidth of 29.9 MHz (50 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

.

Figure 8:  A-MPR margin with the proposed reduction of A-MPR for CA_NS_08 for an aggregated bandwidth of 24.95 MHz (25 RB’s + 100 RB’s) with 4 PUCCH resources on each CC.

# Almost-Contiguous Resource Allocation

The question remains as to how precisely define an almost-contiguous resource allocation.  As noted above, the reasons for defining an almost-contiguous resource allocation are that

- For a given allocation ratio, the A-MPR for a contiguous allocation is typically much less than the A-MPR for a non-contiguous allocation.

- Many, if not most subframes include PUCCH transmissions, and the PUCCH resources are typically configured at the edges of the carrier.

- When equal bandwidth component carriers are aggregated, it is not possible to have a resource allocation with an allocation ratio of 0.5 or greater which is contiguous and which does not collide with the PUCCH regions at the edge of the component carriers.

While the UE only has knowledge of the location of its own PUCCH resources and not the location of all of the semi-statically configured PUCCH resources, it does not really need to know the entire PUCCH region configured by the network.  The reason for this is that the A-MPR needed by the UE to meet the emissions requirement for an almost-contiguous resource allocation does not depend on why the RB’s at the edge of the carrier were punctured; that is, whether these resource blocks were punctured for use by another UE as PUCCH or PUSCH.  Rather, it only matters that these RB’s were not included in this UE’s allocation.

Figure 9:  A non-contiguous resource allocation over two aggregated component carriers

A non-contiguous resource allocation is shown in Figure 9.  In the Figure, the resource allocation within each component carrier is contiguous.  The gap between the left allocation and the boundary between the two component carriers is labelled as Gap 1.  The gap between the right allocation and the boundary between the component carriers is labelled as Gap 2.  Using Figure 9, we propose to define an almost-contiguous resource allocation as the following.

Almost-contiguous resource allocation: For two contiguously aggregated carriers, an almost-contiguous resource allocation is a non-contiguous resource allocation with resource blocks allocated in each of the two component carriers, for which the resource allocation in each component carrier is contiguous, and for which the number of unallocated resource blocks between each of the contiguous resource allocations and the boundary between the aggregated carriers is less than or equal to four resource blocks.

The reason for limiting the gaps between each of the contiguous allocations and the boundary between the aggregated carriers to four resource blocks is that to date simulation results have only been provided for this gap size.  If the configured PUCCH region is often larger than four resource blocks, then it may be necessary to consider a larger gap size.

Note that with this definition, it is not necessary to signal to the UE with each allocation whether or not the almost-contiguous A-MPR is applicable.  For a given allocation, the UE can determine whether or not the almost-contiguous A-MPR is applicable from the allocation and the definition of almost-contiguous resource allocation.

# Summary

In this contribution as well as references [1-6], it has been noted that the A-MPR allowed for non-contiguous resource allocations is often much larger than that allowed for a contiguous resource allocation having the same allocation ratio.  As a result, contiguous resource allocations are generally preferred since less A-MPR is allowed for these allocations.  However, if the aggregated carriers have equal bandwidth, there is a problem in that it is not possible to have a contiguous resource allocation with an allocation ratio of 0.5 or greater in a subframe in which any user transmits on the PUCCH, if the PUCCH is configured at the edge of the component carriers.

In order to allow reduced A-MPR for resource allocations having an allocation ratio of 0.5 or greater, it has been proposed to define an almost-contiguous resource allocation as an allocation which is contiguous except for puncturing by the PUCCH region at the edge of the component carriers and to define the A-MPR for such almost-contiguous allocations as described in Section 3.  The significant reduction in A-MPR that can be achieved with this proposal is shown in Section 4, and this reduction is on the order of 4.5 dB for allocation ratios of 0.5.  The simulation results in Section 5 show the A-MPR margin with the proposed change.  The simulation results seem to indicate that the A-MPR allowed with the proposed change is adequate to meet the emissions requirements.

.

# References

- R4-146503, A-MPR for Contiguous CA with Almost-contiguous Resource Allocations and CA_NS Signaling, Motorola Mobility, RAN4 #72bis.

- R4-147328, Almost-contiguous resource allocation, Nokia, RAN4 #73.

- R4-150925, A-MPR for CA with Almost-contiguous Resource Allocations and CA_NS signaling, Motorola Mobility, RAN4 #74.

- R4-152218, A-MPR Reduction for Contiguous CA with Almost-contiguous Resource Allocations, Motorola Mobility, RAN4 #74bis.

- R4-153393, A-MPR for Almost Contiguous Resource Allocations, Motorola Mobility, RAN4#75.

- R4-153591, CA_NS for no additional requirements and almost contiguous MPR scheme, Nokia Networks, RAN4#75

- R4-153399, Almost-contiguous Evaluation Results, Qualcomm RAN4#75

- R4-153801, B42C CA_NS_08 A-MPR, Qualcomm, RAN4#75.

- R4-73AH-0088, On concerns about validity of B42&43 PA model in A-MPR simulations, Nokia, RAN4 #73 Ad Hoc.

- 3GPP TS 36.101.