3GPP TSG-RAN WG4 Meeting #76 	R4-155115

# Beijing, China, 24 - 28 Aug 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3130 | rev | 1 | Current version: | 12.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TM9 performance with CRS assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CRSIM-Perf |  |  |  |  |  |  | Date: |  | 2015-08 -11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel- |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | RAN4 agreed to have TM9 performance requirements when CRS assistance information is provided. Up to now, there is no section to capture these requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TM9 performance requirements when CRS assistance informaiton is provided |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no TM9 requirements in case CRS assistance inforamation is provided |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### 8.3.1.1C	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.1.1C-2, with the addition of parameters in Table 8.3.1.1C-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.1.1C-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.1.1C-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  |  |  | dB |  |  |  | 0 |  | 0 |  | 0 |  |  |  |  |
|  |  |  |  |  |  |  | dB |  |  |  | 0 (Note 1) |  | 0 (Note 1) |  | 0 (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | dB |  |  |  | -3 |  | -3 |  | -3 |  |  |  |  |
| at antenna port |  |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  |  |  |  |  | dB |  |  |  | Reference Value in Table 2 |  | [10.45] |  | [4.6] |  |  |  |  |
| BWChannel |  |  |  |  |  |  | MHz |  |  |  | 10 |  | 10 |  | 10 |  |  |  |  |
| Subframe Configuration |  |  |  |  |  |  |  |  |  |  | Non-MBSFN |  | Non-MBSFN |  | Non-MBSFN |  |  |  |  |
| Time Offset between Cells |  |  |  |  |  |  | s |  |  |  | N/A |  | 3 |  | -1 |  |  |  |  |
| Frequency shift between Cells |  |  |  |  |  |  | Hz |  |  |  | N/A |  | 300 |  | -100 |  |  |  |  |
| Cell Id |  |  |  |  |  |  |  |  |  |  | 0 |  | 1 |  | 128 |  |  |  |  |
| Cell-specific reference signals |  |  |  |  |  |  |  |  |  | Antenna ports 0,1 |  |  |  |  |  |  |  |  |  |  |
| CSI reference signals |  |  |  |  |  |  |  |  |  |  | Antenna ports 15,16 |  | N/A |  | N/A |  |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  |  |  |  | Subframes |  |  |  | 5 / 2 |  | N/A |  | N/A |  |  |  |  |
| CSI reference signal configuration |  |  |  |  |  |  |  |  |  |  | 8 |  | N/A |  | N/A |  |  |  |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  |  |  |  | Subframes / bitmap |  |  |  | [3 / 0010000000000000] |  | N/A |  | N/A |  |  |  |  |
| Number of control OFDM symbols |  |  |  |  |  |  |  |  |  |  | 2 |  | 2 |  | 2 |  |  |  |  |
| PDSCH transmission mode |  |  |  |  |  |  |  |  |  |  | TM9-1layer |  | N/A |  | N/A |  |  |  |  |
| Precoding granularity |  |  |  |  |  |  |  |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms |  | N/A |  | N/A |  |  |  |  |
| Beamforming model |  |  |  |  |  |  |  |  |  |  | Annex B.4.1 |  | N/A |  | N/A |  |  |  |  |
| Interference model |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | As specified in clause B.5.4 | As specified in clause B.5.4 |  |  |  |
| Probability of occurrence of transmission in interference cells |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  | 20 | 20 |  |  |  |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | % | N/A | 80 | 80 |
|  |  |  | Rank 2 |  |  | % |  |  |  | N/A |  |  |  |  |  | 20 | 20 |  |  |  |  |
| Cyclic prefix |  |  |  |  |  |  |  |  |  |  | Normal |  | Normal |  | Normal |  |  |  |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 4: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 5: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3.1.1C-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | TBD | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 |  |  | 2x2 Low | 70 | TBD | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### 8.3.2.1D	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.2.1.D-2, with the addition of parameters in Table 8.3.2.1.D-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.2.1.D-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell 2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.2.1.D-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  |  |  |  | Unit |  |  | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  |  |  |  |  |  | 1 |  |  | 1 | 1 |  |  |  |  |
| Special subframe configuration |  |  |  |  |  |  |  |  | 4 |  |  | 4 | 4 |  |  |  |  |
| Downlink power allocation |  |  |  |  |  | dB |  |  | 0 |  |  | 0 | 0 |  |  |  |  |
|  |  |  |  |  |  | dB |  |  | 0 (Note 1) |  |  | 0 (Note 1) | 0 (Note 1) |  |  |  |  |
|  |  |  |  |  |  | dB |  |  | -3 |  |  | -3 | -3 |  |  |  |  |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  |  |  |  | dB |  |  | Reference Value in Table 2 |  |  | [10.45] | [4.6] |  |  |  |  |
| BWChannel |  |  |  |  |  | MHz |  |  | 10 |  |  | 10 | 10 |  |  |  |  |
| Subframe Configuration |  |  |  |  |  |  |  |  | Non-MBSFN |  |  | Non-MBSFN | Non-MBSFN |  |  |  |  |
| Time Offset between Cells |  |  |  |  |  | s |  |  | N/A |  |  | 3 | -1 |  |  |  |  |
| Frequency shift between Cells |  |  |  |  |  | Hz |  |  | N/A |  |  | 300 | -100 |  |  |  |  |
| Cell Id |  |  |  |  |  |  |  |  | 0 |  |  | 1 | 126 |  |  |  |  |
| Cell-specific reference signals |  |  |  |  |  |  |  | Antenna ports 0,1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI reference signals |  |  |  |  |  |  |  |  | Antenna ports 15,16 |  |  | N/A | N/A |  |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  |  |  | Subframes |  |  | 5 / 4 |  |  | N/A | N/A |  |  |  |  |
| CSI reference signal configuration |  |  |  |  |  |  |  |  | 8 |  |  | N/A | N/A |  |  |  |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  |  |  | Subframes / bitmap |  |  | [4 / 0010000000000000] |  |  | N/A | N/A |  |  |  |  |
| Number of control OFDM symbols |  |  |  |  |  |  |  |  | 2 |  |  | 2 | 2 |  |  |  |  |
| PDSCH transmission mode |  |  |  |  |  |  |  |  | TM9-1layer |  |  | N/A | N/A |  |  |  |  |
| Interference model |  |  |  |  |  |  |  | N/A |  |  | As specified in clause B.5.4 |  |  |  | As specified in clause B.5.4 |  |  |  |  |
| Probability of occurrence of transmission in interference cells |  |  |  |  | % |  |  |  |  |  | 20 |  |  |  | 20 |  |  |  |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  |  | % |  |  | N/A |  |  | 80 |  |  |  | 80 |  |  |  |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| Precoding granularity |  |  |  |  |  |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms |  |  | N/A | N/A |  |  |  |  |
| Beamforming model |  |  |  |  |  |  |  |  | Annex B.4.1 |  |  | N/A | N/A |  |  |  |  |
| Cyclic prefix |  |  |  |  |  |  |  |  | Normal |  |  | Normal | Normal |  |  |  |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 5: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 6: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.1.D-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | TBD | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 |  |  | 2x2 Low | 70 | TBD | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

## B.5.4	Transmission mode 9 interference model

This subclause provides transmission mode 9 interference modelling for each explicitly modelled interfering cell in the requirement scenario. In each subframe, each interfering cell shall transmit randomly modulated data over the entire PDSCH region and the full transmission bandwidth according to the probabilities of occurrence. Transmitted physical channels shall include PSS, SSS and PBCH. Probabilites of occurrence in each subframe are as specified in the requirement scenario. If the probabilities of occurrence in each subframe are not specified in the requirement scenario, as default, they are equal to 1.

For each subframe and each CQI subband as defined in subclause 7.2 of [6], a transmission rank shall be randomly determined independently from other CQI subbands as well as other interfering cells. Probabilities of occurrence of each possible transmission rank are as specified in the requirement scenario.

For each subframe and each CQI subband, a precoding matrix for the number of layers  associated to the selected rank shall be selected randomly from Table 6.3.4.2.3-2 of [4].

The generic beamforming model in subclause B.4.3 shall be applied assuming cell-specific reference signals and CSI reference signals as specified in the requirement scenario. Random precoding with selected rank and precoding matrices for each subframe and each CQI subband shall be applied to 16QAM randomly modulated layer symbols including the user-specific reference symbols over antenna port 7 when the rank is one and antenna ports 7, 8 when the rank is two.

For unallocated REs in the control region, precoding for transmit diversity for the number of antenna ports in the requirement scenario shall be applied to QPSK randomly modulated layer symbols, as specified in subclause 6.3.4.3 of [4]. The EPRE ratio for these REs shall be as defined for PDCCH in Annex C.3.2.

----------------------------------------------------- End  of Change ------------------------------------------------------------