3GPP TSG-RAN WG4 (Radio) Meeting #76	R4-153991

Beijing, China, 24 – 28 August 2015

Agenda Item:	7.20.1

Source: 	Alcatel-Lucent, Sprint, Qualcomm Incorporated

Title: 	TP for TR 36.833-8-41: Specific UE RF requirements for LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4DL

Document for:	Approval

# 1.	Introduction

The LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4 DL work item was approved at TSG RAN #66 [1]. This contribution discusses the specific UE RF requirements for supporting 4-CC DL intra-band non-contiguous CA for Band 41, and provides a text proposal to include the findings into the TR 36.833-8-41 [2] of this work item.

# 2.	Discussion

2.1	Reference sensitivity power level

Band 41 is a TDD band and there is no simultaneous downlink and uplink operation in the UE. In general, TDD UE uplink transmission has no impact on its reference sensitivity, i.e. the ΔRIBNC = 0 for CA_41C-41C, CA_41A-41D, and CA_41D-41A. These CA configurations should be added to Table 7.3.1A-3 of TS 36.101.

2.2	In-band blocking

The current in-band blocking requirements specified in Clause 7.6.1 of TS 36.101 for UE supporting intra-band non-contiguous carrier aggregation with two downlink sub-blocks are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the UE in intra-band non-contiguous CA operation, and thus can be applied for CA_41C-41C, CA_41A-41D, and CA_41D-41A. Hence no change in the in-band blocking requirements is needed.

2.3	Out-of-band blocking

The current out-of-band blocking requirements specified in Clause 7.6.2 of TS 36.101 for UE supporting intra-band non-contiguous carrier aggregation with two downlink sub-blocks are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the UE in intra-band non-contiguous CA operation, and thus can be applied for CA_41C-41C, CA_41A-41D, and CA_41D-41A. Hence no change in the out-of-band blocking requirements is needed.

# 3.	Conclusion

This contribution discusses the specific UE RF requirements for supporting 4-CC DL intra-band non-contiguous CA for Band 41. It is proposed to include the text proposal below into TR 36.833-8-41.

# 4.	Text proposal

<Start of change>

### 5.2.1	Reference sensitivity power level

Band 41 is a TDD band and there is no simultaneous downlink and uplink operation in the UE. In general, TDD UE uplink transmission has no impact on its reference sensitivity, i.e. the ΔRIBNC = 0 for CA_41C-41C, CA_41A-41D, and CA_41D-41A. These CA configurations should be added to Table 7.3.1A-3 of TS 36.101.

### 5.2.2	In-band blocking

The current in-band blocking requirements specified in Clause 7.6.1 of TS 36.101 for UE supporting intra-band non-contiguous carrier aggregation with two downlink sub-blocks are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the UE in intra-band non-contiguous CA operation, and thus can be applied for CA_41C-41C, CA_41A-41D, and CA_41D-41A. Hence no change in the in-band blocking requirements is needed.

### 5.2.3	Out-of-band blocking

The current out-of-band blocking requirements specified in Clause 7.6.2 of TS 36.101 for UE supporting intra-band non-contiguous carrier aggregation with two downlink sub-blocks are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the UE in intra-band non-contiguous CA operation, and thus can be applied for CA_41C-41C, CA_41A-41D, and CA_41D-41A. Hence no change in the out-of-band blocking requirements is needed.

<End of change>

# References

[1]	RP-142022, “New WID: LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4DL”, Sprint.

[2]	R4-151292, “TR 36.833-8-41 V0.1.0”, Alcatel-Lucent, Sprint.

[3]	3GPP TS 36.101 v11.11.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.