3GPP TSG-RAN WG4 (Radio) Meeting #76	R4-154000

Beijing, China, 24 – 28 August 2015

Agenda Item:	9.3

Source: 	Alcatel-Lucent

Title: 	System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm

Document for:	Approval

# 1.	Introduction

The study item on New Band 41 UE power class supporting +26 dBm was approved at TSG RAN #68 [1]. The objectives of this study item include the co-existence and compatibility of LTE systems deployed in the 2.5 GHz band and the impact on eNode B blocking requirements. Hence system level simulations need to be performed for the coexistence study.

This contribution proposes the system level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm, and provides a text proposal to record the assumptions and methodology into the TR 36.8xx [2] for this study item.

# 2.	Discussion

The simulation methodology and assumptions shall mainly be based on TR 36.942 [3], with slight modifications to better represent operator deployment scenario and 26 dBm UE operation.

#### 2.1	Cell layouts

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 2.1), with wrap around. The number of sites shall be equal to or higher than 19 [3]. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure 2.1: Uncoordinated macro cellular deployment

The inter-site distances considered in this study are provided in Table 2.1 below.

Table 2.1: Inter-site distances

#### 2.2	Macro cell Propagation model - Urban and Suburban Areas

Macro cell propagation model for urban area is applicable for scenarios in urban and suburban areas outside the high rise core where the buildings are of nearly uniform height [3]:

(1)

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Dhb is the base station antenna height in metres, measured from the average rooftop level

Considering a carrier frequency of 2.6 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

(2)

where:

R is the base station-UE separation in kilometres

#### 2.3	Macro cell Propagation model - Rural Area

For rural area, the Hata model in [3] is not applicable for a carrier frequency of 2.6 GHz, while the modified Hata model can be used [4]:

Case 1:		d  0.04 km

(3)

where:

d is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Hm is the mobile antenna height in metres

Hb is the base station antenna height in metres

Case 2:		d  0.1 km

(4)

where:

Note that for short range devices in the case of low base station antenna height, Hb,  is replaced by:

Case 3:		0.04 km  d  0.1 km

(5)

When L is below the free space attenuation for the same distance, the free space attenuation should be used instead.

Considering a carrier frequency of 2.6 GHz, a base station antenna height of 45 meters above ground, a mobile antenna height of 1.5 meters above ground, and a base station-UE separation smaller than 20 km, equations (3) and (4) can be simplified, respectively, as equations (6) and (7) below:

Case 1:		d  0.04 km

(6)

Case 2:		d  0.1 km

(7)

Note that the propagation loss using equation (7) is less than that using equation (6) for a base station-UE separation smaller than 0.6 km and the propagation loss in equation (6) should be chosen. Hence equation (5) need not be used and the propagation model can be given by the following equations:

Case 1:		d  0.6 km

(8)

Case 2:		d  0.6 km

(9)

where:

d is the base station-UE separation in kilometres

#### 2.4	Power control modelling

In TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(10)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. For 26 dBm UE, it is assumed that Rmin has 3dB more dynamic range.

The parameter sets for power control are specified in TR 36.942 (copied in Table 2.2 below).

Table 2.2: Power control algorithm parameter

| Parameter set | Gamma | CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 109 | 110 | 112 | 115 |
| Set 2 | 0,8 | TBD | TBD | 129 | 133 |

However, the power control parameters were specified for carrier frequency of 2 GHz and 500 m cell range. The path loss model is based on the equation below [3]:

(11)

where:

R is the base station-UE separation in kilometres

Based on equations (2), (9) and (11), the power control parameters can be modified to account for different propagation model, carrier frequency, BS antenna height and cell range. For example, in Table 2.2, CLx-ile = 109 dB for Set 1and 20 MHz bandwidth, which corresponds to R = 0.31 km according to equation (11) assuming total antenna gain of 0 dBi. For 1.03 km inter-site distance and 2.6 GHz carrier frequency, using equation (2), CLx-ile = 130.5 + 37.6*log10(1.03*2/3/0.5*0.31) = 117 dB. Similarly, the modified power control parameters for other cases can be calculated as shown in Table 2.3 below.

Table 2.3: Modified power control algorithm parameter

(a) For 1.03 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 117 | 118 | 120 | 123 |
| Set 2 | 0,8 | TBD | TBD | 137 | 141 |

(b) For 1.52 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 123 | 124 | 126 | 129 |
| Set 2 | 0,8 | TBD | TBD | 143 | 147 |

(c) For 2.83 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 133 | 134 | 136 | 139 |
| Set 2 | 0,8 | TBD | TBD | 153 | 157 |

(d) For 6.31 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 118 | 119 | 121 | 124 |
| Set 2 | 0,8 | TBD | TBD | 136 | 140 |

(e) For 7.99 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 122 | 123 | 124 | 127 |
| Set 2 | 0,8 | TBD | TBD | 140 | 143 |

#### 2.5	Practical considerations for 26 dBm UE deployment

The fractional power control equation (10) can be simplified into the following

for 	(12)

Note that the unit in the equation above is dB. If a UE is close to its serving eNodeB such that , whether it is a 23 dBm UE or a 26 dBm UE, it should transmit similar power and the transmitted power should be less than 23 dBm. The 26 dBm UE extends the cell range by being able to transmit more than 23 dBm power at areas where the 23 dBm UE can only transmit at its maximum power of 23dBm.

From equation (12), we have  and , where denotes the transmit power of a 23 dBm UE at a location with coupling loss , and denotes the transmit power of a 26 dBm UE at a location with the path loss of . Based on the reasoning above, we have and hence

(13)

where:

and

Based on equation (13) and Table 2.3, the modified power control parameters for 26 dBm UE can be calculated as shown in Table 2.4 below,

Table 2.4: Modified power control algorithm parameter

(a) For 26 dBm UE, 1.03 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 120 | 121 | 123 | 126 |
| Set 2 | 0,8 | TBD | TBD | 140 | 144 |

(b) For 26 dBm UE, 1.52 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 126 | 127 | 129 | 132 |
| Set 2 | 0,8 | TBD | TBD | 147 | 151 |

(c) For 26 dBm UE, 2.83 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 136 | 137 | 139 | 142 |
| Set 2 | 0,8 | TBD | TBD | 157 | 161 |

(d) For 26 dBm UE, 6.31 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 121 | 122 | 124 | 127 |
| Set 2 | 0,8 | TBD | TBD | 140 | 144 |

(e) For 26 dBm UE, 7.99 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 125 | 126 | 127 | 130 |
| Set 2 | 0,8 | TBD | TBD | 144 | 147 |

#### 2.6	Other simulation assumptions

Other simulation assumptions are summarized in Table 2.5 below:

Table 2.5: Simulation parameters for Band 41 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz |  |
| Inter-site distance | Use Table 2.1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 30 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB | 27 dB |

(b) With 26 dBm UE

|  | Base Station |  | HPUE |
| --- | --- | --- | --- |
| Carrier frequency | 2600 MHz |  |  |
| Channel bandwidth | 10 MHz |  |  |
| Inter-site distance | Use Table 2.1 |  |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |  |
| Frequency reuse | 1x3x1 |  |  |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation |  | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) |  | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 30 dB |  | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB |  | 9 dB |
| Transmit power | 46 dBm |  | 26 dBm |
| Antenna height | 45 m |  | 1.5 m |
| ACLR | 45 dB |  | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB |  | 27 dB |

#### 2.6	Simulation procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 41 UL to UL coexistence study, assuming parameters of both systems are according to Table 2.5(a). Power control parameters in Table 2.3 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 41victim system performance degradation results in this scenario are used as the baseline.

2)	Run the Band 41 UL to UL coexistence study, assuming +26 dBm power class UE is deployed in Band 41 interfering system only, and obtain the victim system performance degradation results. The simulation parameters in Tables 2.5(a) and 2.5(b) are used for the victim and interfering system, respectively. And the power control parameters in Tables 2.3 and 2.4 are used for the victim and interfering system, respectively.

3)	Compare the Band 41 victim system performance degradation obtaining in steps 1) and 2), choose the 26 dBm UE ACLR value so that the victim system performance degradation due to 26 dBm UE in 2) is acceptable compared to 1).

# 3.	Conclusion

This contribution proposes the system level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm. It is proposed to include the text proposal below into TR 36.8xx.

# 4.	Text proposal

<Start of change>

# 2.	Simulation methodology and assumptions

The simulation methodology and assumptions shall mainly be based on TR 36.942 [3], with slight modifications to better represent operator deployment scenario and 26 dBm UE operation.

#### 2.1	Cell layouts

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 2.1), with wrap around. The number of sites shall be equal to or higher than 19 [3]. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure 2.1: Uncoordinated macro cellular deployment

The inter-site distances considered in this study are provided in Table 2.1 below.

Table 2.1: Inter-site distances

#### 2.2	Macro cell Propagation model - Urban and Suburban Areas

Macro cell propagation model for urban area is applicable for scenarios in urban and suburban areas outside the high rise core where the buildings are of nearly uniform height [3]:

(1)

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Dhb is the base station antenna height in metres, measured from the average rooftop level

Considering a carrier frequency of 2.6 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

(2)

where:

R is the base station-UE separation in kilometres

#### 2.3	Macro cell Propagation model - Rural Area

For rural area, the Hata model in [3] is not applicable for a carrier frequency of 2.6 GHz, while the modified Hata model can be used [4]:

Case 1:		d  0.04 km

(3)

where:

d is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Hm is the mobile antenna height in metres

Hb is the base station antenna height in metres

Case 2:		d  0.1 km

(4)

where:

Note that for short range devices in the case of low base station antenna height, Hb,  is replaced by:

Case 3:		0.04 km  d  0.1 km

(5)

When L is below the free space attenuation for the same distance, the free space attenuation should be used instead.

Considering a carrier frequency of 2.6 GHz, a base station antenna height of 45 meters above ground, a mobile antenna height of 1.5 meters above ground, and a base station-UE separation smaller than 20 km, equations (3) and (4) can be simplified, respectively, as equations (6) and (7) below:

Case 1:		d  0.04 km

(6)

Case 2:		d  0.1 km

(7)

Note that the propagation loss using equation (7) is less than that using equation (6) for a base station-UE separation smaller than 0.6 km and the propagation loss in equation (6) should be chosen. Hence equation (5) need not be used and the propagation model can be given by the following equations:

Case 1:		d  0.6 km

(8)

Case 2:		d  0.6 km

(9)

where:

d is the base station-UE separation in kilometres

#### 2.4	Power control modelling

In TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(10)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. For 26 dBm UE, it is assumed that Rmin has 3dB more dynamic range.

The parameter sets for power control are specified in TR 36.942 (copied in Table 2.2 below).

Table 2.2: Power control algorithm parameter

| Parameter set | Gamma | CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 109 | 110 | 112 | 115 |
| Set 2 | 0,8 | TBD | TBD | 129 | 133 |

However, the power control parameters were specified for carrier frequency of 2 GHz and 500 m cell range. The path loss model is based on the equation below [3]:

(11)

where:

R is the base station-UE separation in kilometres

Based on equations (2), (9) and (11), the power control parameters can be modified to account for different propagation model, carrier frequency, BS antenna height and cell range. For example, in Table 2.2, CLx-ile = 109 dB for Set 1and 20 MHz bandwidth, which corresponds to R = 0.31 km according to equation (11) assuming total antenna gain of 0 dBi. For 1.03 km inter-site distance and 2.6 GHz carrier frequency, using equation (2), CLx-ile = 130.5 + 37.6*log10(1.03*2/3/0.5*0.31) = 117 dB. Similarly, the modified power control parameters for other cases can be calculated as shown in Table 2.3 below.

Table 2.3: Modified power control algorithm parameter

(a) For 1.03 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 117 | 118 | 120 | 123 |
| Set 2 | 0,8 | TBD | TBD | 137 | 141 |

(b) For 1.52 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 123 | 124 | 126 | 129 |
| Set 2 | 0,8 | TBD | TBD | 143 | 147 |

(c) For 2.83 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 133 | 134 | 136 | 139 |
| Set 2 | 0,8 | TBD | TBD | 153 | 157 |

(d) For 6.31 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 118 | 119 | 121 | 124 |
| Set 2 | 0,8 | TBD | TBD | 136 | 140 |

(e) For 7.99 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 122 | 123 | 124 | 127 |
| Set 2 | 0,8 | TBD | TBD | 140 | 143 |

#### 2.5	Practical considerations for 26 dBm UE deployment

The fractional power control equation (10) can be simplified into the following

for 	(12)

Note that the unit in the equation above is dB. If a UE is close to its serving eNodeB such that , whether it is a 23 dBm UE or a 26 dBm UE, it should transmit similar power and the transmitted power should be less than 23 dBm. The 26 dBm UE extends the cell range by being able to transmit more than 23 dBm power at areas where the 23 dBm UE can only transmit at its maximum power of 23dBm.

From equation (12), we have  and , where denotes the transmit power of a 23 dBm UE at a location with coupling loss , and denotes the transmit power of a 26 dBm UE at a location with the path loss of . Based on the reasoning above, we have and hence

(13)

where:

and

Based on equation (13) and Table 2.3, the modified power control parameters for 26 dBm UE can be calculated as shown in Table 2.4 below,

Table 2.4: Modified power control algorithm parameter

(a) For 26 dBm UE, 1.03 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 120 | 121 | 123 | 126 |
| Set 2 | 0,8 | TBD | TBD | 140 | 144 |

(b) For 26 dBm UE, 1.52 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 126 | 127 | 129 | 132 |
| Set 2 | 0,8 | TBD | TBD | 147 | 151 |

(c) For 26 dBm UE, 2.83 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 136 | 137 | 139 | 142 |
| Set 2 | 0,8 | TBD | TBD | 157 | 161 |

(d) For 26 dBm UE, 6.31 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 121 | 122 | 124 | 127 |
| Set 2 | 0,8 | TBD | TBD | 140 | 144 |

(e) For 26 dBm UE, 7.99 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile for 26 dBm UE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 125 | 126 | 127 | 130 |
| Set 2 | 0,8 | TBD | TBD | 144 | 147 |

#### 2.6	Other simulation assumptions

Other simulation assumptions are summarized in Table 2.5 below:

Table 2.5: Simulation parameters for Band 41 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz |  |
| Inter-site distance | Use Table 2.1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 30 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB | 27 dB |

(b) With 26 dBm UE

|  | Base Station |  | HPUE |
| --- | --- | --- | --- |
| Carrier frequency | 2600 MHz |  |  |
| Channel bandwidth | 10 MHz |  |  |
| Inter-site distance | Use Table 2.1 |  |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |  |
| Frequency reuse | 1x3x1 |  |  |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation |  | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) |  | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 30 dB |  | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB |  | 9 dB |
| Transmit power | 46 dBm |  | 26 dBm |
| Antenna height | 45 m |  | 1.5 m |
| ACLR | 45 dB |  | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X |
| ACS | 45 dB |  | 27 dB |

#### 2.6	Simulation procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 41 UL to UL coexistence study, assuming parameters of both systems are according to Table 2.5(a). Power control parameters in Table 2.3 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 41victim system performance degradation results in this scenario are used as the baseline.

2)	Run the Band 41 UL to UL coexistence study, assuming +26 dBm power class UE is deployed in Band 41 interfering system only, and obtain the victim system performance degradation results. The simulation parameters in Tables 2.5(a) and 2.5(b) are used for the victim and interfering system, respectively. And the power control parameters in Tables 2.3 and 2.4 are used for the victim and interfering system, respectively.

3)	Compare the Band 41 victim system performance degradation obtaining in steps 1) and 2), choose the 26 dBm UE ACLR value so that the victim system performance degradation due to 26 dBm UE in 2) is acceptable compared to 1).

<End of change>

# References

[1]	RP-151104, “Study on- New Band 41 UE power class supporting +26 dBm”, Sprint, CMCC, Alcatel-Lucent, Softbank, C-Spire, Sequans, Ericsson, Intel, Samsung, SouthernLINC, Dish, Blackberry, China Unicom.

[2]	R4-154979, “TR outline proposal for FS_LTE_B41_HPUE”, Sprint Corporation.

[3]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[4]	Report ITU-R SM.2028-1, “Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services or systems”.