3GPP TSG-RAN WG4 #76	R4-154651

August 24th – 28th, 2015

Bejing, China

Agenda item:	7.13.2

Source: 	Qualcomm Incorporated

Title: 	5GHz UE ACS and In-band blocking requirements.

Document for:	Approval

# Introduction

In last RAN meeting held in Malmo (RAN #68), a new work item on Licence-Assisted Access (LAA) to unlicensed spectrum was approved [1]. In this contribution, we make a proposal on how UE minimum requirements for adjacent channel selectivity (ACS) and in-band blocking (IBB) could be derived for LAA.

# Discussion

LAA WID was approved in RAN #68. Among several items specified, the following goal was also included:

The work item should also specify base station and UE core requirements of 5GHz spectrum (based on regulatory limits) including applicable band/bands definition and a limited set of example band combinations. The 5 GHz band/bands definition should include DL only and UL/DL operations (without UL requirements being defined in Rel-13). The 5GHz unlicensed band/bands definition(s) should not introduce new frame structure(s) (if any) and/or new TDD UL/DL configurations (if any) for the licensed bands. (RAN4)”

In this contribution we focus on UE core requirements with emphasis on how the minimum performance for adjacent channel sensitivity and in-band blocking could be introduced for LAA. In particular, we will focus on the difference in terms of interference scenarios between legacy licensed spectrum and 5GHz unlicensed spectrum.

# 5GHz interference environment

RF performance requirements for adjacent channel selectivity and in-band blocking is based on the estimated and/or measured interferer radio environment. In this contribution, following the proposal in [2], we will assume that a single band, namely band 45, will be defined for LAA operation in 5GHz spectrum. In 5GHz band, because of the wide presence of Wi-Fi APs and STAs we can assume that interferer will likely occupy 20MHz channel. Indeed, we can estimate that 20MHz transmission is the more likely to happen and that impact of 20MHz transmission is more detrimental to LAA receiver compared to higher bandwidth. For the above reasons, we will derive core requirements for LAA in 5GHz by assuming that typical interfere will occupy 20MHz channel.

Observation 1: Typical aggressor signal in 5GHz band occupies 20MHz channel.

# UE ACS minimum requirement

Following Observation 1, it can be noted that compared to the legacy requirement, the minimum requirement for ACS in 5GHz band should be updated by considering the different aggressor bandwidth in the test parameters. In other words, we propose to keep same ACS value, i.e. 27dB for 20MHz channels, but increasing the interferer bandwidth to reflect a more realistic interference scenario in 5GHz unlicensed spectrum.

Proposal 1: ACS value for 20MHz channel bandwidth in 5GHz band should be 27dB.

Proposal 2: For 5GHz band, the bandwidth of the interferer signal for ACS test should be 20MHz.

Based on Proposal 2, we suggest to only modify ACS tables for Case 1 and Case 2 as follows.

Table 7.5.1-2: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz |
|  | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
|  | PInterferer | dBm | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB |  | REFSENS +45.5dB | REFSENS +42.5dB | REFSENS +39.5dB |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 293, 30, 31, 323, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44 | BWInterferer | MHz | 1.4 | 3 | 5 |  | 5 | 5 | 5 |
|  | FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 | 5+0.0025 / -5-0.0025 |  | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| 45 | BWInterferer | MHz |  |  |  |  |  |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  |  |  |  | 20+0.0025 / -20.5-0.0025 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The requirement for these bands are applicable only in inter-band carrier aggregation modes. |  |  |  |  |  |  |  |  |  |

Table 7.5.1-3: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
|  | Power in Transmission Bandwidth Configuration | dBm | -56.5 | -56.5 | -56.5 | -56.5 | -53.5 | -50.5 |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 293, 30, 31, 323, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44 | BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
|  | FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| 45 | BWInterferer | MHz |  |  |  |  |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  |  |  | 20+0.0025 / -20.5-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: The requirement for these bands are applicable only in inter-band carrier aggregation modes. |  |  |  |  |  |  |  |  |

In case of carrier aggregation, assuming that only 20MHz channels are allowed in 5GHz, a similar requirement can be derived for the case of intra band carrier aggregation. In other words, even for the CA we propose to leave unchanged the ACS value, namely 24dB for class C CA, and modify the aggressor bandwidth:

Proposal 3: ACS value for intra band CA class C should be 24dB.

Proposal 4: In case of intra-band CA class C in 5GHz band, the bandwidth of the interferer signal for ACS test should be 20MHz.

# UE In-band blocking requirement

Similarly to what done for ACS, we propose to update in-band blocking requirement by taking into account the different interference scenario available in 5GHz band. Based on Observation 1, we propose to update the blocker bandwidth to be 20MHz in the in-band blocking test for 5GHz band.

Proposal 5: For 5GHz band, the bandwidth of the interferer signal for in-band blocking test should be 20MHz.

Following proposal 2, a possible approach for the modification of in band blocking parameters table in TS 36.101 is proposed below:

Table 7.6.1.1-1: In band blocking parameters

| E-UTRA band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
|  | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 293, 31, 323, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44 | BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
|  | FIoffset, case 1 | MHz | 2.1+0.0125 | 4.5+0.0075 | 7.5+0.0125 | 7.5+0.0025 | 7.5+0.0075 | 7.5+0.0125 |
|  | FIoffset, case 2 | MHz | 3.5+0.0075 | 7.5+0.0075 | 12.5+0.0075 | 12.5+0.0125 | 12.5+0.0025 | 12.5+0.0075 |
| 45 | BWInterferer | MHz |  |  |  |  |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  |  |  | 30+0.125 |
|  | FIoffset, case 2 | MHz |  |  |  |  |  | 50+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The requirement for these bands are applicable only in inter-band carrier aggregation modes. |  |  |  |  |  |  |  |  |

Considering the nature of interferer in 5GHz band, the definition of in-band blocking can be also extended as follows:

Proposal 6: In 5GHz band, in-band blocking should be defined for an unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band.

In case of carrier aggregation, similar approach can be adopted to update current specification. Similar to what done in Table 7.6.1.1-1, we propose to update Table 7.6.1.1A-1 in TS 36.101 by considering BWinterferer=20MHz, Foffset, case 1=30MHz and Foffset, case 2=50MHz for case C in 5GHz band.

Proposal 7: In case of intra-band CA class C in 5GHz band, the bandwidth of the interferer signal for in-band blocking test should be 20MHz, Foffset, case 1 should be 30MHz and Foffset, case 2 should be 50MHz.

# Conclusion

In this contribution we analysed UE ACS and in-band blocking minimum requirements for LAA in 5GHz band. Taking into account the different environment scenario due to the large presence of Wi-Fi APs and STAs in 5GHz spectrum, we made the following observation:

Observation 1: Typical aggressor signal in 5GHz band occupies 20MHz channel.

Based on Observation 1, we also made the following proposals for UE ACS and in-band blocking minimum requirements:

Proposal 1: ACS value for 20MHz channel bandwidth in 5GHz band should be 27dB.

Proposal 2: For 5GHz band, the bandwidth of the interferer signal for ACS test should be 20MHz.

Proposal 3: ACS value for intra band CA class C should be 24dB.

Proposal 4: In case of intra-band CA class C in 5GHz band, the bandwidth of the interferer signal for ACS test should be 20MHz.

Proposal 5: For 5GHz band, the bandwidth of the interferer signal for in-band blocking test should be 20MHz.

Proposal 6: In 5GHz band, in-band blocking should be defined for an unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band.

Proposal 7: In case of intra-band CA class C in 5GHz band, the bandwidth of the interferer signal for in-band blocking test should be 20MHz, Foffset, case 1 should be 30MHz and Foffset, case 2 should be 50MHz.

# Reference

- RP-151045, “New Work Item on Licensed-Assisted Access to Unlicensed Spectrum”, Ericsson, Huawei, Qualcomm, Alcatel-Lucent, RAN#68, June 2015.

- R4-153878, “TP for 36.889: Band plan for unlicensed spectrum in 5GHz”, Ericsson, Qualcomm, Intel, Nokia Networks, Huawei, May 2015.

- 3GPP TR 36.889 V13.0.0 (2015-06), “Study on Licensed-Assisted Access to Unlicensed Spectrum”.

- R4-153540, “Band plan for unlicensed spectrum in 5GHz”, Ericsson, Qualcomm, Intel, Nokia Networks, May 2015.