3GPP TSG-RAN WG4 Meeting #76	R4-154716

# Beijing, China, 24-28 August 2015

Agenda item:	7.40.2.2

Source: 	China Telecom, China Unicom

Title: 	TP for TR36.852-13 on CA_3A-41A

Document for:	Approval

1. Introduction

The WID on B3+B41 2DL TDD-FDD CA was approved in [1]. In this contribution, we propose TIB,c and RIB,c requirements for this TDD-FDD CA combination , while the MSD is also proposed to finalize the open issues of this WI.

2. Conclusion

We proposal to accept the text for TR36.852-13 to capture TIB,c and RIB,c and reference sensitivities for B3+B41 2DL/1UL band combination.

3.	References

- RP-142239, “New WI Proposal: LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 41”, China Telecom, China Unicom

<< Unchanged sections omitted >>

<< Start of changed sections >>

7.2.8.1.3	∆TIB and ∆RIB values

For two simultaneous DLs and one UL the TIB,c and RIB values are shown in table 7.2.8.1.3-1, and in table 7.2.8.1.3-2:

# Table 7.2.8.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-41A | 3 | 0.5 |
|  | 41 | 0.8 |

# Table 7.2.8.1.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_3A-41A | 3 | 0 |
|  | 41 | 0.5 |

For this combination, MSD requirement of Band 3 and B41 DL should be specified as in table 7.2.8.1.3-3.

Table 7.2.8.1.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-41A | 3 |  |  | -94 | -91 | -89.2 | -87.9 | FDD |
|  | 41 |  |  | -94.2 | -91.2 | -89.7 | -88.6 |  |

<< End of changed sections >>