3GPP TSG-RAN Meeting #76 	R4-155098

# Beijing, China 24-28 August 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3157 | rev | 1 | Current version: | 10.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release 10 CR to align NS_04 values to meet FCC OOBE requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11, LTE_TDD_2600_US-Core |  |  |  |  |  |  | Date: |  | 2015-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Align NS_04 values to meet revised FCC OOBE requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to NS_04 OOBE requirements and AMPR values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced UE performance when UE’s operate in the US market |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.24, 6.6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR … CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start Change>

### 6.2.4	UE maximum output power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 the specific requirements and identified subclauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 to 6.2.4-6 are in addition to the allowed MPR requirements specified in subclause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5, 10, 15, 20 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2 | 41 |  |  |  |
|  |  |  | 5,10, 15, 20 | Table 6.2.4-4 |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 | ≤ 1 |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 | Table 6.2.4-2 |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4-3 | Table 6.2.4-3 |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 23 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-5 | Table 6.2.4-5 |
| .. |  |  |  |  |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4-6 | Table 6.2.4-6 |
| NS_22 | 6.6.3.3.15 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4-7 |  |
| NS_23 | 6.6.3.3.16 | 42, 43 | 5, 10, 15, 20 | N/A |  |
| .. |  |  |  |  |  |
| NS_32 | - | - | - | - | - |

<End Change>

<Start Change>

Table 6.2.4-4: A-MPR requirements for "NS_04"

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc[MHz] | ≤2499.5 |  |  |  |  |  | >2499.5 |
|  | RBstart | ≤8 |  |  |  | >8 |  | 0-24 |
|  | LCRB [RBs] | >0 |  |  |  | >0 |  | >0 |
|  | A-MPR [dB] | ≤2 |  |  |  | 0 |  | 0 |
| 10 | Fc[MHz] | ≤2504 |  |  |  |  |  | >2504 |
|  | RBstart | 0-8 |  |  |  | 9-35 | 36-49 | 0-49 |
|  | LCRB[RB] | ≤15 | >15 and <25 |  | ≥25 | N/A | >0 | >0 |
|  | RBstart + LCRB [RBs] | N/A | N/A |  | N/A | ≥45 | N/A 3 | N/A |
|  | A-MPR [dB] | ≤3 | ≤1 |  | ≤2 | ≤1 | 0 | 0 |
| 15 | Fc[MHz] | ≤2510.8 |  |  |  |  |  | >2510.8 |
|  | RBstart | 0-13 |  |  |  | 14 – 59 | 60 – 74 | 0-74 |
|  | LCRB[RB] | ≤18 or ≥36 |  | >18 and <36 |  | N/A | >0 | >0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥62 | N/A 3 | N/A |
|  | A-MPR [dB] | ≤3 |  | ≤1 |  | ≤1 | 0 | 0 |
| 20 | Fc[MHz] | ≤2517.5 |  |  |  |  |  | >2517.5 |
|  | RBstart | 0 – 22 |  |  |  | 23 – 76 | 77 – 99 | 0-99 |
|  | LCRB[RB] | ≤18 or ≥40 |  | >18 and <40 |  | N/A | >0 | >0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥86 | N/A 3 | N/A |
|  | A-MPR [dB] | ≤3 |  | ≤1 |  | ≤1 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: Refers to any RB allocation that starts in Region A or C is allowed the specified A-MPR NOTE 4: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 5: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |  |

<End Change>

<Start Change>

##### 6.6.2.2.2	Minimum requirement (network signalled value "NS_04")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.2-1.

Table 6.6.2.2.2-1: Additional requirements

|  |  |  |  |  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 |  | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 |  | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 |  | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 |  | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 |  | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 |  | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 |  | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  |  | -25 | 1 MHz |

Note:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth

<End Change>