3GPP TSG-RAN WG4 Meeting #76 	R4-155311

Beijing, China 24 – 28 August, 2015

Source:	Dish Network, Ericsson, Nokia Networks, LG Electronics, SK Telecom, KT, LG Uplus

Title:	Way forward on 2GHz harmonisation

Work item	FS_LTE_1980_2170_Korea & LTE_1980_2170_REG1

Agenda item:	8.1 & 9.1

Document for:	Approval

Background

Currently there are two tracks being considered for the 1980-2010MHz paired with 2170-2200MHz band (B65)

- LTE_1980_2170_REG1 which is Release 13 work item [1] with a completion date of Dec 2015

- FS_LTE_1980_2170_Korea which is a Region 3 Study item [2] with a completion date of xxxx

In [R4-152491] the following way forward was agreed;

- The agenda item for LTE_1980_2170_REG1 work item should only include Region 1 issues, particularly co-existence with 3GPP services in the adjacent bands. This work would be captured in TR36.872

- The agenda item for FS_LTE_1980_2170_Korea Study item should be used to cover Region 3 issues and address only co-existence below the operating band (i.e. PHS/B39) and above the operating band (i.e. B34). Any agreement or proposal for Region 3 should be maintained in the existing study item report TR36.861

2	 Way forward

It is important to structure the work so the objective of Region 1 and Region 3 can be realized in a timely manner and taking into account the possibilities of a new work item to include the delta requirements for Region 3. Since there are three meeting dates prior to closure of Release 13 as it may be useful to structure the work to avoid duplication

Therefore it is proposed the work is completed as two parts.

Part 1 (Region 1 focus)

- Agree the CR(s) to add the new band for Region 1 for all applicable specifications for all the working groups.

- Current outstanding issues are Rx 	sensitivity and Region 1 co-existence for Region 1

- Framework CR can be added as a TP to TR36.872 as an Annex and submitted as a standalone CR at RAN#70

Part 2 (Region 3 focus)

- Additional  delta CR to address co-existence with PHS, Band 34 and Band 39 in Band 65

- To include additional delta requirements for Band 65, RAN4 can open the new WI for MSS band in Region3 when SI of MSS band in Region3 is closed.

- Therefore, UE coexistence requirements for B65 UE to protect Band 34, Band 39 and PHS band should be completed at RAN #70.

- If some open issues are remained at the target completion date, the open issues will be treated as TEI.

3 	Summary

This document proposed the following objectives in a phased approach;

Objective 1 	Draft CR can be added used as baseline during RAN4#76bis discussions for REG3

Objective 2	Complete the additional work to address co-existence with PHS, Band B34 and Band 39 for Region 3 as part of the SI: FS_LTE_1980_2170_Korea and create an additional delta CR to add these requirements to the Band 65 REG1 specification.