3GPP TSG-RAN WG4 Meeting #76bis	R4-155951

Sophia Antipolis, France, 12 – 16 Oct, 2015

Source:	Nokia Networks

Title:	Discussion on DMRS sequence for UL IRC

Agenda item:	7.6.3

Document for:	Discussion

### 1	Introduction

The progress of the UL IRC WI [1] work has reached the stage to define and specify detailed assumptions [2] for simulation work.  The DMRS sequences for serving and interfering cells have not been defined in the simulation assumption [2].  Although the choice of DMRS might not have significant impact on MMSE-IRC performance, it is an important parameter for conformance test of BS MMSE-IRC.

In the last RAN4 meeting, many agreements have been reached in the ad-hoc meeting [3]. On DMRS, it is agreed that

- DMRS bases sequence:

  - Use the different base sequences for serving UE and interference UEs.

In this paper, we provide some analysis on the background of UL DMRS design.  Our views on the choice of UL DMRS sequences are provided for MMSE-IRC tests for UE in serving and interfering cells.

### 2	UL demodulation reference sequence (UL DMRS)

The LTE’s UL DMRS (demodulation reference symbol) sequence is based on cyclic extension of Zadoff-Chu sequences. From 36.211 [4], the definition of UL reference signal sequence is provided here (Section 5.5.1 of 36.211):

Reference signal sequence  is defined by a cyclic shift  of a base sequence  according to

where  is the length of the reference signal sequence and. Multiple reference signal sequences are defined from a single base sequence through different values of .

… the base sequence  is given by

where the  root  Zadoff-Chu sequence is defined by

when the length of the base sequence is equal to or greater than .

The base sequence is a cyclic extension of Zadoff-Chu sequence with length of NZCRS.  Each base sequence is uniquely identified by (u,v), which defines the group number u and base sequence number v (0 or 1) within the group.

There are 30 groups of base sequence in LTE, where the sequence-group number u∈{0,1,⋯,29}.  Within each group, different phase-rotated sequences are used with different cyclic shift α.

The sequence-group number u in one slot ns, as specified in 36.211 [4], is determined by the group hopping pattern fghns and sequence-shift pattern fss, as

u=fghns+fssmod 30

Only the sequence-shift pattern fss is related with cell ID, which is determined by

fssPUCCH=NIDcellmod 30

and

fssPUSCH=fssPUCCH+Δss mod 30

where Δss∈{0,1,⋯,29} is configured by higher layers.

Note that the sequence-group number u is determined by the group hopping pattern and the sequence-shift pattern.  The sequence-shift pattern fss is directly related with cell ID; and it is also configurable by network.  Therefore, the sequence-group number u is under control of eNB network.

In summary, the UL DMRS sequence is based on cyclic extension of Zadoff-Chu sequence.  Each DMRS sequence is a cyclic shift (with cyclic shift α) of a base sequence.  One DMRS base sequence belongs to one of the 30 sequence-groups. We also observe that:

Observation 1:	The sequence-group number is related to the cell-ID and it is assigned by the network.

#### 2.1	Cross-correlation property of inter-cell UL DMRS

Zadoff-Chu sequence has very good cross-correlation property.  The cross-correlation of two different base sequences typically has relatively low but still non-zero values.  However, the DMRS sequences of different cyclic shifts from the same base sequence can be completely orthogonal; meaning that the cross-correlation between two DMRS sequences is zero.  Usually there are 12 orthogonal DMRS, given that the cyclic shift α=mπ/6, for m={0,⋯, 11}.

The perfect DMRS cross-correlation indicates that there will be no DMRS interference over an AWGN or frequency non-selective channel.  For UL IRC algorithm, where channel estimation is based on the DMRS symbol, this ensures good channel estimation performance.

Since the DMRS sequence-group number u is controlled by the network, with cell planning, the network can ensure that

- The sequence-group for PUSCH (with cell-ID and sequence-shift pattern) can be kept same for neighbor cells.

- If sequence-group hopping is enabled, the cell-ID of neighbor cells can be configured so that the same group-hopping pattern is used for neighbor cells.

However, for some network scenarios, different sequence-group numbers may be applied for neighbor cells.  Different sequence-group numbers may be assigned for neighbor cells.

Proposal 1:	For homogenous network scenario, use same sequence-group number for serving and interfering cell DMRS.

Proposal 2:	For het-net scenario, use different sequence-group number for serving and interfering cell DMRS.

### 3	Conclusions

This contribution provides some analysis on the DMRS selection for MMSE-IRC.  In summary, the UL DMRS sequence is based on cyclic extension of Zadoff-Chu sequence.  Each DMRS sequence is a cyclic shift (with cyclic shift α) of a base sequence.  One DMRS base sequence belongs to one of the 30 sequence-groups. We also observe that:

Observation 1:	The sequence-group number is related to the cell-ID and it is assigned by the network.

Based on the observation, for the MMSE-IRC tests, we propose that

Proposal 1:	For homogenous network scenario, use same sequence-group number for serving and interfering cell DMRS.

Proposal 2:	For het-net scenario, use different sequence-group number for serving and interfering cell DMRS.

# Reference

[1]	RP-142223	“New WI on performance requirements of MMSE-IRC receiver for LTE BS”, China Telecom, RAN #66, Dec 2014

[2]	R4-153669	“TP on link level performance evaluation”, Huawei, China Telecom, ZTE, Samsung, Nokia networks, Alcatel-Lucent, RAN #75, May 2015

[3]	R4-155202	“Meeting minutes for BS MMSE-IRC ad hoc”, Huawei, China Telecom, RAN4 #76, Aug 2015

[4]	3GPP TS 36.211