3GPP TSG-RAN WG4 Meeting #76-BIS                                                	R4-156112

Sophia Antipolis, France, 12 – 16 Oct, 2015

Source:		Ericsson

Title:			TP on wanted emissions limits for AAS BS

Agenda item:	7.2.4

Document for:	Approval

# Introduction

This text proposal is for the 3GPP AAS TR 37.842 and is based on the contributions for discussion in R4-156353 and R4-156357 which elaborate different aspects around specifying the conducted output power requirements for an AAS BS and proposes the principles on specifying the BS rated carrier output power for an AAS BS.

<Start of text proposal>

### 8.1.4	AAS Base station output power

The objectives of defining conducted output power of the AAS BS are to verify the capabilities of the transmitter to feed antennas and the accuracy of the maximum output power.

Regulatory review on output power is presented in [2]. The following summarises regulatory views and requirements for BS employing antenna arrays.

-	No specific limits for AAS for terrestrial mobile systems are given in ITU-R Recommendations.

-	For the ECC regulation in Europe there is not output power limit specified and does not give any additional definitions in the AAS area, however there are several spurious emissions limits and Block Edge Mask requirements which impact and limit the maximum transmission power.

-	For the FCC regulation in US, it suggests the transmission limits applicable to the sum at the antenna connectors.

-	For the MIC regulation in Japan, the measurement and calculation method of output power from multiple outputs:

-	For an adaptive array antenna are given as follows: Measure and sum the output powers or emissions (which are specified by absolute values in the technical conditions) of all the antenna connectors. In this case, the output power of one array element shall be configured with the maximum output power. Then, the output powers of remaining array elements shall be configured as the sum of output powers of all array elements are maximized.

- For MIMO: Measure the output power or emission on each antenna connector.

Based on the above regulatory views and requirements, the following output power definitions are adapted from the non-AAS BS specifications for the AAS-BS with specific conditions at the transceiver array boundary.

The maximum carrier output power, PAAS_max,c of the AAS base station is the mean power level measured from a transceiver unit at the transceiver array boundary, during the transmitter ON period for a specific carrier in a specified reference condition.

The rated carrier output power, PAAS_rated,c of the AAS base station is the mean power level for a transceiver unit that the manufacturer has declared to be available at the transceiver array boundary, for a specific carrier during the transmitter ON period.

The rated carrier output power PAAS_rated,cof the AAS BS shall be as specified in Table 8.1.4-1.

Table 8.1.4-1: AAS Base Station rated output power

| AAS BS class | P Rated,c |
| --- | --- |
| Wide Area AAS BS | (note 1) |
| Medium Range AAS BS | ≤ 38 dBm + 10log(min(#activeTX, 8)) – 10log(#activeTX) |
| Local Area AAS BS | ≤ 24 dBm + 10log(min(#activeTX, 8)) – 10log(#activeTX) |
| NOTE 1: There is no upper limit for the rated output power or the total rated out power of the Wide Area AAS Base Station. |  |

The output power requirement is set on the accuracy of the declared output power for each transceiver at the transceiver boundary.

The accuracy requirement shall be as follows:

In normal conditions, the maximum carrier output power, PAAS_max,c of the AAS base station shall remain within +2 dB and -2 dB of the rated carrier output power, PAAS_rated,c declared by the manufacturer.

In extreme conditions, the maximum carrier output power, PAAS_max,c of the AAS base station shall remain within +2.5 dB and -2.5 dB of the rated carrier output power, PAAS_rated,c declared by the manufacturer.

Additional regional requirement may be specified if applicable.

<End of text proposal>