3GPP TSG-RAN WG4 #76bis	R4-156410

Sophia-Antipolis, France, 12th Oct– 16th Oct 2015

Source: 	Ericsson

Title:	Discussion on the open issues for D2D demodulation

Agenda Item:	6.1.2

Document for:	Discussion

# Introduction

In RAN4#76 meeting, the group had some extensive discussion on how to setup power imbalance test. In [1], the parameters for the imbalance test were proposed. In this contribution, we provide our view on how to setup the key parameters for the power imbalance test.

# Discussion on the power imbalance test

In RAN4#76 meeting, for the power imbalance test, we have the following agreements:

- -21dBC is assumed for in-channel selective assumption

On how to compute the SNRs/power imbalance test between the two links, the following steps are agreed:

- Select SINR2 from simulation results for decoding SNR@70% throughput point.

- Select SNR2 such that SNR2 >> SINR2 (e.g., 5dB higher)

- Compute SNR1 from the relation: SINR2 = SNR2 – 10*log10(10^((SNR1 + ICS)/10)+1).

- Tentative SNR2=[5] dB

The relationship of SINR2, SNR2, and SNR1 are shown in Figure 1.

Figure 1: The relationship of SINR2, SNR2 and SNR1

Link level simulation results for Discovery are shown in Figure 2, and the link level results for the communication are shown in Figure 3 and Figure 4. The simulation assumptions are shown in Appendix A.

Figure 2: Link level simulation results for power imbalance test (Discovery)

Figure 3: Link level simulation results for power imbalance test (5MHz) (Communication)

Figure 4: Link level simulation results for power imbalance test (10MHz) (Communication)

The required SNRs for discovery and communication are shown in Table 1.

Table 1: Required SNR for discovery and communication

|  |  | Require SINR2 (dB) @ BLER=30% | SNR1 (dB) |
| --- | --- | --- | --- |
| Discovery |  | -0.74 | 25.4 |
| Communication | 5MHz | -5.5 | 31.1 |
|  | 10MHz | -6 | 31.6 |

In RAN4#76 discussion, some companies mentioned the upper limit for the SNR for demodulation. For example, in traditional demodulation test, too high SNR is generally not used. The reason behind is the EVM. In general, for RX EVM, 6% is used. The upper bound of SINR is about 24.4 dB. High SNR does not bring too much benefit for the SINR improvements. In the power imbalance test case, higher SNR level does not bring any problems since 24.4 dB SINR is good enough for all the channels detection.

Proposal: Taking Table 1 into account for the SNR1 setup in the power imbalance test.

# Conclusion

In this contribution, we make our analysis on the power imbalance test, we have the following proposals:

Proposal 1:  Taking Table 1 into account for the SNR1 setup in the power imbalance test.

# Appendix A: Simulation assumption

In Table 2, parameters for the power imbalance test are given according to [1].

Table 2: Parameters for the power imbalance test

| Test / Simulation Parameters |  | Test 1 (Discovery) | Test 2 (Communication) |
| --- | --- | --- | --- |
| # E-UTRA cells |  | 1 (serving cell) |  |
| # D2D UEs |  | 2 | 2 |
| D2D UE 1 | Transmission | PSDCH (0 HARQ) | PSCCH/PSSCH |
|  | PSSCH RMC | N/A | QPSK, TCR 2/3 |
|  | Timing offset w.r.t. serving cell | 0 us | 0 us |
|  | Freq offset w.r.t. serving cell | 0 Hz | 0 Hz |
|  | Rx power level | D2D UE2 + [19dB] | D2D UE2 + [19dB] |
|  | RB allocation (PSDCH/PSSCH) | 2RB[4,5] | 2RB[4,5] |
| D2D UE 2 | Transmission | PSDCH (0 HARQ) | PSCCH/PSSCH |
|  | PSSCH RMC | N/A | QPSK, TCR 2/3, 2RBs |
|  | Timing offset w.r.t. serving cell | 0 us | 0 us |
|  | Freq offset w.r.t. serving cell | 0 Hz | 0 Hz |
|  | Rx power level | - | - |
|  | RB allocation (PSDCH/PSSCH) | adjacent 2 RB of UE1[6,7] | adjacent 2 RB of UE1[6,7] |
| Leakage model from UE1 Link to UE 2 Link at Rx UE due to ICS for simulations only |  | Option 1 : -19dBc leakage into adjacent 2 RBs, operating SIR is 0dB for UE2 Option 2 : -22dBc leakage into adjacent 2 RBs, operating SIR is 3dB for UE2 Other options are not precluded (Leakage model over entire BW is not needed. This is for simulation purpose to derive the Link 2 operating SIR) |  |
| Propagation Channel | Serving cell | AWGN |  |
|  | D2D UE 1 |  |  |
|  | D2D UE 2 |  |  |
| Noc |  | -98dBm/15kHz |  |
| Channel BWs |  | 5MHz |  |
| Simulation results needed |  | PSDCH BLER for UE 2 link | PSSCH BLER for UE 2 link |

# Reference

- R4-153676, WF on D2D Demodulation with Multiple sidelinks, LG Electronics, Huawei, HiSilicon, Qualcomm, Intel, Samsung, May 2015

- 36.104, Base Station (BS) radio transmission and reception, 3GPP