3GPP TSG-RAN WG4 Meeting #77	R4-156999

Anaheim, CA, United States, 16th November – 20th November, 2015

Agenda Item:	9.5.1

Source: 	Intel Corporation

Title: 	Discussion on UE demodulation performances for high speed train scenario with SFN deployment

Document for:	Discussion

- Introduction

The study item “Study on performance enhancements for high speed scenario in LTE” was approved at RAN plenary meeting #66 [1]. One objective of this SI is to identify new conditions that affect the system performance, including degradation of UE demodulation performance, for the new high speed train (HST) scenarios [2]. A common high speed train scenario is SFN deployment with RRHs along high speed train railways [2, 3], and a specified channel model for HST was agreed as in [3].

In this contribution, we provide the evaluation of UE demodulation performance under HST SFN channel assumptions as in [3], compared with that under the Extended Vehicular A (EVA) channel model.

- Simulation assumptions and results

In this contribution, the HST channel is modelled as an SFN with bi-directional RRHs reception, which is detailed in [3]. The simulation parameters for Transmission Mode 1 (TM 1) are summarized in Table I.

| Parameter | Value | Unit |
| --- | --- | --- |
| Bandwidth | 10 | MHz |
| Duplex mode | FDD |  |
| MCS | 19 |  |
| Antenna configuration | 1×2 |  |
| Transmission mode | 1 |  |
| Reference receiver | MMSE |  |
| Ds (RRH to RRH distance) | 2000 | m |
| Dmin (RRH to railway distance) | 50 | m |
| v (train speed) | 350 | km/h |
| fd (max.Doppler shift) | 872 | Hz |

Table I. Simulation parameters for the HST channel model (TM 1)

Figure 1 plots the simulation results for legacy UE’s normalized throughput in the HST-SFN channel relative to the throughput in the EVA channel with 5Hz maximum Doppler shift.

Fig. 1 Simulation results for legacy UE’s normalized throughput in the HST-SFN channel relative to the throughput in the EVA channel with 5Hz maximum Doppler shift (TM 1).

Table II lists the simulation parameters for a TM3 transmission.

| Parameter | Value | Unit |
| --- | --- | --- |
| Bandwidth | 10 | MHz |
| Duplex mode | FDD |  |
| MCS | 19 |  |
| Antenna configuration | 2×2 |  |
| Transmission mode | 3 |  |
| Reference receiver | MMSE |  |
| Ds (RRH to RRH distance) | 2000 | m |
| Dmin (RRH to railway distance) | 50 | m |
| v (train speed) | 350 | km/h |
| fd (max.Doppler shift) | 872 | Hz |

Table II. Simulation parameters for the HST channel model (TM 3)

Figure 2 plots the simulation results for legacy UE’s normalized throughput in the HST-SFN channel relative to the throughput in the EVA channel with 70Hz maximum Doppler shift.

Fig. 2 Simulation results for legacy UE’s normalized throughput in the HST-SFN channel relative to the throughput in the EVA channel with 70Hz maximum Doppler shift (TM 3).

Observation: Figs. 1 and 2 show that the UE’s demodulation performance in the HST-SFN channel degrades significantly, which can be up to 1/3 throughput degradation, compared to that in the EVA channels.

Proposal: Enhancement algorithms at BS and/or UE side are necessary to maintain satisfactory demodulation performance for the HST scenario.

- Conclusion

In this contribution, we provide the evaluation of UE demodulation performance under HST SFN channel assumptions as in [3], compared with that under the Extended Vehicular A (EVA) channel model.

Observation: Simulation results show that the UE’s demodulation performance in the HST-SFN channel degrades significantly, which can be up to 1/3 throughput degradation, compared to that in the EVA channels.

Proposal: Enhancement algorithms at BS and/or UE side are necessary to retain satisfactory demodulation performance for the HST scenario.

- Reference

[1] RP-142307, “Study on performance enhancements for high speed scenario in LTE”, NTT DOCOMO INC., Huawei, HiSilicon.

[2] R4-150122, “New channel mode for SFN deployment”, Huawei, HiSilicon, Feb. 2015

[3] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.