3GPP TSG-RAN WG4 Meeting #77	     R4-157328

Anaheim, CA, US, 16 – 20 Nov, 2015

Agenda Item:	7.12.2

Source: 	Samsung

Title: 	LLS simulation results for RS-SINR

Document for:	Discussion

Introduction

RAN2 has discussed a new UE measurement RS-SINR for WI “Multicarrier Load Distribution of UEs in LTE”, and sent an LS to RAN4 in [1]. In RAN4 #76 meeting, way forward on RS-SINR measurement was agreed and link level simulation assumptions for RS-SINR were provided in [2].

In RAN4 #76bis meeting, further agreements on RS-SINR measurement were approved in [3]. The RS-SINR reporting range will be further dicussed in the next meeting.

In this contribution, we provide the link level simulation results for RS-SINR measurement according to the agreed simulation assumptions.

Simulation assumptions

The simulation assumptions are listed in Table 1. For the measurement sampling rate, there are 5 samples per 200 ms L1 measurement period, i.e. only 1 subframe is measured in per 40 ms interval.

Table 1: Simulation assumptions for RS-SINR measurement

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6 resource blocks, 50 resource blocks |  |
| System bandwidth | 50 resource blocks |  |
| L1 measurement period | 200 ms |  |
| Measurement sampling rate | - | Implementation dependent (NOTE 1) |
| L3 filtering | disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 2 |  |
| Propagation conditions | AWGN, ETU 30 and EPA 5 |  |
| DRX/DTX | OFF |  |
| CP Length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -8dB, -6dB, -3dB, 0dB, 3dB | AWGN noise |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results. |  |  |

Smulation results

The absolute RS-SINR measurement accuracy results are summarized in below Table 2 and Table 3 for 6 RBs and 50 RBs measurement bandwidth respectively.

The absolute RSRP and RSRQ measurement accuracy results are also summarized in Table 4~7 in Annex A for reference.

Table 2: RS-SINR measurement accuracy results (6 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | 0.75 | 2.39 | 3.83 | 3.08 | 3.83 |
|  | -6 | 0.17 | 1.63 | 2.94 | 2.77 | 2.94 |
|  | -3 | -0.04 | 1.08 | 2.11 | 2.15 | 2.11 |
|  | 0 | -0.04 | 0.81 | 1.65 | 1.70 | 1.65 |
|  | 3 | -0.04 | 0.67 | 1.41 | 1.45 | 1.41 |
| ETU30 | -8 | -0.48 | 1.33 | 3.19 | 3.67 | 3.19 |
|  | -6 | -1.08 | 0.52 | 2.11 | 3.20 | 2.11 |
|  | -3 | -1.49 | -0.18 | 1.02 | 2.51 | 1.49 |
|  | 0 | -1.70 | -0.61 | 0.41 | 2.11 | 1.70 |
|  | 3 | -2.05 | -1.05 | -0.10 | 1.95 | 2.05 |
| EPA5 | -8 | -0.42 | 1.52 | 4.13 | 4.54 | 4.13 |
|  | -6 | -0.72 | 0.88 | 3.06 | 3.78 | 3.06 |
|  | -3 | -0.88 | 0.39 | 1.90 | 2.78 | 1.90 |
|  | 0 | -0.81 | 0.21 | 1.31 | 2.12 | 1.31 |
|  | 3 | -0.72 | 0.15 | 1.02 | 1.74 | 1.02 |

Table 3: RS-SINR measurement accuracy results (50 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | -0.01 | 0.76 | 1.47 | 1.47 | 1.47 |
|  | -6 | -0.02 | 0.55 | 1.10 | 1.12 | 1.10 |
|  | -3 | -0.02 | 0.37 | 0.75 | 0.77 | 0.75 |
|  | 0 | -0.02 | 0.27 | 0.56 | 0.58 | 0.56 |
|  | 3 | -0.01 | 0.22 | 0.45 | 0.46 | 0.45 |
| ETU30 | -8 | -1.12 | -0.20 | 0.65 | 1.77 | 1.12 |
|  | -6 | -1.10 | -0.39 | 0.28 | 1.38 | 1.10 |
|  | -3 | -1.14 | -0.59 | -0.08 | 1.05 | 1.14 |
|  | 0 | -1.35 | -0.83 | -0.39 | 0.96 | 1.35 |
|  | 3 | -1.80 | -1.21 | -0.73 | 1.07 | 1.80 |
| EPA5 | -8 | -0.62 | 0.23 | 1.12 | 1.74 | 1.12 |
|  | -6 | -0.53 | 0.12 | 0.76 | 1.30 | 0.76 |
|  | -3 | -0.40 | 0.06 | 0.50 | 0.90 | 0.50 |
|  | 0 | -0.30 | 0.04 | 0.37 | 0.67 | 0.37 |
|  | 3 | -0.24 | 0.04 | 0.32 | 0.56 | 0.32 |

From the above simulation results, it can be observed:

- The RS-SINR accuracy can be improved by enlarging the measurement bandwidth.

- For SNR = -6dB side condition, the RS-SINR absolute accuracy is around 3.1dB for 6RBs measurement bandwidth, and 1.2dB for 50RBs measurement bandwidth.

- For SNR = -3dB side condition, the RS-SINR absolute accuracy is around 2.1dB for 6RBs measurement bandwidth, and 1.2dB for 50RBs measurement bandwidth.

The above observations are summarized as below:

Observation 1: RS-SINR accuracy can be improved by enlarging the measurement bandwidth

Observation 2: RS-SINR absolute accuracy requirement can be specified as ±3.5dB for Es/Ios≥-6dB, and ±2.5dB for Es/Ios≥-3dB.

Conclusion

In this contribution, we provide the link level simulation results for RS-SINR measurement accuracy for RAN2 WI “Multicarrier Load Distribution of UEs in LTE”. Based on the simulation results, below observations are summarized.

Observation 1: RS-SINR accuracy can be improved by enlarging the measurement bandwidth

Observation 2: RS-SINR absolute accuracy requirement can be specified as ±3.5dB for Es/Ios≥-6dB, and ±2.5dB for Es/Ios≥-3dB.

Reference

- R2-151785, LS on a new measurement quantity for Multicarrier Load Distribution, RAN WG2

- R4-155139, Way forward on RS-SINR measurement, CMCC, Intel, Ericsson, Nokia

- R4-156636, Way forward on MC load distribution RRM issues, Ericsson, CMCC, Nokia Nerworks, ZTE

Annex A: RSRP and RSRQ measurement results

Table 4: RSRP measurement accuracy results (6 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | 0.57 | 1.97 | 3.16 | 2.59 | 3.16 |
|  | -6 | 0.04 | 1.28 | 2.32 | 2.28 | 2.32 |
|  | -3 | -0.09 | 0.76 | 1.52 | 1.61 | 1.52 |
|  | 0 | -0.07 | 0.49 | 1.03 | 1.10 | 1.03 |
|  | 3 | -0.04 | 0.34 | 0.72 | 0.76 | 0.72 |
| ETU30 | -8 | -0.64 | 0.97 | 2.61 | 3.25 | 2.61 |
|  | -6 | -1.08 | 0.31 | 1.64 | 2.72 | 1.64 |
|  | -3 | -1.24 | -0.17 | 0.75 | 1.99 | 1.24 |
|  | 0 | -1.13 | -0.34 | 0.32 | 1.45 | 1.13 |
|  | 3 | -1.00 | -0.39 | 0.09 | 1.10 | 1.00 |
| EPA5 | -8 | -0.51 | 1.13 | 3.53 | 4.05 | 3.53 |
|  | -6 | -0.75 | 0.59 | 2.49 | 3.24 | 2.49 |
|  | -3 | -0.78 | 0.19 | 1.41 | 2.20 | 1.41 |
|  | 0 | -0.67 | 0.05 | 0.82 | 1.49 | 0.82 |
|  | 3 | -0.49 | 0.01 | 0.52 | 1.01 | 0.52 |

Table 5: RSRQ measurement accuracy results (6 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | 0.62 | 2.01 | 3.17 | 2.55 | 3.17 |
|  | -6 | 0.12 | 1.34 | 2.35 | 2.23 | 2.35 |
|  | -3 | 0.01 | 0.87 | 1.61 | 1.60 | 1.61 |
|  | 0 | 0.11 | 0.66 | 1.18 | 1.07 | 1.18 |
|  | 3 | 0.20 | 0.57 | 0.94 | 0.74 | 0.94 |
| ETU30 | -8 | -0.42 | 1.14 | 2.75 | 3.17 | 2.75 |
|  | -6 | -0.84 | 0.50 | 1.79 | 2.63 | 1.79 |
|  | -3 | -0.94 | 0.06 | 0.94 | 1.89 | 0.94 |
|  | 0 | -0.82 | -0.06 | 0.56 | 1.39 | 0.82 |
|  | 3 | -0.65 | -0.05 | 0.39 | 1.04 | 0.65 |
| EPA5 | -8 | -0.31 | 1.30 | 3.66 | 3.97 | 3.66 |
|  | -6 | -0.51 | 0.77 | 2.62 | 3.13 | 2.62 |
|  | -3 | -0.53 | 0.41 | 1.58 | 2.12 | 1.58 |
|  | 0 | -0.36 | 0.32 | 1.05 | 1.41 | 1.05 |
|  | 3 | -0.16 | 0.33 | 0.78 | 0.94 | 0.78 |

Table 6: RSRP measurement accuracy results (50 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | -0.02 | 0.66 | 1.28 | 1.30 | 1.28 |
|  | -6 | -0.04 | 0.46 | 0.92 | 0.96 | 0.92 |
|  | -3 | -0.03 | 0.28 | 0.58 | 0.61 | 0.58 |
|  | 0 | -0.02 | 0.19 | 0.38 | 0.41 | 0.38 |
|  | 3 | -0.02 | 0.13 | 0.26 | 0.28 | 0.26 |
| ETU30 | -8 | -1.04 | -0.20 | 0.55 | 1.59 | 1.04 |
|  | -6 | -0.94 | -0.32 | 0.25 | 1.18 | 0.94 |
|  | -3 | -0.82 | -0.39 | -0.01 | 0.81 | 0.82 |
|  | 0 | -0.74 | -0.41 | -0.13 | 0.61 | 0.74 |
|  | 3 | -0.71 | -0.42 | -0.19 | 0.52 | 0.71 |
| EPA5 | -8 | -0.58 | 0.16 | 0.96 | 1.54 | 0.96 |
|  | -6 | -0.49 | 0.08 | 0.64 | 1.13 | 0.64 |
|  | -3 | -0.35 | 0.03 | 0.37 | 0.72 | 0.37 |
|  | 0 | -0.24 | 0.01 | 0.24 | 0.48 | 0.24 |
|  | 3 | -0.16 | 0.01 | 0.16 | 0.32 | 0.16 |

Table 7: RSRQ measurement accuracy results (50 RB)

| Channel | SNR(dB) | 5% | 50% | 95% | Δ(5,95) | Absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| AWGN | -8 | 0.03 | 0.71 | 1.32 | 1.29 | 1.32 |
|  | -6 | 0.04 | 0.53 | 0.99 | 0.95 | 0.99 |
|  | -3 | 0.09 | 0.40 | 0.69 | 0.60 | 0.69 |
|  | 0 | 0.17 | 0.36 | 0.56 | 0.39 | 0.56 |
|  | 3 | 0.24 | 0.37 | 0.50 | 0.26 | 0.50 |
| ETU30 | -8 | -0.93 | -0.11 | 0.63 | 1.56 | 0.93 |
|  | -6 | -0.82 | -0.20 | 0.34 | 1.15 | 0.82 |
|  | -3 | -0.65 | -0.23 | 0.14 | 0.78 | 0.65 |
|  | 0 | -0.51 | -0.18 | 0.08 | 0.59 | 0.51 |
|  | 3 | -0.40 | -0.13 | 0.09 | 0.49 | 0.40 |
| EPA5 | -8 | -0.47 | 0.26 | 1.04 | 1.52 | 1.04 |
|  | -6 | -0.36 | 0.20 | 0.73 | 1.09 | 0.73 |
|  | -3 | -0.18 | 0.20 | 0.52 | 0.70 | 0.52 |
|  | 0 | -0.02 | 0.24 | 0.45 | 0.46 | 0.45 |
|  | 3 | 0.11 | 0.29 | 0.43 | 0.32 | 0.43 |