3GPP TSG-RAN WG4 Meeting #77	    R4-157329

Anaheim, CA, US, 16 – 20 Nov, 2015

Agenda Item:	7.7.5

Source: 	Samsung

Title: 	Cell detection simulation results for Rel-13 MTC UE

Document for:	Discussion

Introduction

In RAN4 #76bis meeting, RAN4 has discussed the RRM part for WI “Further LTE Physical Layer Enhancements for MTC”, and the simulation assumptions for cell identification procedure of Rel-13 MTC UE under enhanced coverage were agreed in [1].

In this contribution, we provide the simulation results of cell detection for Rel-13 MTC UE according to the agreed assumptions.

Smulation Results

The detailed simulation assumptions are listed in Table A.1~4 in Annex A.

Following Table 1 ~ 5 show the simulation results of 90th percentile of cell detection delay for Rel-13 MTC UE under AWGN EPA5, ETU30, EPA1 and ETU1 repectively.

The Es/Noc is -6.45dB, -8.45dB, -10.45dB, -13.45dB and -16.45dB for Test 1~5. Then the Es/Iot is -8dB, -10dB, -13dB, -15dB and -18dB respectively.

For the sampling rate during the cell identification, the UE will have a cell detection attempt per 10ms.

Table 1: Cell detection delay under AWGN (Unit: ms)

| Test # Case # | SINR = -8dB | SINR = -10dB | SINR = -12dB | SINR = -15dB | SINR = -18dB |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 40 | 180 | 830 | ≥2000 | ≥2000 |
| Case 2 | 50 | 210 | 800 | 1060 | 1090 |
| Case 3 | 30 | 80 | 130 | 150 | 170 |
| Case 4 | 40 | 170 | 850 | ≥2000 | ≥2000 |
| Case 5 | 60 | 230 | 840 | ≥2000 | ≥2000 |
| Case 6 | 60 | 200 | 580 | 1060 | 1140 |
| Case 7 | 50 | 120 | 180 | 190 | 190 |
| Case 8 | 60 | 220 | 920 | ≥2000 | ≥2000 |

Table 2: Cell detection delay under EPA5 (Unit: ms)

| Test # Case # | SINR = -8dB | SINR = -10dB | SINR = -12dB | SINR = -15dB | SINR = -18dB |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 170 | 300 | 510 | 1130 | 1930 |
| Case 2 | 180 | 290 | 450 | 860 | 1230 |
| Case 3 | 110 | 140 | 180 | 260 | 280 |
| Case 4 | 190 | 280 | 540 | 1200 | 1830 |
| Case 5 | 170 | 260 | 480 | 1080 | 1840 |
| Case 6 | 160 | 270 | 430 | 800 | 1220 |
| Case 7 | 110 | 140 | 170 | 250 | 280 |
| Case 8 | 180 | 280 | 500 | 1010 | 1780 |

Table 3: Cell detection delay under ETU30 (Unit: ms)

| Test # Case # | SINR = -8dB | SINR = -10dB | SINR = -12dB | SINR = -15dB | SINR = -18dB |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 110 | 190 | 410 | 1190 | 1780 |
| Case 2 | 100 | 160 | 360 | 850 | 1090 |
| Case 3 | 80 | 120 | 170 | 260 | 280 |
| Case 4 | 100 | 180 | 390 | 1120 | 1630 |
| Case 5 | 100 | 180 | 400 | 1110 | 1740 |
| Case 6 | 100 | 170 | 370 | 820 | 1140 |
| Case 7 | 80 | 110 | 170 | 270 | 270 |
| Case 8 | 100 | 190 | 390 | 1170 | 1640 |

Table 4: Cell detection delay under EPA1 (Unit: ms)

| Test # Case # | SINR = -8dB | SINR = -10dB | SINR = -12dB | SINR = -15dB | SINR = -18dB |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 510 | 710 | 990 | 1600 | 1850 |
| Case 2 | 390 | 570 | 790 | 1150 | 1280 |
| Case 3 | 200 | 260 | 300 | 400 | 460 |
| Case 4 | 500 | 670 | 970 | 1520 | ≥2000 |
| Case 5 | 420 | 630 | 940 | 1520 | ≥2000 |
| Case 6 | 380 | 510 | 640 | 1040 | 1300 |
| Case 7 | 170 | 240 | 270 | 400 | 410 |
| Case 8 | 430 | 630 | 990 | 1390 | 1760 |

Table 5: Cell detection delay under ETU1 (Unit: ms)

| Test # Case # | SINR = -8dB | SINR = -10dB | SINR = -12dB | SINR = -15dB | SINR = -18dB |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 520 | 750 | 1020 | 1620 | 1890 |
| Case 2 | 420 | 610 | 810 | 1130 | 1240 |
| Case 3 | 260 | 310 | 370 | 450 | 500 |
| Case 4 | 530 | 720 | 1100 | 1580 | 1850 |
| Case 5 | 470 | 690 | 940 | 1610 | 1740 |
| Case 6 | 360 | 530 | 790 | 1010 | 1270 |
| Case 7 | 220 | 270 | 340 | 480 | 510 |
| Case 8 | 470 | 760 | 1010 | 1470 | 1850 |

Based on the above simulation results, it can be observed:

- For AWGN channel, the Rel-13 MTC UE can meet the legacy requirement 600ms for SINR ≥ -10dB.

- For EPA5 and ETU30 channel, the Rel-13 MTC UE can meet the 600ms requirement when SINR ≥ -12dB.

- For EPA1 and ETU1 channel, the Rel-13 MTC UE can meet the 600ms requirement when SINR ≥ -8dB.

The above observations are summarized as:

Observation 1: Rel-13 MTC UE can meet the legacy cell detection requirement 600ms when SINR ≥ -8dB.

Conclusion

In this contribution, we provide the simulation results of cell identification performance for Rel-13 MTC UE. Based on the simulation results, below observation is summarized.

Observation 1: Rel-13 MTC UE can meet the legacy cell detection requirement 600ms when SINR ≥ -8dB.

Reference

- R4-156656, Simulation assumptions for cell identification for Rel-13 MTC under enhanced coverage, Nokia Networks, Ericsson

Annex A: Simulation Assumptions

Table A.1: Link Simulation Parameters for cell identification

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Carrier frequency | GHz | 2 GHz | 2 GHz | 2 GHz |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| PSS and SSS PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Number of RB’s |  | 6 | 6 | 6 |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | 1 (FDD) | 1 (FDD) | 1 (FDD) |
| CP Length | - | Normal | Normal | Normal |
| DRX |  | OFF | OFF | OFF |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| 1) Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 1.5 | 3.0 |
| Es/Noc | dB | -4.82 | -9.71 | Test 1: -6.45 Test 2: -8.45 Test 3: -10.45 Test 4: -13.45 Test 5: -16.45 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| PSS Sequence ID | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| SSS Sequence ID | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| Propagation Condition | - | AWGN, EPA5, ETU30, EPA1, ETU1 |  |  |
| Noise Model | - | AWGN |  |  |
| NOTE : Noc value doesn’t include the three simulated eNB signals’ power |  |  |  |  |

Table A.2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| Cell 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence ID’s. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Number of receive antennas | 1 |

Table A.3: Cell ID Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 5 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 6 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 7 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 8 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |

Table A.4: PSC, SSC indices for simulations

| Label | Code index | Cell group index |
| --- | --- | --- |
| psc1 | 29 | - |
| psc2 | 25 | - |
| psc3 | 34 | - |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |