3GPP TSG-RAN WG4 Meeting #77	    R4-157330

Anaheim, CA, US, 16 – 20 Nov, 2015

Agenda Item:	7.13.4

Source: 	Samsung

Title: 	Simulation results for cell detection with LAA

Document for:	Discussion

Introduction

In RAN4 #76bis meeting, RAN4 has discussed the RRM part for WI “Licensed-Assisted Access to Unlicensed Spectrum”, and the simulation assumptions for cell identification with LAA were agreed in [1].

In this contribution, we provide the simulation results of cell detection performance for LAA according to the agreed assumptions.

Smulation Results

The detailed simulation assumptions are listed in Table A.1~4 in Annex A.

Following Table 1 ~ 3 show the simulation results of 90th percentile of cell detection delay for LAA under AWGN EPA5, and ETU30 repectively.

Table 1: Cell detection delay under AWGN (Unit: ms)

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 160 | 160 | 160 | 160 | 160 | 160 |
| Case 2 | 160 | 160 | 160 | 160 | 160 | 160 |
| Case 3 | 160 | 160 | 160 | 160 | 160 | 160 |
| Case 4 | 160 | 160 | 160 | 160 | 160 | 160 |

Table 2: Cell detection delay under EPA5 (Unit: ms)

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 640 | 640 | 480 | 480 | 320 | 320 |
| Case 2 | 800 | 640 | 480 | 480 | 480 | 320 |
| Case 3 | 640 | 480 | 480 | 480 | 320 | 320 |
| Case 4 | 800 | 640 | 480 | 480 | 480 | 320 |

Table 3: Cell detection delay under ETU30 (Unit: ms)

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 960 | 800 | 640 | 480 | 480 | 320 |
| Case 2 | 800 | 640 | 640 | 480 | 480 | 320 |
| Case 3 | 800 | 640 | 480 | 480 | 320 | 320 |
| Case 4 | 960 | 640 | 640 | 480 | 480 | 320 |

Following Table 4 ~ 6 show the simulation results of cell identification probability for a given number of 1ms DRS occasions under AWGN EPA5, and ETU30 repectively.

Table 4: Successful identification probability for 1ms DRS occasion under AWGN

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 97% | 100% | 100% | 100% | 100% | 100% |
| Case 2 | 95% | 100% | 100% | 100% | 100% | 100% |
| Case 3 | 99% | 100% | 100% | 100% | 100% | 100% |
| Case 4 | 97% | 100% | 100% | 100% | 100% | 100% |

Table 5: Successful identification probability for 1ms DRS occasion under EPA5

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 44% | 51% | 59% | 62% | 70% | 76% |
| Case 2 | 42% | 51% | 59% | 66% | 68% | 74% |
| Case 3 | 48% | 58% | 59% | 67% | 73% | 76% |
| Case 4 | 42% | 52% | 58% | 63% | 69% | 73% |

Table 6: Successful identification probability for 1ms DRS occasion under ETU30

| SINR Case # | SINR = -6dB | SINR = -5dB | SINR = -4dB | SINR = -3dB | SINR = -2dB | SINR = -1dB |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 37% | 42% | 51% | 57% | 65% | 73% |
| Case 2 | 39% | 44% | 53% | 60% | 66% | 76% |
| Case 3 | 40% | 47% | 57% | 59% | 69% | 76% |
| Case 4 | 35% | 43% | 51% | 61% | 67% | 74% |

Based on the above simulation results, it can be observed:

- Since a DRS occasion is transmitted every 160 ms, for meeting legacy requirement 600ms, the UE need to detect the cell with LAA in 3 DRS occasions.

- For AWGN channel, the cell detection delay can meet the 600ms requirement when SINR ≥ -6dB.

- For EPA5 channel, the cell detection delay can meet the 600ms requirement when SINR ≥ -4dB.

- For ETU30 channel, the cell detection delay can meet the 600ms requirement when SINR ≥ -3dB.

- When the successful identification probability for 1ms DRS occasion is larger than round 55%, the cell detection delay for LAA can meet the 600ms requirement.

The above observations are summarized as:

Observation 1: Cell detection delay for LAA with 160ms DRS occasion interval can meet the 600ms legacy requirement when SINR ≥ -3dB.

Observation 2: To meet the 600ms requirement, the successful identification probability for a DRS occasion should be larger than 55%.

Conclusion

In this contribution, we provide the simulation results of cell detection performance for LAA. Based on the simulation results, below observations are summarized.

Observation 1: Cell detection delay for LAA with 160ms DRS occasion interval can meet the 600ms legacy requirement when SINR ≥ -3dB.

Observation 2: To meet the 600ms requirement, the successful identification probability for a DRS occasion should be larger than 55%.

Reference

- R4-156644, Simulation assumptions for cell identification with LAA, Ericsson

Annex A: Simulation Assumptions

Table A.1: Simulation parameters for cell identification on unlicensed carrier

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Cell Identifier | - | known to UE | known to UE | unknown to UE, to be identified |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| PSS and SSS PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Carrier frequency | GHz | 5 | 5 | 5 |
| PSS/SSS transmission bandwidth | Number of RB’s | 6 | 6 | 6 |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| Relative Delay of 1st Path (synchronous) | ms | 0 | 0 | CP/2 |
| Es/Noc | dB | -0.25 dB | 1 dB | {-1,0,1,2,3,4} dB (Es/Iot= {-6,-5,-4,-3,-2,-1} dB) |
| PSS Sequence ID | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| SSS Sequence ID | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| PSS/SSS configuration |  | Rel-12 PSS/SSS configuration |  |  |
| Propagation Condition | - | AWGN, EPA5, ETU30 |  |  |
| Noc Model | - | AWGN |  |  |
| DRS occasion periodicity (M) | ms | A DRS occasion is transmitted every 160 ms |  |  |
| DRS occasion duration (N) | - | A DRS occasion for a cell comprises 1 subframe (N=1). |  |  |
| DRS availability | - | DRS is present in all DRS occasions |  |  |
| L1 measurement period | - | i*160 ms, where - i=1 (single shot) - i=2 and i=3 (multi-shot) |  |  |
| NOTE : Noc value doesn’t include the three simulated eNB signals’ power |  |  |  |  |

Table A.2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or asynchronous (by signaling) | No |
| DRX | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence id. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| UE receive antennas | 2 (uncorrelated) |

Table A.3: Cell ID Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |

Table A.4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |