3GPP TSG-RAN WG4 Meeting #77	 R4-157340

# Anaheim, CA,US, 16- 20 Nov, 2015

Agenda Item:	7.5.1

Source: 	Samsung

Title: 	Simulation results for non-TM10 mode

Document for:	Discussion

1	Introduction

A new work item “CRS Interference Mitigation for LTE Homogenous Deployments” was approved in RAN#66. As described in WI scope, DM-RS based transmission modes (including TM9 and TM10) should be covered on this WI.

In last RAN4 meeting, such test cases were agreed for non-TM10 mode:

- The test cases defined for gain test are defined for the following transmission modes:

| Gain test | TM4 |
| --- | --- |
|  | TM9 |

- In the gain tests, the MCSs can follow the following table as baseline just for simulation for information next meeting and other options are not precluded.

|  | MCS |
| --- | --- |
| TM4 | MCS= Option1:[16] Option 2: [18] Option3: [14] |
| TM9 | MCS=[14] |

In this contribution, we provide simulation results for CRS-IM demodulation test cases.

- Analysis

TM4

Regarding MCS levels, MCS 14,16and 18 was proposed to further evaluate and down select. Simulation results for TM4 with different MCS levels were shown in figure 1~3.

Figure1: Throughput vs. SNR for TM4 MCS14

Figure2: Throughput vs. SNR for TM4 MCS16

Figure3: Relative Throughput vs. SNR for TM4 MCS18

The reference SNR points for 70% relative throughput were summarized in table 1 below:

Table 1: The target SNR@70% throughput and CRS-IC gain

| TM | MCS | 2Cell-IC | 1 cell-IC | w/o CRS-IC |
| --- | --- | --- | --- | --- |
| TM4 | 14 | 6.1 | 6.7 | 9.2 |
|  | 16 | 7.7 | 8.4 | 10.9 |
|  | 18 | 9.1 | 9.7 | 11.9 |

Based on above simulation results, for MCS down selection, we prefer MSC16 considering the operating point (with 2.5dB around IM margin) is more close to system evaluation output.

Proposal1: Choosing MCS 16 for TM4 Test.

TM9

Figure4: Throughput vs. SNR for TM9 with MCS 14

Table 2: The target SNR@70% throughput and CRS-IC gain

| TM | MCS | 2 cell CRS-IC | 1 cell CRS-IC | w/o CRS-IC |
| --- | --- | --- | --- | --- |
| TM9 | 14 | 8.4 | 9.1 | 11.6 |

- Conclusion

In this contribution, we provide simulation results for non-TM10 mode, below proposal was given:

Proposal1: Choosing MCS 16 for TM4 test.