3GPP TSG-RAN WG4 Meeting #77                                                                       R4-157371

# Anaheim, CA, US, 16 – 20 Nov., 2015

Source:	LG Electronics

Title:	Simulation results of CRS based RSRP with LAA

Agenda item:	7.13.4

Document for:	Discussion

Introduction

In this paper, we present the simulation results for CRS based RSRP and RSRQ measurement accuracy under LAA with agreed simulation assumptions[1]. Based on the simulation results, we provide our view on CRS based RSRP and RSRQ measurement accuracy.

Simulation assumption

The link level simulation assumptions for RSRP and RSRQ are shown in Table 1.

Table 1: Simulation parameters for measurement in LAA

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6, 25,50RBs | RSRP measured over 6, 25,50 RBs |
| System bandwidth | 50 resource blocks |  |
| DMTC (Discovery signal Measurement Timing Configuration) | 160ms |  |
| DRS occasion duration | 12 symbols | The signals comprising the LAA DRS are the same as symbols 0-11 of the Rel-12 DRS for FDD (LAA assumes normal CP) - DRS occasion duration is 12 symbols [1] |
| DRS availability | - | DRS is present in all DRS occasions |
| L1 measurement | i*160 ms, where - i=1 (single shot) - i=2 and i=3 (multi-shot) |  |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 and 2 | The number of transmit antennas in DRS and outside DRS is the same. |
| Receive antennas | 2 | The receive diversity rule as defined in TS 36.214. Both antennas with equal gain, no correlation between them. |
| DRX/DTX | OFF | DRX/DTX to be considered at later stage |
| Propagation conditions | Baseline: AWGN, EPA5, ETU30 |  |
| CP Length | Normal |  |
| Frequency band | 5GHz |  |
| Ês/Iot | -6,-5,-4, -3,-2,-1 dB |  |

Performance Metrics

The CDF curves are to be provided for:

- RSRP measurement accuracy : Delta RSRP = (estimated RSRP – ideal RSRP)

- RSRQ measurement accuracy: Delta RSRQ = (estimated RSRQ – ideal RSRQ)

Simulation results

RSRP

Table 3.1-1, Table3.1-2 and Table 3.1-3 show delta CRS-RSRP in the measurement bandwidth of 6RB, 25RB  and 50RB  respectively.

# Table 3.1-1. CRS based RSRP measurement accuracy with measurement bandwidth of 6RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.66 | -0.55 | 1.80 | -1.21 | 1.14 | 0.21 | -1.31 | 1.95 | -1.52 | 1.74 | -0.78 | -3.40 | 1.36 | -2.61 | 2.14 |
|  | -5 | 0.58 | -0.53 | 1.62 | -1.11 | 1.04 | 0.20 | -1.18 | 1.68 | -1.38 | 1.48 | -0.81 | -3.42 | 1.27 | -2.61 | 2.08 |
|  | -4 | 0.51 | -0.47 | 1.48 | -0.98 | 0.97 | 0.16 | -1.07 | 1.56 | -1.23 | 1.39 | -0.87 | -3.42 | 1.18 | -2.55 | 2.04 |
|  | -3 | 0.47 | -0.43 | 1.31 | -0.90 | 0.84 | 0.13 | -1.01 | 1.32 | -1.14 | 1.19 | -0.89 | -3.35 | 1.12 | -2.46 | 2.01 |
|  | -2 | 0.43 | -0.33 | 1.19 | -0.76 | 0.77 | 0.10 | -0.94 | 1.25 | -1.04 | 1.16 | -0.91 | -3.34 | 1.05 | -2.42 | 1.96 |
|  | -1 | 0.39 | -0.30 | 1.08 | -0.68 | 0.69 | 0.09 | -0.89 | 1.13 | -0.98 | 1.04 | -0.94 | -3.44 | 0.89 | -2.50 | 1.84 |
| 2 | -6 | 0.55 | -0.32 | 1.38 | -0.87 | 0.83 | 0.21 | -0.80 | 1.33 | -1.01 | 1.12 | -0.77 | -2.50 | 0.77 | -1.73 | 1.54 |
|  | -5 | 0.46 | -0.32 | 1.22 | -0.78 | 0.76 | 0.19 | -0.74 | 1.17 | -0.93 | 0.98 | -0.79 | -2.56 | 0.72 | -1.77 | 1.50 |
|  | -4 | 0.43 | -0.27 | 1.12 | -0.70 | 0.69 | 0.15 | -0.68 | 1.02 | -0.83 | 0.87 | -0.82 | -2.51 | 0.62 | -1.69 | 1.44 |
|  | -3 | 0.39 | -0.26 | 1.00 | -0.64 | 0.61 | 0.12 | -0.62 | 0.89 | -0.74 | 0.77 | -0.86 | -2.51 | 0.57 | -1.66 | 1.43 |
|  | -2 | 0.34 | -0.18 | 0.91 | -0.52 | 0.57 | 0.10 | -0.55 | 0.79 | -0.66 | 0.68 | -0.88 | -2.54 | 0.53 | -1.67 | 1.41 |
|  | -1 | 0.31 | -0.17 | 0.83 | -0.47 | 0.52 | 0.08 | -0.53 | 0.70 | -0.61 | 0.62 | -0.91 | -2.58 | 0.39 | -1.67 | 1.30 |
| 3 | -6 | 0.50 | -0.21 | 1.18 | -0.71 | 0.68 | 0.21 | -0.62 | 1.11 | -0.83 | 0.90 | -0.77 | -2.11 | 0.52 | -1.34 | 1.28 |
|  | -5 | 0.41 | -0.21 | 1.03 | -0.62 | 0.61 | 0.19 | -0.54 | 0.98 | -0.74 | 0.78 | -0.77 | -2.20 | 0.42 | -1.43 | 1.20 |
|  | -4 | 0.38 | -0.18 | 0.95 | -0.56 | 0.58 | 0.15 | -0.53 | 0.85 | -0.68 | 0.70 | -0.81 | -2.16 | 0.35 | -1.35 | 1.17 |
|  | -3 | 0.34 | -0.16 | 0.86 | -0.50 | 0.52 | 0.12 | -0.49 | 0.72 | -0.61 | 0.59 | -0.84 | -2.20 | 0.32 | -1.36 | 1.17 |
|  | -2 | 0.31 | -0.11 | 0.79 | -0.42 | 0.48 | 0.11 | -0.41 | 0.65 | -0.53 | 0.54 | -0.89 | -2.19 | 0.27 | -1.30 | 1.16 |
|  | -1 | 0.27 | -0.13 | 0.71 | -0.40 | 0.44 | 0.09 | -0.41 | 0.58 | -0.49 | 0.49 | -0.90 | -2.18 | 0.16 | -1.28 | 1.06 |

# Table 3.1-2. CRS based RSRP measurement accuracy with measurement bandwidth of 25RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.22 | -0.35 | 0.81 | -0.57 | 0.59 | -0.01 | -0.78 | 0.75 | -0.77 | 0.75 | -0.70 | -2.18 | 0.64 | -1.48 | 1.34 |
|  | -5 | 0.27 | -0.22 | 0.77 | -0.49 | 0.50 | 0.03 | -0.67 | 0.72 | -0.70 | 0.69 | -0.71 | -2.18 | 0.59 | -1.47 | 1.30 |
|  | -4 | 0.29 | -0.15 | 0.76 | -0.44 | 0.47 | 0.04 | -0.59 | 0.67 | -0.63 | 0.63 | -0.73 | -2.14 | 0.54 | -1.40 | 1.28 |
|  | -3 | 0.27 | -0.12 | 0.67 | -0.39 | 0.39 | 0.06 | -0.53 | 0.67 | -0.60 | 0.61 | -0.73 | -2.09 | 0.53 | -1.37 | 1.26 |
|  | -2 | 0.24 | -0.11 | 0.61 | -0.36 | 0.36 | 0.09 | -0.45 | 0.63 | -0.54 | 0.54 | -0.74 | -2.08 | 0.51 | -1.34 | 1.25 |
|  | -1 | 0.25 | -0.07 | 0.59 | -0.33 | 0.34 | 0.11 | -0.38 | 0.60 | -0.48 | 0.49 | -0.75 | -2.11 | 0.49 | -1.36 | 1.25 |
| 2 | -6 | 0.16 | -0.24 | 0.58 | -0.40 | 0.42 | -0.01 | -0.51 | 0.51 | -0.50 | 0.52 | -0.69 | -1.77 | 0.26 | -1.08 | 0.96 |
|  | -5 | 0.21 | -0.14 | 0.58 | -0.35 | 0.37 | 0.03 | -0.44 | 0.50 | -0.47 | 0.47 | -0.70 | -1.76 | 0.22 | -1.06 | 0.92 |
|  | -4 | 0.25 | -0.07 | 0.57 | -0.32 | 0.32 | 0.04 | -0.39 | 0.46 | -0.43 | 0.42 | -0.73 | -1.73 | 0.19 | -1.01 | 0.92 |
|  | -3 | 0.23 | -0.05 | 0.51 | -0.28 | 0.28 | 0.07 | -0.33 | 0.47 | -0.40 | 0.40 | -0.72 | -1.70 | 0.19 | -0.98 | 0.91 |
|  | -2 | 0.20 | -0.04 | 0.47 | -0.24 | 0.27 | 0.09 | -0.28 | 0.44 | -0.37 | 0.36 | -0.74 | -1.69 | 0.14 | -0.95 | 0.88 |
|  | -1 | 0.22 | -0.01 | 0.46 | -0.23 | 0.24 | 0.10 | -0.22 | 0.42 | -0.32 | 0.33 | -0.74 | -1.71 | 0.15 | -0.96 | 0.89 |
| 3 | -6 | 0.14 | -0.20 | 0.47 | -0.34 | 0.33 | -0.01 | -0.42 | 0.40 | -0.41 | 0.41 | -0.70 | -1.56 | 0.09 | -0.86 | 0.79 |
|  | -5 | 0.18 | -0.10 | 0.50 | -0.28 | 0.32 | 0.03 | -0.35 | 0.43 | -0.39 | 0.39 | -0.70 | -1.53 | 0.05 | -0.83 | 0.75 |
|  | -4 | 0.23 | -0.04 | 0.50 | -0.27 | 0.27 | 0.04 | -0.32 | 0.38 | -0.36 | 0.34 | -0.73 | -1.52 | 0.01 | -0.79 | 0.74 |
|  | -3 | 0.21 | -0.02 | 0.44 | -0.23 | 0.23 | 0.07 | -0.26 | 0.40 | -0.33 | 0.33 | -0.71 | -1.52 | 0.01 | -0.80 | 0.73 |
|  | -2 | 0.18 | -0.01 | 0.40 | -0.19 | 0.22 | 0.09 | -0.22 | 0.38 | -0.30 | 0.29 | -0.74 | -1.50 | -0.02 | -0.76 | 0.71 |
|  | -1 | 0.20 | 0.01 | 0.40 | -0.19 | 0.20 | 0.09 | -0.16 | 0.36 | -0.25 | 0.27 | -0.74 | -1.54 | -0.02 | -0.80 | 0.72 |

# Table 3.1-3. CRS based RSRP measurement accuracy with measurement bandwidth of 50RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.31 | -0.07 | 0.72 | -0.38 | 0.41 | 0.12 | -0.43 | 0.71 | -0.55 | 0.59 | -0.48 | -1.53 | 0.52 | -1.05 | 1.00 |
|  | -5 | 0.27 | -0.07 | 0.63 | -0.35 | 0.35 | 0.12 | -0.40 | 0.66 | -0.52 | 0.54 | -0.49 | -1.52 | 0.52 | -1.03 | 1.01 |
|  | -4 | 0.25 | -0.06 | 0.55 | -0.30 | 0.30 | 0.10 | -0.38 | 0.57 | -0.48 | 0.47 | -0.52 | -1.55 | 0.49 | -1.03 | 1.00 |
|  | -3 | 0.21 | -0.07 | 0.50 | -0.28 | 0.29 | 0.09 | -0.35 | 0.54 | -0.44 | 0.46 | -0.53 | -1.53 | 0.46 | -1.00 | 0.98 |
|  | -2 | 0.19 | -0.05 | 0.46 | -0.25 | 0.26 | 0.07 | -0.36 | 0.50 | -0.42 | 0.43 | -0.53 | -1.52 | 0.42 | -0.99 | 0.95 |
|  | -1 | 0.18 | -0.04 | 0.41 | -0.22 | 0.23 | 0.07 | -0.31 | 0.49 | -0.38 | 0.42 | -0.53 | -1.54 | 0.41 | -1.02 | 0.94 |
| 2 | -6 | 0.27 | 0.00 | 0.56 | -0.27 | 0.29 | 0.12 | -0.25 | 0.50 | -0.37 | 0.38 | -0.47 | -1.21 | 0.24 | -0.74 | 0.71 |
|  | -5 | 0.24 | -0.02 | 0.50 | -0.26 | 0.26 | 0.11 | -0.23 | 0.47 | -0.34 | 0.36 | -0.46 | -1.22 | 0.21 | -0.75 | 0.68 |
|  | -4 | 0.21 | -0.01 | 0.44 | -0.22 | 0.22 | 0.10 | -0.22 | 0.42 | -0.32 | 0.32 | -0.50 | -1.23 | 0.17 | -0.73 | 0.67 |
|  | -3 | 0.18 | -0.01 | 0.38 | -0.19 | 0.20 | 0.08 | -0.20 | 0.38 | -0.28 | 0.30 | -0.52 | -1.22 | 0.16 | -0.70 | 0.68 |
|  | -2 | 0.17 | -0.01 | 0.34 | -0.18 | 0.18 | 0.07 | -0.19 | 0.35 | -0.26 | 0.28 | -0.52 | -1.25 | 0.14 | -0.72 | 0.67 |
|  | -1 | 0.16 | 0.00 | 0.32 | -0.16 | 0.16 | 0.07 | -0.18 | 0.32 | -0.25 | 0.25 | -0.53 | -1.26 | 0.13 | -0.73 | 0.65 |
| 3 | -6 | 0.25 | 0.03 | 0.50 | -0.22 | 0.25 | 0.12 | -0.18 | 0.43 | -0.30 | 0.31 | -0.47 | -1.08 | 0.12 | -0.61 | 0.60 |
|  | -5 | 0.22 | 0.01 | 0.44 | -0.20 | 0.22 | 0.11 | -0.17 | 0.39 | -0.29 | 0.28 | -0.48 | -1.09 | 0.11 | -0.62 | 0.59 |
|  | -4 | 0.20 | 0.01 | 0.38 | -0.18 | 0.18 | 0.10 | -0.16 | 0.35 | -0.26 | 0.25 | -0.50 | -1.10 | 0.07 | -0.60 | 0.57 |
|  | -3 | 0.17 | 0.01 | 0.33 | -0.15 | 0.16 | 0.08 | -0.15 | 0.33 | -0.23 | 0.25 | -0.52 | -1.10 | 0.05 | -0.58 | 0.57 |
|  | -2 | 0.15 | 0.01 | 0.30 | -0.14 | 0.15 | 0.07 | -0.15 | 0.30 | -0.22 | 0.22 | -0.53 | -1.10 | 0.03 | -0.57 | 0.56 |
|  | -1 | 0.14 | 0.01 | 0.28 | -0.13 | 0.13 | 0.07 | -0.14 | 0.28 | -0.21 | 0.21 | -0.53 | -1.12 | 0.02 | -0.59 | 0.55 |

RSRQ

Table 3.2-1, Table3.2-2 and Table 3.2-3 show delta CRS-RSRQ in the measurement bandwidth of 6RB, 25RB  and 50RB  respectively.

# Table 3.2-1. CRS based RSRQ measurement accuracy with measurement bandwidth of 6RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.66 | -0.55 | 1.82 | -1.21 | 1.16 | 0.20 | -1.31 | 1.93 | -1.52 | 1.73 | -0.76 | -3.37 | 1.35 | -2.61 | 2.11 |
|  | -5 | 0.56 | -0.52 | 1.63 | -1.08 | 1.07 | 0.20 | -1.16 | 1.65 | -1.36 | 1.44 | -0.80 | -3.37 | 1.29 | -2.58 | 2.08 |
|  | -4 | 0.51 | -0.45 | 1.48 | -0.96 | 0.97 | 0.14 | -1.08 | 1.55 | -1.22 | 1.41 | -0.87 | -3.35 | 1.18 | -2.48 | 2.05 |
|  | -3 | 0.47 | -0.42 | 1.31 | -0.89 | 0.85 | 0.12 | -1.02 | 1.34 | -1.13 | 1.23 | -0.87 | -3.31 | 1.12 | -2.44 | 1.99 |
|  | -2 | 0.42 | -0.30 | 1.17 | -0.73 | 0.74 | 0.10 | -0.93 | 1.21 | -1.03 | 1.11 | -0.91 | -3.33 | 1.08 | -2.42 | 1.99 |
|  | -1 | 0.38 | -0.30 | 1.07 | -0.69 | 0.69 | 0.08 | -0.86 | 1.17 | -0.94 | 1.09 | -0.94 | -3.41 | 0.94 | -2.47 | 1.88 |
| 2 | -6 | 0.54 | -0.32 | 1.40 | -0.86 | 0.86 | 0.21 | -0.80 | 1.31 | -1.01 | 1.11 | -0.77 | -2.48 | 0.75 | -1.71 | 1.52 |
|  | -5 | 0.46 | -0.32 | 1.24 | -0.78 | 0.78 | 0.18 | -0.71 | 1.16 | -0.90 | 0.98 | -0.79 | -2.56 | 0.72 | -1.77 | 1.51 |
|  | -4 | 0.42 | -0.27 | 1.12 | -0.69 | 0.70 | 0.14 | -0.69 | 1.03 | -0.82 | 0.89 | -0.82 | -2.50 | 0.61 | -1.68 | 1.43 |
|  | -3 | 0.38 | -0.26 | 1.01 | -0.64 | 0.62 | 0.12 | -0.63 | 0.88 | -0.75 | 0.77 | -0.85 | -2.50 | 0.57 | -1.65 | 1.42 |
|  | -2 | 0.34 | -0.17 | 0.90 | -0.51 | 0.56 | 0.10 | -0.57 | 0.78 | -0.67 | 0.68 | -0.87 | -2.55 | 0.50 | -1.68 | 1.37 |
|  | -1 | 0.29 | -0.18 | 0.81 | -0.48 | 0.52 | 0.08 | -0.55 | 0.71 | -0.63 | 0.63 | -0.90 | -2.55 | 0.42 | -1.64 | 1.32 |
| 3 | -6 | 0.49 | -0.20 | 1.20 | -0.70 | 0.71 | 0.20 | -0.61 | 1.10 | -0.81 | 0.89 | -0.77 | -2.09 | 0.49 | -1.33 | 1.25 |
|  | -5 | 0.41 | -0.21 | 1.03 | -0.61 | 0.62 | 0.18 | -0.54 | 0.95 | -0.72 | 0.77 | -0.78 | -2.20 | 0.42 | -1.42 | 1.20 |
|  | -4 | 0.37 | -0.18 | 0.96 | -0.56 | 0.59 | 0.13 | -0.54 | 0.84 | -0.68 | 0.71 | -0.80 | -2.14 | 0.35 | -1.34 | 1.15 |
|  | -3 | 0.33 | -0.17 | 0.86 | -0.50 | 0.53 | 0.11 | -0.49 | 0.71 | -0.60 | 0.60 | -0.85 | -2.19 | 0.33 | -1.34 | 1.17 |
|  | -2 | 0.31 | -0.12 | 0.77 | -0.42 | 0.47 | 0.11 | -0.43 | 0.63 | -0.54 | 0.52 | -0.88 | -2.17 | 0.26 | -1.29 | 1.14 |
|  | -1 | 0.26 | -0.14 | 0.68 | -0.41 | 0.42 | 0.08 | -0.42 | 0.58 | -0.51 | 0.50 | -0.90 | -2.16 | 0.17 | -1.26 | 1.07 |

# Table 3.2-2. CRS based RSRQ measurement accuracy with measurement bandwidth of 25RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.22 | -0.35 | 0.80 | -0.56 | 0.58 | -0.01 | -0.76 | 0.72 | -0.75 | 0.73 | -0.70 | -2.18 | 0.63 | -1.47 | 1.34 |
|  | -5 | 0.26 | -0.22 | 0.77 | -0.48 | 0.51 | 0.02 | -0.68 | 0.71 | -0.70 | 0.69 | -0.71 | -2.19 | 0.57 | -1.48 | 1.28 |
|  | -4 | 0.29 | -0.15 | 0.76 | -0.44 | 0.47 | 0.04 | -0.60 | 0.68 | -0.64 | 0.65 | -0.73 | -2.13 | 0.54 | -1.40 | 1.27 |
|  | -3 | 0.26 | -0.12 | 0.67 | -0.38 | 0.41 | 0.05 | -0.54 | 0.66 | -0.59 | 0.61 | -0.73 | -2.09 | 0.51 | -1.36 | 1.24 |
|  | -2 | 0.24 | -0.11 | 0.60 | -0.35 | 0.36 | 0.08 | -0.46 | 0.64 | -0.54 | 0.56 | -0.74 | -2.08 | 0.49 | -1.34 | 1.24 |
|  | -1 | 0.24 | -0.08 | 0.58 | -0.32 | 0.34 | 0.09 | -0.39 | 0.59 | -0.48 | 0.50 | -0.75 | -2.08 | 0.49 | -1.33 | 1.24 |
| 2 | -6 | 0.16 | -0.24 | 0.57 | -0.41 | 0.40 | -0.01 | -0.50 | 0.50 | -0.48 | 0.51 | -0.70 | -1.77 | 0.25 | -1.08 | 0.94 |
|  | -5 | 0.20 | -0.14 | 0.58 | -0.35 | 0.37 | 0.03 | -0.44 | 0.49 | -0.47 | 0.47 | -0.70 | -1.76 | 0.21 | -1.06 | 0.92 |
|  | -4 | 0.24 | -0.07 | 0.57 | -0.32 | 0.33 | 0.03 | -0.39 | 0.46 | -0.42 | 0.43 | -0.72 | -1.72 | 0.19 | -1.00 | 0.91 |
|  | -3 | 0.22 | -0.05 | 0.52 | -0.27 | 0.30 | 0.06 | -0.34 | 0.47 | -0.39 | 0.41 | -0.73 | -1.69 | 0.16 | -0.96 | 0.89 |
|  | -2 | 0.19 | -0.04 | 0.46 | -0.23 | 0.26 | 0.08 | -0.28 | 0.44 | -0.36 | 0.36 | -0.74 | -1.69 | 0.13 | -0.94 | 0.87 |
|  | -1 | 0.21 | -0.02 | 0.44 | -0.23 | 0.23 | 0.09 | -0.23 | 0.41 | -0.32 | 0.33 | -0.74 | -1.70 | 0.15 | -0.96 | 0.89 |
| 3 | -6 | 0.14 | -0.20 | 0.46 | -0.34 | 0.33 | -0.01 | -0.42 | 0.40 | -0.41 | 0.41 | -0.71 | -1.57 | 0.07 | -0.86 | 0.78 |
|  | -5 | 0.18 | -0.11 | 0.49 | -0.29 | 0.31 | 0.03 | -0.36 | 0.42 | -0.39 | 0.40 | -0.71 | -1.54 | 0.05 | -0.83 | 0.76 |
|  | -4 | 0.22 | -0.03 | 0.48 | -0.25 | 0.26 | 0.03 | -0.32 | 0.38 | -0.35 | 0.35 | -0.73 | -1.51 | 0.01 | -0.79 | 0.73 |
|  | -3 | 0.20 | -0.03 | 0.45 | -0.22 | 0.25 | 0.06 | -0.27 | 0.39 | -0.33 | 0.33 | -0.72 | -1.52 | 0.00 | -0.80 | 0.72 |
|  | -2 | 0.18 | -0.01 | 0.39 | -0.19 | 0.21 | 0.08 | -0.21 | 0.37 | -0.29 | 0.29 | -0.74 | -1.49 | -0.02 | -0.75 | 0.72 |
|  | -1 | 0.19 | 0.00 | 0.38 | -0.20 | 0.19 | 0.08 | -0.17 | 0.35 | -0.25 | 0.26 | -0.74 | -1.53 | -0.02 | -0.78 | 0.73 |

# Table 3.2-3. CRS based RSRQ measurement accuracy with measurement bandwidth of 50RB

| (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 1 | -6 | 0.31 | -0.08 | 0.72 | -0.38 | 0.42 | 0.12 | -0.44 | 0.71 | -0.56 | 0.60 | -0.49 | -1.53 | 0.53 | -1.04 | 1.01 |
|  | -5 | 0.27 | -0.07 | 0.63 | -0.34 | 0.36 | 0.11 | -0.41 | 0.65 | -0.52 | 0.55 | -0.49 | -1.51 | 0.51 | -1.02 | 1.00 |
|  | -4 | 0.24 | -0.07 | 0.55 | -0.32 | 0.31 | 0.08 | -0.38 | 0.58 | -0.46 | 0.49 | -0.52 | -1.55 | 0.48 | -1.03 | 1.00 |
|  | -3 | 0.21 | -0.07 | 0.49 | -0.28 | 0.28 | 0.08 | -0.37 | 0.55 | -0.45 | 0.47 | -0.53 | -1.52 | 0.46 | -1.00 | 0.99 |
|  | -2 | 0.19 | -0.07 | 0.45 | -0.25 | 0.27 | 0.06 | -0.36 | 0.48 | -0.42 | 0.42 | -0.54 | -1.52 | 0.41 | -0.99 | 0.94 |
|  | -1 | 0.17 | -0.05 | 0.41 | -0.22 | 0.24 | 0.06 | -0.32 | 0.47 | -0.38 | 0.42 | -0.54 | -1.53 | 0.40 | -1.00 | 0.93 |
| 2 | -6 | 0.27 | 0.00 | 0.56 | -0.27 | 0.29 | 0.11 | -0.25 | 0.49 | -0.37 | 0.37 | -0.47 | -1.21 | 0.24 | -0.74 | 0.71 |
|  | -5 | 0.24 | -0.01 | 0.49 | -0.25 | 0.26 | 0.10 | -0.23 | 0.47 | -0.34 | 0.36 | -0.47 | -1.20 | 0.21 | -0.73 | 0.68 |
|  | -4 | 0.21 | -0.01 | 0.43 | -0.22 | 0.22 | 0.08 | -0.23 | 0.41 | -0.31 | 0.33 | -0.51 | -1.23 | 0.16 | -0.72 | 0.67 |
|  | -3 | 0.17 | -0.01 | 0.37 | -0.18 | 0.20 | 0.08 | -0.21 | 0.37 | -0.29 | 0.30 | -0.52 | -1.23 | 0.16 | -0.71 | 0.68 |
|  | -2 | 0.15 | -0.02 | 0.34 | -0.17 | 0.18 | 0.06 | -0.20 | 0.35 | -0.26 | 0.29 | -0.53 | -1.25 | 0.14 | -0.72 | 0.67 |
|  | -1 | 0.14 | -0.01 | 0.31 | -0.15 | 0.17 | 0.06 | -0.19 | 0.32 | -0.25 | 0.26 | -0.53 | -1.26 | 0.12 | -0.72 | 0.65 |
| 3 | -6 | 0.25 | 0.02 | 0.49 | -0.23 | 0.24 | 0.11 | -0.19 | 0.43 | -0.30 | 0.32 | -0.48 | -1.08 | 0.12 | -0.61 | 0.60 |
|  | -5 | 0.22 | 0.01 | 0.43 | -0.20 | 0.21 | 0.10 | -0.18 | 0.39 | -0.28 | 0.29 | -0.48 | -1.08 | 0.11 | -0.60 | 0.59 |
|  | -4 | 0.19 | 0.01 | 0.38 | -0.18 | 0.19 | 0.09 | -0.17 | 0.34 | -0.25 | 0.25 | -0.51 | -1.10 | 0.06 | -0.59 | 0.57 |
|  | -3 | 0.16 | 0.01 | 0.32 | -0.15 | 0.16 | 0.07 | -0.16 | 0.32 | -0.23 | 0.25 | -0.52 | -1.10 | 0.04 | -0.58 | 0.57 |
|  | -2 | 0.14 | 0.00 | 0.29 | -0.14 | 0.15 | 0.06 | -0.16 | 0.29 | -0.22 | 0.23 | -0.53 | -1.11 | 0.03 | -0.57 | 0.56 |
|  | -1 | 0.13 | 0.00 | 0.28 | -0.13 | 0.15 | 0.06 | -0.15 | 0.27 | -0.21 | 0.21 | -0.54 | -1.13 | 0.01 | -0.59 | 0.55 |

Based on the simulation results, we can see some observations as follows.

- Observation 1: For CRS based RSRP measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5 at  SNR of-1dB, but cannot be met with one shot measurement of  DRS period under ETU30 at  SNR of-1dB.

- Observation 2: For CRS based RSRP measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 3: For CRS based RSRP measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 4: For CRS based RSRQ measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5 at  SNR of-1dB, but cannot be met with one shot measurement of  DRS period under ETU30 at  SNR of-1dB.

- Observation 5: For CRS based RSRQ measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 6: For CRS based RSRQ measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

Based on these observations, we suggest following proposals.

- Proposal 1: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the measurement bandwidth of 25RB and 50RB to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

- Proposal 2: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the side condition of -1dB(Es/Iot)  to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

- Proposal 3: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the measurement period of one DRS period  to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

Conclusion

In this paper, we provided simulation results of a CRS based RSRP and RSRQ measurement accuracy with agreed simulation assumptions. From simulation results, we also provided some observations as follows.

- Observation 1: For CRS based RSRP measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5 at  SNR of-1dB, but cannot be met with one shot measurement of  DRS period under ETU30 at  SNR of-1dB.

- Observation 2: For CRS based RSRP measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 3: For CRS based RSRP measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 DRS based RSRP measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 4: For CRS based RSRQ measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5 at  SNR of-1dB, but cannot be met with one shot measurement of  DRS period under ETU30 at  SNR of-1dB.

- Observation 5: For CRS based RSRQ measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

- Observation 6: For CRS based RSRQ measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 DRS based RSRQ measurement accuracy can be met with one shot measurement of  DRS period under AWGN, EAP5, ETU30 at  SNR of-1dB.

Based on those, we suggest following proposals.

- Proposal 1: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the measurement bandwidth of 25RB and 50RB to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

- Proposal 2: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the side condition of -1dB(Es/Iot)  to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

- Proposal 3: For CRS based RSRP and RSRQ measurement accuracy of LAA, we propose the measurement period of one DRS period  to reuse Rel-12 DRS based RSRP and RSRP measurement accuracy.

- Reference

- R4-156645, “Simulation assumptions for RSRP/RSRQ evaluation for LAA”, Huawei, Hisilicon