# 3GPP TSG-RAN WG4 Meeting #77										R4-157449

# Anaheim, November 16th – 20th, 2015

Source: 	Nokia Networks

Title: 					MSD for CA_3A-7A-28A 3DL/2UL

Agenda Item:	7.36.2

Document for:	Discussion

# Introduction

In this contribution we present simulation results on maximum sensitivity degradation (MSD) for CA_3A-7A-28A in two 3DL/2UL configurations.

# Simulation assumptions

The IMD levels were obtained by simulating modulated signals, hence no correction factor was needed.

Tables 1 and 2 present the RF component parameters used in the simulations.

Table 1. Intercept points of the UE RF front-end

|  | IIP2 [dBm] | IIP3 [dBm] | IIP5 [dBm] |
| --- | --- | --- | --- |
| Diplexer | 112 | 85 | 55 |
| Switch | 112 | 68 | 55 |
| Duplexer | 102 | 74 | 52 |
| LNA | 10 | -10 | N/A |

Table 2. RF front-end gain/attenuation parameters

| Component | Parameter | Value |
| --- | --- | --- |
| Antenna | Loss between antennas [dB] | 10 |
| Diplexer | Insertion loss [dB] | 0.5 |
|  | High-low isolation [dB] | 15 |
| Switch | Insertion loss [dB] | 0.5 |
| Duplexer | Insertion loss [dB] | 3 |
|  | TX filter attenuation at RX band [dB] | 45 |
|  | TX-RX isolation [dB] | 48 |
| PCB | Isolation PA out – PA in [dB] | 60 |
| PA | In-band gain [dB] | 30 |

# MSD values

Table 3 presents the test configurations and simulated MSD values.

Table 3: MSD test configurations and simulated MSD values

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B28 | B3 | IMD2 | fB7 - fB3 | 1747 | 5 | 25 | 796.0 | 5 | 18.0 |
|  | B7 |  |  | 2543 | 5 | 25 |  |  |  |
|  | B7 | IMD2 | fB7 - fB28 | 2543 | 5 | 25 | 1832.5 | 5 | 17.3 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |

# Conclusions

This contribution presented simulated MSD values for two 3DL/2UL configurations of CA_3A-7A-28A, which exhibit substantial desensitization.

References,

[1] RP-151230, “Revision of WI: LTE Advanced 3DL/2UL inter-band Carrier Aggregation,” Nokia Networks, LG Electronics

[2] R4-156832, "MSD test configuration of CA_3A-7A-28A for 3DL/2UL inter-band CA,” LG Electronics, Qualcomm Inc.