3GPP TSG-RAN WG4 Meeting #77	R4-157560

# Anaheim, CA, US, 16 – 20 Nov, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3346 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE receiver requirements for Rel-13 MTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2015-11-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce RF requirements for rel-13 eMTC into TS36.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RF requirements such as operating bands, reference sensitivity and measurement channels for eMTC are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE RF requirements for eMTC will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3A, 5.5F, 7.2, 7.3.1F, A.2.2, A.2.3, A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE catergory [M1])

The requirements in clauses 5, 6 and 7 which are specific to CA, UL-MIMO, ProSe, Dual Connectivity , UE category 0, and category [M1] are specified as suffix A, B, C, D, E where;

a)	Suffix A additional requirements need to support CA

b)	Suffix B additional requirements need to support UL-MIMO

c)	Suffix C additional requirements need to support Dual Connectivity

d)	Suffix D additional requirements need to support ProSe

e)	Suffix E additional requirements need to support UE category 0

f)	Suffix F additional requirements need to support UE category [M1]

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffix A, B, C, D, E and F) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional subclause requirements (suffix A, B, C, D, and E) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional subclause.

A terminal which supports more than one feature (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, and UE category [M1]) in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

## <Next Changed section>

## 7.2	Diversity characteristics

The requirements in Section 7 assume that the receiver is equipped with two Rx port as a baseline. These requirements apply to all UE categories unless stated otherwise. Requirements for 4 ports are FFS. With the exception of subclause 7.9 all requirements shall be verified by using both (all) antenna ports simultaneously.

For a category 0 and a category [M1] UE the requirements in Section 7 assume that the receiver is equipped with single Rx port.

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to both the UE antenna ports for all UE categories except category 0 and category [M1], or to the single antenna port for UE category 0 and UE category [M1], at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

## <Next Changed section>

### 7.3.1E	Minimum requirements (QPSK) for UE category 0

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1E-1A/Table 7.3.1E-1B and Table 7.3.1E-2.

Table 7.3.1E-1A: Reference sensitivity for FDD and TDD UE category 0 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | -100.2 | -97.2 | -95.5 | -92.5 | -90.7 | -89.5 | FDD |
| 3 | -99.2 | -96.2 | -94.5 | -91.5 | -89.7 | -88.5 | FDD |
| 4 | -102.2 | -99.2 | -97.5 | -94.5 | -92.7 | -91.5 | FDD |
| 5 | -100.7 | -97.7 | -95.5 | -92.5 |  |  | FDD |
| 8 | -99.7 | -96.7 | -94.5 | -91.5 |  |  | FDD |
| 13 |  |  | -94 | -91 |  |  | FDD |
| 20 |  |  | -94.5 | -91.5 | -88.2 | -87 | FDD |
| 39 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 41 |  |  | -95.5 | -92.5 | -90.7 | -89.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |  |

Table 7.3.1E-1B: Reference sensitivity for HD-FDD UE category 0 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | -101 | -98 | -96.3 | -93.3 | -91.5 | -90.3 | HD-FDD |
| 3 | -100 | -97 | -95.3 | -92.3 | -90.5 | -89.3 | HD-FDD |
| 4 | -103 | -100 | -98.3 | -95.3 | -93.5 | -92.3 | HD-FDD |
| 5 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | HD-FDD |
| 8 | -100.5 | -97.5 | -95.3 | -92.3 |  |  | HD-FDD |
| 13 |  |  | -95.3 | -92.3 |  |  | HD-FDD |
| 20 |  |  | -95.3 | -92.3 | -89.5 | -88.3 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1E-1A/Table 7.3.1E-1B shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1E-2.

NOTE:	Table 7.3.1E-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex X (informative).

Table 7.3.1E-2: FDD and TDD UE category 0 Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 2 | 6 | 15 | 25 | [361] | [361] | [361] | FDD and HD-FDD |
| 3 | 6 | 15 | 25 | [361] | [361] | [361] | FDD and HD-FDD |
| 4 | 6 | 15 | 25 | [361] | [361] | [361] | FDD and HD-FDD |
| 5 | 6 | 15 | 25 | [251] |  |  | FDD and HD-FDD |
| 8 | 6 | 15 | 25 | 251 |  |  | FDD and HD-FDD |
| 13 |  |  | 201 | [201] |  |  | FDD and HD-FDD |
| 20 |  |  | 25 | 201 | 202 | 202 | FDD and HD-FDD |
| 39 |  |  | 25 | [361] | [361] | [361] | TDD |
| 41 |  |  | 25 | [361] | [361] | [361] | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: 2 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16. |  |  |  |  |  |  |  |

### 7.3.1F	Minimum requirements (QPSK) for UE category [M1]

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1F-1A/Table 7.3.1F-1B and Table 7.3.1F-2.

Table 7.3.1F-1A:  Reference sensitivity for FDD and TDD UE category [M1] QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | [-102.2] | [-102.2] | [-102.2] | [-102.2] | FDD |
| 2 | [-100.2] | [-100.2] | [-100.2] | [-100.2] | [-100.2] | [-100.2] | FDD |
| 3 | [-99.2] | [-99.2] | [-99.2] | [-99.2] | [-99.2] | [-99.2] | FDD |
| 4 | [-102.2] | [-102.2] | [-102.2] | [-102.2] | [-102.2] | [-102.2] | FDD |
| 5 | [-100.7] | [-100.7] | [-100.7] | [-100.7] |  |  | FDD |
| 7 |  |  | [-100.2] | [-100.2] | [-100.2] | [-100.2] | FDD |
| 8 | [-99.7] | [-99.7] | [-99.7] | [-99.7] |  |  | FDD |
| 11 |  |  | [-102.23] | [-102.23] |  |  | FDD |
| 12 | [-99.2] | [-99.2] | [-99.2] | [-99.2] |  |  | FDD |
| 13 |  |  | [-99.2] | [-99.2] |  |  | FDD |
| 18 |  |  | [-102.74] | [-102.74] | [-102.74] |  | FDD |
| 19 |  |  | [-102.74] | [-102.74] | [-102.74] |  | FDD |
| 20 |  |  | [-99.7] | [-99.7] | [-99.7] | [-99.7] | FDD |
| 21 |  |  | [-102.23] | [-102.23] | [-102.23] |  | FDD |
| 26 | [-100.2] | [-100.2] | [-100.2] | [-100.2] | [-100.2] |  | FDD |
| 27 | [-100.7] | [-100.7] | [-100.7] | [-100.7] |  |  | FDD |
| 28 |  | [-100.7] | [-100.7] | [-100.7] | [-100.7] | [-100.7] | FDD |
| 31 | [-96.5] | [-96.5] | [-96.5] |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | [-103.7] | [-103.7] | [-103.7] | [-103.7] | TDD |
| 41 |  |  | [-101.7] | [-101.7] | [-101.7] | [-101.7] | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18/19 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

Table 7.3.1F-1B:  Reference sensitivity for HD-FDD UE category [M1] QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | [-103.5] | [-103.5] | [-103.5] | [-103.5] | HD-FDD |
| 2 | [-101] | [-101] | [-101] | [-101] | [-101] | [-101] | HD-FDD |
| 3 | [-100] | [-100] | [-100] | [-100] | [-100] | [-100] | HD-FDD |
| 4 | [-103] | [-103] | [-103] | [-103] | [-103] | [-103] | HD-FDD |
| 5 | [-101.5] | [-101.5] | [-101.5] | [-101.5] |  |  | HD-FDD |
| 7 |  |  | [-101.5] | [-101.5] | [-101.5] | [-101.5] | HD-FDD |
| 8 | [-100.5] | [-100.5] | [-100.5] | [-100.5] |  |  | HD-FDD |
| 11 |  |  | [-103.53] | [-103.53] |  |  | HD-FDD |
| 12 | [-100] | [-100] | [-100] | [-100] |  |  | HD-FDD |
| 13 |  |  | [-100.5] | [-100.5] |  |  | HD-FDD |
| 18 |  |  | [-103.54] | [-103.54] | [-103.54] |  | HD-FDD |
| 19 |  |  | [-103.54] | [-103.54] | [-103.54] |  | HD-FDD |
| 20 |  |  | [-100.5] | [-100.5] | [-100.5] | [-100.5] | HD-FDD |
| 21 |  |  | [-103.53] | [-103.53] | [-103.53] |  | HD-FDD |
| 26 | [-101] | [-101] | [-101] | [-101] | [-101] |  | HD-FDD |
| 27 | [-101.5] | [-101.5] | [-101.5] | [-101.5] |  |  | HD-FDD |
| 28 |  | [-102] | [-102] | [-102] | [-102] | [-102] | HD-FDD |
| 31 | [-97.3] | [-97.3] | [-97.3] |  |  |  | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18/19 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1F-1A/Table 7.3.1F-1B shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1F-2.

NOTE:	Table 7.3.1F-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex X (informative).

Table 7.3.1F-2: FDD and TDD UE category [M1] Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 2 | 6 | [61] | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 3 | 6 | [61] | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 4 | 6 | [61] | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 5 | 6 | [61] | [61] | [61] |  |  | FDD and HD-FDD |
| 7 |  |  | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 8 | 6 | [61] | [61] | [61] |  |  | FDD and HD-FDD |
| 11 |  |  | [61] | [61] |  |  | FDD and HD-FDD |
| 12 | 6 | [61] | [61] | [61] |  |  | FDD and HD-FDD |
| 13 |  |  | [61] | [61] |  |  | FDD and HD-FDD |
| 18 |  |  | [61] | [61] | [61] |  | FDD and HD-FDD |
| 19 |  |  | [61] | [61] | [61] |  | FDD and HD-FDD |
| 20 |  |  | [61] | [61] | [62] | [62] | FDD and HD-FDD |
| 21 |  |  | [61] | [61] | [61] |  | FDD and HD-FDD |
| 26 | 6 | [61] | [61] | [61] | [61] |  | FDD and HD-FDD |
| 27 | 6 | [61] | [61] | [61] |  |  | FDD and HD-FDD |
| 28 |  | [61] | [61] | [61] | [61] | [61] | FDD and HD-FDD |
| 31 | 6 | [53] | [53] |  |  |  | FDD and HD-FDD |
| ... |  |  |  |  |  |  |  |
| 39 |  |  | [61] | [61] | [61] | [61] | TDD |
| 41 |  |  | [61] | [61] | [61] | [61] | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: 2 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 NOTE 3: 3refers to Band 31; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

## <Next Changed section>

## A.2.2	Reference measurement channels for FDD

### A.2.2.1	Full RB allocation

#### A.2.2.1.1	QPSK

Table A.2.2.1.1-1 Reference Channels for QPSK with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/5 | 1/6 |
| Payload size | Bits | 600 | 1544 | 2216 | 5160 | 4392 | 4584 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 | 4320 | 7200 | 14400 | 21600 | 28800 |
| Total symbols per Sub-Frame |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |

Table A.2.2.1.1-1a Reference Channels for QPSK with full/maximum RB allocation for UE category 0

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | [36] | [36] | [36] |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/5 | 1/8 | [1/10] | [1/10] | [1/10] |
| Payload size | Bits | 600 | 872 | 904 | [1000] | [1000] | [1000] |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 | 4320 | 7200 | [10368] | [10368] | [10368] |
| Total symbols per Sub-Frame |  | 864 | 2160 | 3600 | [5184] | [5184] | [5184] |
| UE Category |  | 0 | 0 | 0 | 0 | 0 | 0 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |

Table A.2.2.1.1-1b Reference Channels for QPSK with full/maximum RB allocation for UE category [M1]

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks3 |  | [4] | [4] | [4] | [4] | [4] | [4] |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 392 | 392 | 392 | 392 | 392 | 392 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame | Bits | [1152] | [1152] | [1152] | [1152] | [1152] | [1152] |
| Total symbols per Sub-Frame |  | [576] | [576] | [576] | [576] | [576] | [576] |
| UE Category |  | [M1] | [M1] | [M1] | [M1] | [M1] | [M1] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. NOTE 3: 2 resource blocks allocated to M-PDCCH |  |  |  |  |  |  |  |

## <End of Changes>