3GPP TSG-RAN WG4 (Radio) Meeting #77	R4-157865

Anaheim, US, 16 – 20 November 2015

Agenda Item:	9.3.3

Source: 	Alcatel-Lucent

Title: 	System level simulation results for coexistence study on new Band 41 UE power class supporting +26 dBm (urban and suburban areas)

Document for:	Discussion

# 1.	Introduction

The study item on New Band 41 UE power class supporting +26 dBm was approved at RAN#68 [1]. The objectives of this study item include the co-existence and compatibility of LTE systems deployed in the 2.5 GHz band and the impact on eNode B blocking requirements. Hence system level simulations need to be performed for the coexistence study.

The system level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm was approved at RAN4#76bis [2]. This contribution provides the system level simulation results using the approved assumptions in the urban and suburban areas.

# 2.	Simulation Results and Discussion

The inter-site distances considered in this study are provided in Table 2.1 below. Note that the UL power control parameters are not adjusted according to the UE maximum output power according to the approved assumptions in the urban and suburban areas. This represents the case where the 26 dBm UE is used to enhance the UL throughput in the urban and suburban areas. The channel bandwidth of 20 MHz is simulated.

Table 2.1: Inter-site distances (ISD)

| Environment | ISD (km) | ISD (miles) |
| --- | --- | --- |
| Urban | 0.75 | 0.47 |
| Suburban | 2.8 | 1.74 |

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 0.75 km inter-site distance are shown in Figure 2.1 below.

(a) With Power Control Parameter Set 1

(b) With Power Control Parameter Set 1’

(c) With Power Control Parameter Set 2

Figure 2.1: For 0.75 km inter-site distance

It can be seen from the CDFs of the UE transmit power in Figure 2.1 that the 26 dBm UE are constantly transmitting 3 dB higher power than the 23 dBm UE. This is expected as the CLx-ile is not adjusted according to the UE maximum output power. Comparing the CDFs of the UE transmit power with Set 1 and Set 1’, it can be seen that more (~5%) of the UE population are transmitting with maximum output power with the more aggressive Set 1.

Moreover, it can be seen from the victim system UL throughput loss Vs ACLR offset results in Figure 2.1 that due to the constantly 3 dB higher transmit power of the interfering UE, the ACLR of the 26 dBm UE need to be improved (~4 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE.

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 2.8 km inter-site distance are shown in Figure 2.2 below.

(a) With Power Control Parameter Set 1

(b) With Power Control Parameter Set 2

Figure 2.2: For 2.8 km inter-site distance

Similar observations can be made from the results in Figure 2.2, namely that the 26 dBm UE are constantly transmitting 3 dB higher power than the 23 dBm UE, more (~2.5%) of the UE population are transmitting with maximum output power with the more aggressive Set 1, and the ACLR of the 26 dBm UE need to be improved (~4 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE

# 3.	Conclusion

This contribution provides the system level simulation results using the approved assumptions in the urban and suburban areas. The simulation results show that if the UL power control parameters are not adjusted according to the UE maximum output power, due to the constantly 3 dB higher transmit power of the interfering UE, the ACLR of the 26 dBm UE need to be improved (~4 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE.

# References

[1]	RP-151104, “Study on- New Band 41 UE power class supporting +26 dBm”, Sprint, CMCC, Alcatel-Lucent, Softbank, C-Spire, Sequans, Ericsson, Intel, Samsung, SouthernLINC, Dish, Blackberry, China Unicom.

[2]	R4-156846, “Way Forward on System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm”, Sprint, Alcatel-Lucent.