3GPP TSG-RAN WG4 Meeting #80bis 	R4-167872

# Ljubljana, SI, 10 - 14 Oct 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 913 | rev |  | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: NPRACH performance requirements (TS36.141) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-09-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of NPRACH performance for NB-IoT |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sections 8.6 for NPRACH performance for NB-IoT and A.x for new NPRACH test preambles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No specified requirements for NPRACH demodulation performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.6 and A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----<< START OF THE CHANGE 1 >>-----------

## 8.6	Performance requirements for NPRACH

#### 8.6.1	Definition and applicability

The performance requirement of NPRACH preamble detection is determined by the total probability of false detection of the preamble(Pfa) and the probability of detection of preamble (Pd).

Pfa is the conditional total probability of erroneous detection of a preamble (i.e. erroneous detection from any detector) when the preamble is not present. Pfa shall be less than or equal to 0.1%.

Pd is the conditional probability of correct detection of a preamble when the preamble is present. The following cases are counted as incorrect detection of a preamble when the preamble is present:

- Miss-Detection

  - not detecting any preamble at all

  - detecting a wrong  preamble other than the one that is sent

- Timing estimation error

  - timing estimation error occurs for a correctly detected preamble when the estimated timing is larger than [3.75]μs away from the strongest path

The performance of Pd is measured by the probability of miss-detection (Pmd) and the probability of timing estimation error (Pte). With the SNR levels listed in table 8.6.2-1, Pmd shall be less than or equal to 1% and Pte shall also be less than or equal to 1%.

The test NPRACH preambles are listed in Table A.x-1.

NPRACH normal mode test in Table 8.6.5-1 is applicable to all BS that support Narrowband-IoT.

#### 8.6.2	Minimum Requirement

The minimum requirement is in TS 36.104 [2] subclause 8.6.1.1 and 8.6.2.1.

#### 8.6.3	Test purpose

The test shall verify the receiver’s ability to detect NPRACH preamble under multipath fading propagation conditions for a given SNR.

#### 8.6.4	Method of test

##### 8.6.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.6.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth.

Table 8.6.4.2-1: AWGN power level at the BS input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in Annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex B.

4)	Adjust the frequency offset of the test signal according to Table 8.6.5-1 .

5)	Adjust the equipment so that the SNR specified in Table 8.6.5-1  is achieved at the BS input during the NPRACH preambles.

6) 	The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.6.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 8.6.4.2-1: NPRACH preamble test pattern

The timing offset is set to [30]us during the test.

#### 8.6.5	Test Requirement

Pfa shall not exceed 0.1%, Pmd shall not exceed 1% and Pte shall not exceed 1% for the SNRs in Table 8.6.5-1 .

Table 8.6.5-1: NPRACH missed detection test requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |
|  | 4 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |
|  | 8 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |

-----<< END OF THE CHANGE 1>>-----------

## -----<< START  OF THE CHANGE 2>>-----------

## A.x	Test preambles for NPRACH

Table A.x -1 Test preambles for NPRACH

| NPRACH Burst format | Format 0 | Format 1 |
| --- | --- | --- |
| Number of repetition | 8 | 8 |
| Number of subcarriers | 12 | 12 |
| Cell-ID | 0 | 0 |
| Initial subcarrier index | 0 | 0 |

-----<< END OF THE CHANGE 2>>-----------