3GPP TSG-RAN WG4 Meeting #80bis	R4-167939

Ljubljana, Slovenia, 10th – 14th Oct 2016

Title: 	Simulation Result for V2V REFSENS

Source: 	Qualcomm Incorporated

Agenda item:	8.19.4

Document for:	Discussion

# Introduction

In RAN4#8bis, it is agreed [1] that further evaluate and align the demodulation results for PSCCH and PSSCH based on the updated RAN1 agreements is needed. The simulation assumption is subjected to change according to the updated RAN1 agreements.

In this paper, we present our discussion and simulation result based on the latest RAN1 agreement.

# Discussion

We first present our view on several aspect of REFSENS simulation assumptions. Those include: including SA/Data multiplexing, SA/Data joint decoding, SA payload, SA power boosting, HARQ retransmissions, soft-combining, frequency offset and RMC.

## SA/Data multiplexing

RAN1 has reached working assumptions for FDM. At the same time, RAN Plenary has agreed to not specify the case where PSSCH and PSCCH are TDMed for Transmission Mode 3 and 4. Thus, we propose to study only FDM.

Proposal 1: For SA/Data multiplexing only FDM of PSCCH and associated PSSCH be studied.

## SA/Data joint decoding

Since SA decoding is a mandatory step for decoding data, one should always consider joint SA/Data decoding. The joint BLER is defined as

BLER = 1 - (1 – BLERSA) * (1 – BLERData) .

Proposal 2: Always consider joint SA/Data decoding. The BLER for joint decoding is defined as

BLER = 1 - (1 – BLERSA) * (1 – BLERData) .

## SA power boosting

Currently, RAN1 agree on the following

  - PSD offset to that of PSSCH is used to determine the transmission power of PSCCH.

    - The PSD offset is fixed to X(FFS in [0, 3]) dB in specifications.

After email discussion it was agreed that a PSD boost of [3]dB will be used. When there is PSD boosting, the SNR for PSCCH is [3] dB higher than that of PSSCH. When calculating the joint BLER, this fact has to be taken into account by shifting the SA BLER curve up by [3] dB.

Proposal 3: If SA power is boosted by [3] dB, the SA BLER curve should be shifted up by [3] dB in the joint BLER calculation.

Currently, only 2 values of X are likely to be chosen, i.e. 0 and 3. We propose that RAN4 consider these two cases as the baseline to save time. The actual result to be chose will depend on the final outcome in RAN1.

Proposal 4: Consider X = 0 and X = 3 as the two baselines.

## HARQ retransmissions and Soft-combining

To keep the study simple, we propose not to consider HARQ retransmission and soft-combining at this stage.

Proposal 5: No HARQ retransmission and no soft-combining are used for REFSENS simulations.

## Frequency offset

As it is agreed 600 Hz that transmit frequency error is ± 0.1ppm and band 47 is at around 6GHz, we think that ±600Hz is an acceptable assumption.

Proposal 6: Assume ±600 Hz frequency offset.

## FMC

We propose to use the FMC given in Table 1. The channel is AWGN.

Table 1 Fixed Reference measurement channel for V2V receiver requirements

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  |  |  |  | 48 |  | 96 |
| Subcarriers per resource block |  |  |  |  | 12 |  | 12 |
| Packets per SA period |  |  |  |  | 1 |  | 1 |
| Modulation |  |  |  |  | QPSK |  | QPSK |
| Target Coding Rate |  |  |  |  | 1/3 |  | 1/3 |
| Transport Block Size |  |  |  |  | 3496 |  | 6968 |
| Transport block CRC | Bits |  |  |  | 24 |  | 24 |
| Number of Code Blocks per Sub-Frame |  |  |  |  | 1 |  | 2 |
| Maximum number of HARQ transmissions |  |  |  |  | [1] |  | [1] |
| Binary Channel Bits per subframe | Bits |  |  |  | 11520 |  | 23040 |
| Max. Throughput averaged over 1 SA period of 100ms | kbps |  |  |  | 34.96 |  | 69.68 |
| UE Category |  |  |  |  | ≥ 1 |  | ≥ 1 |
| Note 1: 2RBs allocated to SA transmission and 4 symbols allocated to RS. Note 2: Throughput (in kbps) will depend on SA period configuration. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

# Simulation results

Figure 1: Decode Results for 10MHz and 20 MHz.

In Figure 1, we present our result on the decoding performance under the simulation assumptions discusses in Section 2. We observe that the SA BLER curve shifted by 3 dB is identically 0. This means that for the case where SA is boosted by 3dB, we can ignore SA BLER in the joint SA/Data BLER. On the other hand, if there is no power boosting, SA BLER has noticeable effect on the joint SA/Data BLER.

Observation 1: We can only neglect SA BLER if there is 3dB power boosting for SA.

For 10 MHz, the 5% BLER point for joint SA/Data BLER is -3.18 if there is no SA power boosting, -3.19 if there is 3dB SA power boosting. For 20 MHz, the 5% BLER point for joint SA/Data BLER is -3.15 if there is no SA power boosting, -3.16 if there is 3dB SA power boosting. These SNR value can be used to define V2V REFSENS.

Proposal 7: If 3dB PSD boost is used for SA,

- For 10MHz bandwidth: SNRV2V = -3.19,

- For 20MHz bandwidth: SNRV2V = -3.16.

If PSD boost is not used,

- For 10MHz bandwidth: SNRV2V = -3.18,

- For 20MHz bandwidth: SNRV2V = -3.15.

# Conclusions

In this contribution, we analyze the V2V REFSENS relevant issues and have following proposals:

Proposal 1: For SA/Data multiplexing only FDM of PSCCH and associated PSSCH be studied.

Proposal 2: Always consider joint SA/Data decoding. The BLER for joint decoding is defined as

BLER = 1 - (1 – BLERSA) * (1 – BLERData) .

Proposal 3: If SA power is boosted by [3] dB, the SA BLER curve should be shifted up by [3] dB in the joint BLER calculation.

Proposal 4: Consider X = 0 and X = 3 as the two baselines.

Proposal 5: No HARQ retransmission and no soft-combining are used for REFSENS simulations.

Proposal 6: Assume ±600 Hz frequency offset.

Observation 1: We can only neglect SA BLER if there is 3dB power boosting for SA.

Proposal 7: If 3dB PSD boost is used for SA,

- For 10MHz bandwidth: SNRV2V = -3.19,

- For 20MHz bandwidth: SNRV2V = -3.16.

If PSD boost is not used,

- For 10MHz bandwidth: SNRV2V = -3.18,

- For 20MHz bandwidth: SNRV2V = -3.15.

# Reference

- R4-167056, “WF on target SNR evaluation for V2V REFSENS requirements”, LG Electronics, Huawei, Hi-Silicon, Ericsson, KT, CATT, LG, Uplus, SKT, 3GPP TSG-RAN WG4 Meeting #80