3GPP TSG-RAN WG4 Meeting #80bis 	R4-167945

Gothenburg, Sweden, 22 – 26 August, 2016

Source: 	Qualcomm Incorporated

Title: 	MPR for V2V

Agenda Item:	8.19.1

Document for:	Discussion

# Introduction

We present our MPR simulation results for LTE-V2V waveforms for both the two cases: 1) SA and Data adjacent in frequency and 2) SA and Data nonadjacent in frequency. Some proposal on the definition of MPR for V2V is also discussed based on the obtained results.

# Discussion

In RAN4#80, a way forward [1] on treating the remaining RF issues for V2V is agreed. MPR is one of the high priority items

- Issue 1: MPR for PSCCH + PSSCH on adjacent/non-adjacent PRBs need further study

  - Considerations: Separate DFT pre-coder, PRB size and location constraints, PSD imbalance

As discussed [2], the waveform with simultaneous PSCCH and PSSCH transmission (adjacent or non-adjacent) will be different from a single-cluster or multi-cluster PUSCH waveform due to multiple DFT pre-coders. IN other words, simultaneous PSCCH and PSSCH is a 2 x SC-FDMA waveform, while PUSCH single-cluster is SC-FDMA and multi-cluster PUSCH is (DFT-S-OFDM). The baseband processing is depicted in the figures below.

| Figure 1: Single-cluster PUSCH transmission (SC-FDMA waveform) | Figure 2: Simultaneous (adjacent) PSCCH + PSSCH transmission (2 x SC-FDMA waveform) |
| --- | --- |

| Figure 3: Multi-cluster PUSCH transmission (DFT-S-OFDM waveform) | Figure 4: Simultaneous (non-adjacent) PSCCH + PSSCH transmission (2 x SC-FDMA waveform) |
| --- | --- |

There are also other agreements in RAN1 and RAN4 that further affect the MPR study:

- Sub channelization. (RAN1)

  - Each V2V transmission must occupy an integer number of sub channels. In case SA and Data are adjacent, the occupied sub channels contain 2 RBs of SA at the start of first sub channel. In case SA and Data are nonadjacent, there will be SA sub channels and Data sub channels. The occupied sub channels include one SA sub channel and multiples Data sub channels. SA sub channels size is 2 RBs.

  - Possible Data sub channel size is

    - {5, 10, 15, 20, 25, 50} RBs if SA and Data are adjacent

    - {4, 5, 6, 8, 10, 12, 15, 16, 18, 20, 24, 25, 27, 30, 32, 36, 40, 48, 72, 96} RBs if SA and Data are nonadjacent.

  - If the number of remaining RBs used for Data transmission does not satisfy the DFT size constrain. The largest number of RBs that is smaller than the remaining RBs and satisfy the DFT size constrain is used.

- Emission Requirements (RAN4): It is agreed that in band 47, UTRA-ACLR1 and UTRA-ACLR2 requirements are not necessary.

- There is a possible PSD boosting of X dB for SA, where X is likely to be 0 or 3 dB. Since SA is more likely to be near the band edge, this may negatively affects the out of band emission.

## QPSK

Figure 1: MPR when SA and Data are adjacent, no PSD boosting for SA

In figure 1 are the MPR result for the case SA and Data are adjacent and QPSK modulation is used in Data (SA modulation is always QPSK). The first observation is that the number of possible waveforms is greatly reduced due to the sub channelization constraint. The second observation is that the MPR is much lower than legacy LTE MPR. This is mainly due to the fact that the emission requirement in band 47 is more relaxed (no UTRA-ACLR requirement).

Figure 2. MPR when SA and Data are adjacent, 3dB boosting for SA

When there is 3dB power boosting for SA, the MPR result is captured in figure 2. As expected, the MPR result is a little worse due to increasing the PSD near the channel edge.

## 16 QAM

Figure 3. MPR when SA and Data are not adjacent, no PSD boosting for SA

Figure 4. MPR when SA and Data are not adjacent, 3dB boosting for SA

The MPR results for the case SA and Data are adjacent and 16QAM modulation is used in Data (SA modulation is always QPSK) are presented in Figure 3 and 4. We can see that the required backoff is a little higher for this modulation order.

## Proposed MPR

Based on the simulation assumption, we propose to define MPR for 2x SC-FDM waveforms in LTE-V2V as follows

Table 1. MPR for 2x SC-FDM waveforms in LTE-V2V without SA power boosting

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  | > 19 |  | > 40 | ≤ 0.5 |
| 16 QAM |  |  |  | >9 and ≤ 19 |  | > 9 and ≤ 30 | ≤ 0.5 |
| 16 QAM |  |  |  | > 19 |  | > 30 | ≤ 1.5 |

Table 2. MPR for 2x SC-FDM waveforms in LTE-V2V with 3dB SA power boosting

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  | > 9 |  | > 9 | ≤ 1 |
| 16 QAM |  |  |  | >9 |  | > 9 | ≤ 1.5 |

Proposal 1: RAN4 to define the MPR for the case SA and Data adjacent as in Table 1 and Table 2.

# Conclusion

Proposal 1: RAN4 to define the MPR for the case SA and Data adjacent as in Table 1 and Table 2.

# References

- R4-167151, WF on V2X UE RF requirements, Qualcomm, Ericsson