3GPP TSG-WG4 Meeting #80bis	R4-168052

# Ljubljana, Slovenia, 10th – 14th October 2016

Agenda item: 	6.3.4.1.1

Source: 	Qualcomm Incorporated

Title: 	NPBCH Simulation Results with Keep-Trying Approach

Document for:	Discussion

# 1	Introduction

For NPBCH, RAN4 [1] agree with the following simulation assumptions for alignment, and related main configurations are listed as in Table 1. This paper provides the simulation result for SNR satisfying 1% Pm-bch with kepp-trying approach.

# Table 1: Minimum performance for NPBCH

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 1 x 1 | 1 | TBD |
| 2 | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 2 x 1 | 1 | TBD |

Table 2:	RMC table for NPBCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.1 | R.NB1.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 50/1600 | 50/1600 |
| Payload (without CRC) | Bits | 34 | 34 |

The simulation assumes that NRS is available only in subframe 0 and subframe 4 of every frame and subframe 9 in every other frame which contains NSSS. This enables channel estimation to be performed using time domain filtering over destaggered NRS tones. A low filter coefficient value is chosen since the channel model considered is a slow fading channel with 1Hz doppler. Simulations results presented here also include RF margin.

# 2	Simulation Results

The following plot shows the NPBCH BLER performance with respect to SNR(dB) for simulation number 1 in Table 1.

Observation 1a: For simulation number 1a, with keep-trying window W=1, 1% Pm-bch is achieved at -7dB SNR.

Observation 1b: For simulation number 1b, with keep-trying window W=2, 1% Pm-bch is achieved at -10 dB SNR.

Observation 1c: For simulation number 1c, with keep-trying window W=3, 1% Pm-bch is achieved at -11.2 dB SNR.

Observation 1d: For simulation number 1d, with keep-trying window W=4, 1% Pm-bch is achieved at -12.1 dB SNR.

Observation 1e: For simulation number 1e, with keep-trying window W=5, 1% Pm-bch is achieved at -12.5 dB SNR.

Observation 1f: For simulation number 1f, with keep-trying window W=6, 1% Pm-bch is achieved at -13.1dB SNR.

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Keep-Trying Window ‘W’ | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1a | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 1 | 1 x 1 | 1 | -7 |
| 1b | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 2 | 1 x 1 | 1 | -10 |
| 1c | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 3 | 1 x 1 | 1 | -11.2 |
| 1d | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 4 | 1 x 1 | 1 | -12.1 |
| 1e | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 5 | 1 x 1 | 1 | -12.5 |
| 1f | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 6 | 1 x 1 | 1 | -13.1 |

The following plot shows the NPBCH BLER performance with respect to SNR(dB) for simulation number 2 in Table 1.

Observation 2a: For simulation number 2a, with keep-trying window W=1, 1% Pm-bch is achieved at -8.5 dB SNR.

Observation 2b: For simulation number 2b, with keep-trying window W=2, 1% Pm-bch is achieved at -10.8 dB SNR.

Observation 2c: For simulation number 2c, with keep-trying window W=3, 1% Pm-bch is achieved at -11.8 dB SNR.

Observation 2d: For simulation number 2d, with keep-trying window W=4, 1% Pm-bch is achieved at -12.5 dB SNR.

Observation 2e: For simulation number 2e, with keep-trying window W=5, 1% Pm-bch is achieved at -12.7 dB SNR.

Observation 2f: For simulation number 2f, with keep-trying window W=6, 1% Pm-bch is achieved at -13.1 dB SNR.

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Keep-Trying Window ‘W’ | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 2a | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 1 | 2 x 1 | 1 | -8.5dB |
| 2b | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 2 | 2 x 1 | 1 | -10.8dB |
| 2c | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 3 | 2 x 1 | 1 | -11.8dB |
| 2d | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 4 | 2 x 1 | 1 | -12.5dB |
| 2e | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 5 | 2 x 1 | 1 | -12.7dB |
| 2f | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 6 | 2 x 1 | 1 | -13.1dB |

Proposal: RAN4 should consider the trade-off between BLER performance and NPBCH acquisition delay with different window sizes and pick an appropriate window size when specifying the NPBCH requirement.

# 3	Conclusions

This paper contains NPBCH simulation results for alignment and made the following observations:

Observation 1a: For simulation number 1a, with keep-trying window W=1, 1% Pm-bch is achieved at -7dB SNR.

Observation 1b: For simulation number 1b, with keep-trying window W=2, 1% Pm-bch is achieved at -10 dB SNR.

Observation 1c: For simulation number 1c, with keep-trying window W=3, 1% Pm-bch is achieved at -11.2 dB SNR.

Observation 1d: For simulation number 1d, with keep-trying window W=4, 1% Pm-bch is achieved at -12.1 dB SNR.

Observation 1e: For simulation number 1e, with keep-trying window W=5, 1% Pm-bch is achieved at -12.5 dB SNR.

Observation 1f: For simulation number 1f, with keep-trying window W=6, 1% Pm-bch is achieved at -13.1dB SNR.

Observation 2a: For simulation number 2a, with keep-trying window W=1, 1% Pm-bch is achieved at -8.5 dB SNR.

Observation 2b: For simulation number 2b, with keep-trying window W=2, 1% Pm-bch is achieved at -10.8 dB SNR.

Observation 2c: For simulation number 2c, with keep-trying window W=3, 1% Pm-bch is achieved at -11.8 dB SNR.

Observation 2d: For simulation number 2d, with keep-trying window W=4, 1% Pm-bch is achieved at -12.5 dB SNR.

Observation 2e: For simulation number 2e, with keep-trying window W=5, 1% Pm-bch is achieved at -12.7 dB SNR.

Observation 2f: For simulation number 2f, with keep-trying window W=6, 1% Pm-bch is achieved at -13.1 dB SNR.

Proposal: RAN4 should consider the trade-off between BLER performance and NPBCH acquisition delay with different window sizes and pick an appropriate window size when specifying the NPBCH requirement.

# 4	References

- R4-79AH-0161, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE