3GPP TSG-RAN WG4 (Radio) Meeting #80bis	R4-168087

Ljubljana, Slovenia, 10 – 14 October, 2016

Agenda Item:	10.3.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposal on UL power control parameters for coexistence study for WP5D on new radio access technology

Document for:	Approval

# 1.	Introduction

The revised study item on New Radio Access Technology was approved at TSG RAN#72 [1]. The objectives of this study item include identifying relevant RF parameters to be used for sharing and co-existence studies. Also a LS was received at TSG RAN#72 [2] from ITU-R WP5D asking for characteristics of terrestrial IMT systems for frequency sharing/interference analysis in 24.25 - 86 GHz. ITU-R WP5D sent another LS on “Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” in [3]. On the other hand, a LS has been sent from ITU-R WP5D to 3GPP describing detailed modelling and simulation of IMT networks for use in sharing and compatibility studies [4]. This topic has been discussed at previous RAN4 meetings, and the way forwards on simulation assumptions were approved in RAN4#80 [5-7]. It is stated in [5] that the TPC model specified in Section 9.1 TR 36.942 [8] is applied, and that parameters assumed in the TPC model are FFS.

This contribution provides the simulation results using the agreed assumptions, and proposes the assumptions on the UL power control parameters for the coexistence study for WP5D in order to facilitate the calibration process and final output of the study.

# 2.	Discussion

To investigate the effects of the UL power control parameters, simulation runs have been performed for the urban macro scenario with the approved assumptions [5-7]. The simulation results on the transmit power of the victim and interfering UE for both coordinated and uncoordinated operations are shown in Figure 1 below, where Gamma of 1 and CLx-ile of 112 (current settings in TR 36.942 [8] for 10 MHz channel bandwidth) are assumed in the simulation.

(a) Coordinated operation

(b) Uncoordinated operation

Figure 1: UE transmit power (using Gamma of 1 and CLx-ile of 112)

It can be seen in Figure 1 that for both coordinated and uncoordinated operations using Gamma of 1 and CLx-ile of 112, around 40% of the victim and interfering UE are transmitting with maximum power of 23 dB. This phenomenon would not be observed in a real-life network normally. To better represent normal network operation, the CL-xile needs to be increased in order to lower the UE transmit power. It can be seen from Figure 1 that the curves would need to be shifted around 20 dB to the left in order to have around 10% of the UE transmitting with maximum power. Hence simulation runs using Gamma of 1 and CLx-ile of (112 + 20 =) 132 have been performed and the results are shown in Figure 2 below for both coordinated and uncoordinated operations.

(a) Coordinated operation

(b) Uncoordinated operation

Figure 2: UE transmit power (using Gamma of 1 and CLx-ile of 132)

It can be seen in Figure 2 that for both coordinated and uncoordinated operations using Gamma of 1 and CLx-ile of 132, as expected, around 10% of the victim and interfering UE are transmitting with maximum power of 23 dB, which could better represent normal network operation.

Similarly, the simulation results on the transmit power of the victim and interfering UE for both coordinated and uncoordinated operations using Gamma of 0.8 and CLx-ile of 129 (current settings in TR 36.942 [8] for 10 MHz channel bandwidth) are shown in Figure 3 below.

(a) Coordinated operation

(b) Uncoordinated operation

Figure 3: UE transmit power (using Gamma of 0.8 and CLx-ile of 129)

It can be seen in Figure 3 that for both coordinated and uncoordinated operations using Gamma of 0.8 and CLx-ile of 129, there is still around 15% of the victim and interfering UE are transmitting with maximum power of 23 dB. To better represent the less aggressive UL power control algorithm Set 2, the CL-xile needs to be increased in order to lower the UE transmit power. It can be seen from Figure 1 that the curves would need to be shifted around 10 dB to the left in order to have around 5% of the UE transmitting with maximum power. Hence simulation runs using Gamma of 0.8 and CLx-ile of (129 + 10 =) 139 have been performed and the results are shown in Figure 4 below for both coordinated and uncoordinated operations.

(a) Coordinated operation

(b) Uncoordinated operation

Figure 4: UE transmit power (using Gamma of 0.8 and CLx-ile of 139)

It can be seen in Figure 4 that for both coordinated and uncoordinated operations using Gamma of 1 and CLx-ile of 132, as expected, around 5% of the victim and interfering UE are transmitting with maximum power of 23 dB, which could better represent the less aggressive UL power control algorithm Set 2.

In view of the above observations, it is proposed to adopt the UL power control parameters for the coexistence study for WP5D as follow:

Table 1: Power control algorithm parameter

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 200 MHz channel bandwidth |
| Set 1 | 1 | 132 |
| Set 2 | 0,8 | 139 |

# 3.	Conclusion and proposal

The simulation results in this contribution have shown that the UL power control parameters for NR coexistence study need to be changed (compared to the ones used for E-UTRA coexistence simulations) in order to better represent the expected network operation in the simulation. Hence it is proposed to adopt the UL power control parameters summarized in Table 2 below for the coexistence study for WP5D:

Table 2: Power control algorithm parameter

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 200 MHz channel bandwidth |
| Set 1 | 1 | 132 |
| Set 2 | 0,8 | 139 |

# References

[1]	RP-161214, “Revision of SI: Study on New Radio Access Technology”, NTT DOCOMO, INC.

[2]	RP-160508, “LS on Characteristics of terrestrial IMT systems for frequency sharing/interf. analysis in 24.25 - 86 GHz”, ITU-R WP5D.

[3]	R4-165140, Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, ITU-R WP5D

[4]	R4-165141, “Modelling and simulation of IMT networks for use in sharing and compatibility studies”, ITU-R WP5D.

[5]	R4-167079, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT DOCOMO, INC.

[6]	R4-167078, “Way forward on BS Beamforming Model for the ITU WP5D coexistence simulations”, Huawei, HiSilicon.

[7]	R4-167123, “Way forward on UE Beamforming Model for the ITU WP5D coexistence simulations”, Qualcomm.

[8]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.