3GPP TSG-RAN4 Meeting #80	R4-168341

# Ljubljana, Slovenia, 10th Oct 2016 - 14th Oct 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.144 | CR | 0003 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UTRA BHH TRP and TRS Requirements for Band VI and XIX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UTRA_TRP_TRS-Core |  |  |  |  |  | Date: |  |  | 2016-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The BHH TRP/TRS Requirements for UMTS band VI, and XIX are missing in the current version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding BHH TRP/TRS requirements for UMTS band VI, and XIX, which are endorsed in RAN4 #80. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement of BHH TRP/TRS at UMTS band VI, and XIX are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1, 7.1.2.1, B.2.1.2.1, B.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[…First change....]

6.1.2	Beside the head and hand phantom position

Beside the head and hand phantom position is defined in TR 25.914 [6] subclauses 5.1.5 and 5.1.6.

6.1.2.1	UTRA FDD

Handheld UE TRP minimum performance requirements for UTRA FDD in beside the head and hand phantom position and the primary mechanical mode are defined in Table 6.1.2.1-1.

Table 6.1.2.1-1: Handheld UE TRP minimum performance requirement for FDD roaming bands in beside the head and hand phantom position and the primary mechanical mode

| Operating band | Power class 1 | Power class 2 | Power class 3 |  | Power class 3bis |  | Power class 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) |  | Power (dBm) |  | Power (dBm) |  |
|  |  |  | Average | Min | Average | Min | Average | Min |
| I | - | - | 13,25 | TBD | TBD | TBD | TBD | TBD |
| II | - | - | 13,25 | TBD | TBD | TBD | TBD | TBD |
| III | - | - | TBD | TBD | TBD | TBD | TBD | TBD |
| IV | - | - | TBD | TBD | TBD | TBD | TBD | TBD |
| V | - | - | 9,40 | TBD | TBD | TBD | TBD | TBD |
| VI | - | - | 9,40 | TBD | TBD | TBD | TBD | TBD |
| VII | - | - | TBD | TBD | TBD | TBD | TBD | TBD |
| VIII | - | - | 9,40 | TBD | TBD | TBD | TBD | TBD |
| IX | - | - | TBD | TBD | TBD | TBD | TBD | TBD |
| XIX |  |  | 9,40 | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: Applicable for dual-mode GSM/UMTS. NOTE 2: Applicable for devices narrower than 72mm as defined in TR 25.914. NOTE 3: Not applicable for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation). |  |  |  |  |  |  |  |  |

[…Next Section...]

7.1.2	Beside the head and hand phantoms position

Beside the head and hand phantom position is defined in TR 25.914 [6] subclauses 5.1.5 and 5.1.6.

7.1.2.1	UTRA FDD

Handheld UE TRS minimum performance requirements for UTRA FDD in beside the head and hand phantoms position and the primary mechanical mode for 1% BER with 12.2kbps DL reference channel as defined in Annex C.3 of [2] are defined in Table 7.1.2.1-1.

Table 7.1.2.1-1: Handheld UE TRS minimum requirements for UTRA FDD roaming bands in beside the head and hand phantoms position and the primary mechanical mode

| Operating band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3,84 MHz | -101,00 | TBD |
| II | dBm/3,84 MHz | -99,00 | TBD |
| III | dBm/3,84 MHz | TBD | TBD |
| IV | dBm/3,84 MHz | TBD | TBD |
| V | dBm/3,84 MHz | -96,75 | TBD |
| VI | dBm/3,84 MHz | -96,75 | TBD |
| VII | dBm/3,84 MHz | TBD | TBD |
| VIII | dBm/3,84 MHz | -96,75 | TBD |
| IX | dBm/3,84 MHz | TBD | TBD |
| XIX | dBm/3,84 MHz | -96,75 | TBD |
| NOTE 1: For power class 3, 3bis and 4 this shall be achieved at the maximum output power. NOTE 2: For the UE which supports both Band III and Band IX operating frequencies, the reference level of TDB dBm TRS <REFÎor> [average and min] shall apply for Band IX. NOTE 3: Applicable for dual-mode GSM/UMTS. NOTE 4: For the UE which supports DB-DC-HSDPA configuration 2, average <REFÎor> level of -98 dBm/3,84 MHz and max <REFÎor> level of -95 dBm/3,84 MHz shall apply for Band II. NOTE 5: For the UE which supports DB-DC-HSDPA configuration 2, average <REFÎor> level of -100 dBm/3,84 MHz and max <REFÎor> level of -97 dBm/3,84 MHz shall apply for Band IV. NOTE 6: Applicable for devices narrower than 72mm as defined in TR 25.914. NOTE 7: Not applicable for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation). |  |  |  |

[…Next Section...]

Annex B (informative):
Recommended performance

B.1	General

This annex introduces the concept of recommended OTA performance for operating bands. This requirement is not mandatory but is recommended.

The concept of recommended performance is to ensure that UE/MS OTA performance is maximised in order to improve user experience and network performance. It is recognised that the ability to meet the recommended performance depends on the number of frequency bands supported by the UE/MS.

B.2	Transmitter total radiated power

The OTA TRP performance for GSM, UTRA and E-UTRA should be greater or equal than the recommended values in this clause.

B.2.1	Recommended performance for handheld UE

[…SNIP…]

B.2.1.2	Beside the head and hand phantoms position

B.2.1.2.1	UTRA FDD

Table B.2.1.2.1-1: Handheld UE TRP recommended performance for UTRA FDD beside the head and hand phantoms position and the primary mechanical mode

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 3bis | Power class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  |  |  | Average | Average | Average |
| I | - | - | 16,25 | TBD | TBD |
| II | - | - | 16,25 | TBD | TBD |
| III | - | - | TBD | TBD | TBD |
| IV | - | - | TBD | TBD | TBD |
| V | - | - | 12,40 | TBD | TBD |
| VI | - | - | 12,40 | TBD | TBD |
| VII | - | - | TBD | TBD | TBD |
| VIII | - | - | 12,40 | TBD | TBD |
| IX | - | - | TBD | TBD | TBD |
| XIX | - | - | 12,40 | TBD | TBD |
| NOTE 1: Applicable for dual-mode GSM/UMTS. NOTE 2: Applicable for devices narrower than 72mm as defined in TR 25.914. NOTE 3: Not applicable for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation). |  |  |  |  |  |

[…Next Section...]

B.3	Receiver total radiated sensitivity

The OTA TRS performance for GSM, UTRA and E-UTRA should be lower or equal than the recommended values shown in this clause.

B.3.1	Recommended performance for handheld UE

[…SNIP...]

B.3.1.2	Beside the head and hand phantoms position

B.3.1.2.1	UTRA FDD

Table B.3.1.2.1-1: TRS recommended performance for UTRA FDD in the beside the head and hand phantoms position for the primary mechanical mode

| Operating band | Unit | <REFÎor> Average |
| --- | --- | --- |
| I | dBm/3,84 MHz | -104,00 |
| II | dBm/3,84 MHz | -102,00 |
| III | dBm/3,84 MHz | TBD |
| IV | dBm/3,84 MHz | TBD |
| V | dBm/3,84 MHz | -99,75 |
| VI | dBm/3,84 MHz | -99,75 |
| VII | dBm/3,84 MHz | TBD |
| VIII | dBm/3,84 MHz | -99,75 |
| IX | dBm/3,84 MHz | TBD |
| XIX | dBm/3,84 MHz | -99,75 |
| NOTE 1: For the UE which supports DB-DC-HSDPA configuration 2, average <REFÎor> level of -101 dBm/3,84 shall apply for Band II. NOTE 2: For the UE which supports DB-DC-HSDPA configuration 2, average <REFÎor> level of -103 dBm/3,84 MHz shall apply for Band IV. NOTE 3: Applicable for devices narrower than 72mm as defined in TR 25.914 NOTE 4: Not applicable for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation) |  |  |